3GPP TSG-RAN2 Meeting #124	R2-2313323

# Chicago, USA, 13rd – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4487 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR DAPS handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For DAPS handover, the network controls the UE transmission power on the source cell and on the target cell by transmitting DAPS-UplinkPowerConfig to the UE. DAPS-UplinkPowerConfig-r16 ::= SEQUENCE { p-DAPS-Source-r16 P-Max, p-DAPS-Target-r16 P-Max, uplinkPowerSharingDAPS-Mode-r16 ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic } } During DAPS handover, if the UE detects radio link failure with the source cell, the UE transmission power with the target cell should not be restricted by DAPS-UplinkPowerConfig. Besides, it is missing to refer to the RAN1 specification for p-DAPS-Source-r16 and and p-DAPS-Target-r16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UE detects radio link failure with the source cell, the UE releases DAPS-UplinkPowerConfig. Add the references to the RAN1 specification for p-DAPS-Source-r16 and and p-DAPS-Target-r16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the case that the UE detects radio link failure with the source cell during DAPS handover, the UE transmission power is restricted by DAPS-UplinkPowerConfig. The UE may encounter handover failure due to the maximum transmission power restricted by p-DAPS-Target-r16. Impact analysis Impacted functionality: DAPS Interoperability: There is no interoperability issue because the change only applies to the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.10.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.10.3	Detection of radio link failure

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection and DAPS-UplinkPowerConfig.

<Unrelated part omitted>

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

<Unrelated part omitted>

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover, as specified in TS 38.213 [13].

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover, as specified in TS 38.213 [13].

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).