3GPP TSG-RAN WG2 Meeting #124	R2-2313371

# Chicago, USA, 13th-17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1723 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the size of SRI field in the NCR related MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater |  |  |  |  |  | Date: |  |  | 2023-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NCR uplink BH beam is indicated by the uplink TCI state ID or SRI. According to the current endorsed 38.321 running CR, If SRI (referring to an SRS-ResourceID) is used, it is contained in the 4 rightmost bits of this field. But according to the RRC spec, the max number of SRS-Resources is 64 as follows: SRS-ResourceId ::= INTEGER (0..maxNrofSRS-Resources-1) … maxNrofSRS-Resources-1 INTEGER ::= 63 Thus, the size of SRI field should be 6 bits rather than 4 bits. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the size of SRI field in the NCR uplink BH beam indication MAC CE from 4 bits to 6 bits. Note: the change is made based on the latest endorsed MAC running CR with the original revision marks kept. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The size of SRI field in the NCR Uplink Backhaul Link Beam Indication MAC CE is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.1.3	MAC Control Elements (CEs)

UNCHANGED PARTS OMMITED

6.1.3.y	NCR Uplink Backhaul Link Beam Indication MAC CE

The NCR Uplink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.y-1):

-	Uplink TCI state ID or SRI: This field is used to indicate the uplink beam to be used for backhaul link transmission. If the dl-OrJointTCI-StateList is configured as specified in TS 38.331 [5] and unifiedTCI-StateType is configured with a value joint, this field contains TCI-StateId (comprising all 7 bits) of a Joint TCI State configured in the active DL BWP, which is used as the uplink beam indication for backhaul link transmission. If the ul-TCI-StateList is configured as specified in TS 38.331 [5], this field contains TCI-UL-State-Id (contained in the 6 rightmost bits) of an UL TCI State configured in the active UL BWP, which is used as the uplink beam indication for backhaul link transmission, with the 1 remaining bit set to zero. Otherwise, this field contains an SRI (contained in the 6 rightmost bits) referring to an SRS-ResourceID configured in the active UL BWP which is used as the uplink beam indication for backhaul link transmission, with the 1 remaining bit set to zero;

-	R: Reserved bit, set to 0.

Figure 6.1.3.y-1: NCR Uplink Backhaul Beam Indication MAC CE

END OF CHANGES