3GPP TSG-RAN WG2 Meeting #124	R2-2313376

Chicago, USA 13rd – 17th Nov, 2023

Agenda Item:	7.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on shared processing for MBS broadcast and unicast reception

Document for:   Discussion and decision

Introduction

In this contribution, we further analyse the remaining issue on capability reporting for shared processing.

Discussion

In previous meetings, it was agreed that frequency/SCS/CFR bandwidth (location) of the non-serving cell will be included in MII.

We would like to share our understanding on how the MII is used by the serving gNB. In order to determine which FSPC can be used for broadcast reception of the non-serving cell, the serving gNB should be aware of the following:1)the FSPC that supports the non-serving cell broadcast based on UE's reported capability; 2) the frequency band/SCS/CC BW capability corresponding to this FSPC matches with the information reported in the MII. Based on this, the serving gNB can enable simultaneous unicast/multicast and broadcast reception by, e.g. releasing the SCell related to the FSPC used for broadcast reception (e.g. FS4 in Figure-1), and scheduling unicast/multicast according to the remaining FS capability (e.g. FS1 and FS3 in the second line in Figure-1).

Figure-1 BC and FSC for unicast in serving cell(s) and broadcast in non-serving cell(s)

Observation 1: The serving gNB determines which FSPC can be used for broadcast reception of non-serving cell according to the MII.

According to the current TS 38.331, the reported band combinations in UE capability can only include the frequency bands within the band filter provided by the gNB (i.e. frequencyBandListFilter).

The UE shall: 1> compile a list of "candidate band combinations" according to the filter criteria in capabilityRequestFilterCommon (if included), only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received; 1> for each band combination included in the list of "candidate band combinations": 2> if the network (E-UTRA) included the eutra-nr-only field, or 2> if the requested rat-Type is eutra: 3> remove the NR-only band combination from the list of "candidate band combinations";

That means, for inter-PLMN scenario, to enable the UE to report the FSPC supporting broadcast reception in non-serving cell, the gNB needs to include the concerned band of inter-PLMN in the band filters. However, once the NW include the concerned band for inter-PLMN broadcast reception in the band filters in the capablity enquiry, the UE has to report UE capabilities related to both unicast and broadcast for the band since the UE doesn’t know whether the NW is only interested in broadcast related capabilities for the band. The issue is unicast related capabilities are much more complicated than the broadcast-only capabilities, especially considering these capabilities are needed to be combined with other capabilities related to other PLMNs in a band combination. Therefore, the UE may report lots of BCs/FeatureSets related to the unicast capabilities of the inter-PLMN bands, which are not useful to the network but will increase the capability size significantly.

Observation 2: To enable the UE to report the FSPC supporting broadcast reception in non-serving cell, the gNB needs to include the concerned bands of inter-PLMN in the band filter. In this case, the UE may also report lots of BCs/FeatureSets related to the unicast capabilities of inter-PLMN bands, which will unnecessarily increase the capability size.

To address this issue above, the NW needs to indicate the UE, in capability enquiry message, which bands are only requested for non-serving cell broadcast capabilities. In this case, the irrelevant inter-PLMN unicast capabilities can be avoided.

Proposal 1: The gNB should indicate the UE in case some bands in the band filter are only requested for MBS reception from non-serving cell in UE capability enquiry procedure.

Conclusion

In this contribution, we provide the following observations and proposals:

Observation 1: The serving gNB determines which FSPC can be used for broadcast reception of non-serving cell according to the MII.

Observation 2: To enable the UE to report the FSPC supporting broadcast reception in non-serving cell, the gNB needs to include the concerned bands of inter-PLMN in the band filter. In this case, the UE may also report lots of BCs/FeatureSets related to the unicast capabilities of inter-PLMN bands, which will unnecessarily increase the capability size.

Proposal 1: The gNB should indicate the UE in case some bands in the band filter are only requested for MBS reception from non-serving cell in UE capability enquiry procedure.

References

[1] RAN2#123-bis chairman’s note.