3GPP TSG-RAN WG2 Meeting #124										       R2-2313383

Chicago, USA, Nov. 13th – 17th, 2023

Source: 	Xiaomi

Title:  	Clarification on the non-serving cell reception capability of MBS broadcast

Agenda Item:	7.11.3

Document for:	Discussion and Decision

# Introduction

According to the discussion on the UE capability for the non-serving cell reception, RAN2 made the following agreement:

RAN2#122 meeting agreements: - The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs.

According to the running CR discussion (i.e. [2] and [3]) for both 38.331 and 38.306, it seems that companies have different understandings on the non-serving cell reception capability of MBS broadcast and on the number of frequencies reported for the shared capability of unicast and MBS broadcast. In this contribution, we provide some clarification on the non-serving cell reception capability of MBS broadcast.

# Discussion

## Non-serving cell reception of MBS broadcast

According to the current 38.306 specification, the UE have the following capability to be indicated/supported for MBS broadcast:

- Broadcast reception (i.e. Optional features without UE radio access capability parameters)

- broadcastSCell-r17 per FSPC

- dci-BroadcastWith16Repetitions-r17 per FSPC

- fdm-BroadcastUnicast-r17 per FSPC

- intraSlotTDM-UnicastGroupCommonPDSCH-r17 per FSPC

38.306: Broadcast reception It is optional for UE to support broadcast reception as specified in TS 38.331 [9]. A UE that supports the feature shall also support: - Group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI; - Group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH; - CFR configuration for broadcast; - CORESET and common search space for broadcast; - DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast; - Inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots; - MCCH change notification indication via DCI; - RRC configured slot-level repetition up to 8 for MTCH; - One G-RNTI per UE is supported for broadcast reception; - Support of FDMed MCCH and PBCH; - Support of up to 64QAM for FR1/FR2; - 4 broadcast MRBs as the minimum number; - PDCP 12 bits SN; - ROHC with profiles 0x0000, 0x0001 and 0x0002; - 4 ROHC context sessions; - RLC UM with 6 bits SN; - RLC UM with 12 bits SN; - DRX with long DRX cycle for MBS broadcast as specified in TS 38.321 [8].

Regarding the basic MBS broadcast reception capability of “Broadcast reception (i.e. Optional features without UE radio access capability parameters)” as quoted above, we think that this Rel-17 basic broadcast reception capability should be also applicable for non-serving cell MBS broadcast reception, so that the gNB can schedule any UE via any cell with the basic MBS broadcast capability.

Proposal 1: When indicating the support of the Rel-18 MBS broadcast reception of non-serving cell, the UE shall also support the Rel-17 basic broadcast reception capability (i.e. optional without UE radio access capability parameters). No extra specification change is expected.

| broadcastSCell-r17 Indicates whether the UE supports MBS reception via broadcast in RRC_CONNECTED, on one frequency indicated in an MBSInterestIndication message, when an SCell is configured and activated on that frequency, as specified in TS 38.331 [9]. NOTE: The UE is not required to receive MBS via broadcast on PCell and SCell simultaneously | FSPC | No | No | No |
| --- | --- | --- | --- | --- |

Regarding broadcastSCell-r17 as quoted above, this capability bit is not applicable for the Rel-18 MBS broadcast reception of non-serving cell, as the concept of SCell is only applicable for serving cell.

Observation 1: broadcastSCell-r17 is not applicable for non-serving cell.

| dci-BroadcastWith16Repetitions-r17 Indicates whether the UE supports up to 16 times dynamic slot-level repetition for broadcast MTCH. | FSPC | No | No | No |
| --- | --- | --- | --- | --- |

Regarding dci-BroadcastWith16Repetitions-r17 as quoted above, we think that the UE can also indicate the optional processing capability of “up to 16 times dynamic slot-level repetition for broadcast MTCH”, as the UE is only required to support “RRC configured slot-level repetition up to 8 for MTCH” if dci-BroadcastWith16Repetitions-r17 is not indicated. This capability bit can be used for the IoT test and by the CN via the network implementation, same as Rel-17.

Proposal 2: When indicating the support for the Rel-18 MBS broadcast reception of non-serving cell, the UE can indicate dci-BroadcastWith16Repetitions-r17 for the same CC. No extra specification change is expected.

| fdm-BroadcastUnicast-r17 Indicates whether the UE supports overlapping PDSCH reception that one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED in a slot are partially or fully overlapping in time domain and non-overlapping in frequency domain. A UE supporting this feature shall also support broadcast reception as specified in clause 5.10. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Regarding fdm-BroadcastUnicast-r17 quoted above, this capability bit is only for the RRC_CONNECTED MBS reception via serving cell. Thus, this capability bit is not applicable for the Rel-18 MBS broadcast of non-serving cell reception.

Observation 2: fdm-BroadcastUnicast-r17 is not applicable for non-serving cell.

| intraSlotTDM-UnicastGroupCommonPDSCH-r17 Indicates whether the UE supports Intra-slot TDM-ed unicast PDSCH and group-common PDSCH. The value indicates that for any two consecutive slots n and n+1, if there are more than 1 broadcast/multicast/unicast PDSCH in either slot, whether to require the minimum time separation (4 OFDM symbols for 30kHz and 7 OFDM symbols for 60kHz) between starting time of any two broadcast/multicast/unicast PDSCHs within the duration of these slots. This feature includes the following functional components: - Supports TDM between one unicast PDSCH and one group-common PDSCH in a slot; - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC; - Support TDM among N (N>1) group-common PDSCHs in a slot per CC; - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC; - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept based on pdsch-ProcessingType1-DifferentTB-PerSlot; - Up to one broadcast PDSCH is supported in a slot. A UE supporting this feature shall support broadcast reception as specified in clause 5.10 and/or indicate support of dynamicMulticastPCell-r17, and shall indicate support of pdsch-ProcessingType1-DifferentTB-PerSlot. NOTE1: Group-common PDSCH(s) are counted as unicast PDSCH(s). NOTE2: The max number of (M+1), N, (K+L) are determined based on the numbers reported by pdsch-ProcessingType1-DifferentTB-PerSlot. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Regarding intraSlotTDM-UnicastGroupCommonPDSCH-r17, since the “unicast PDSCH” as explained in this capability bit is only for the RRC_CONNECTED UE, this capability bit is not applicable for the Rel-18 MBS broadcast of non-serving cell reception.

Observation 3: intraSlotTDM-UnicastGroupCommonPDSCH-r17 is not applicable for non-serving cell.

## Simultaneous reception capability via multiple cells

According to the UE capability discussion for Rel-18 MBS broadcast of non-serving cell reception, the current running CR of 38.306 captures the followings:

#### 4.2.7.6 FeatureSetDownlinkPerCC parameters < unchanged part is omitted> broadcastNonServingCell-r18 Indicates whether the UE supports MBS broadcast reception from a non-serving cell. NOTE: The UE is not required to receive simultaneously on the serving cell(s) and a non-serving cell.; FSPC; No; N/A; N/A. < unchanged part is omitted>

Since the UE’s reception capability for non-serving cell MBS broadcast is FSPC (i.e. FeatureSetDownlinkPerCC), RAN2 needs to discuss the following two issues:

- Issue 1: Simultaneous reception capability between serving cell reception and MBS broadcast non-serving cell reception

- Issue 2: Simultaneous reception capability between multiple MBS broadcast non-serving cells.

Figure 1: Simultaneous reception of serving cell and non-serving cell

For Issue 1, we think that the UE reception capability (i.e. either unicast or multicast or broadcast) of the serving cell as indicated with the Rel-17 capability signal can be reused, regardless of whether the UE indicates the support of the Rel-18 MBS broadcast reception of non-serving cell. This means that when the UE indicates that on CC can support MBS broadcast reception of non-serving cell, the UE should assume that other CC(s) of the same feature set would be configured as serving cells, as illustrated in Figure 1.

Proposal 3: When indicating the support for the Rel-18 MBS broadcast reception of non-serving cell, the serving cell reception capability is independently indicated. No extra specification change is expected.

For Issue 2, as agreed in RAN2#122 meeting, indicating the support for the Rel-18 MBS broadcast reception of non-serving cell “does not imply simultaneous reception on multiple CCs”. We think that the UE can support up-to 1 non-serving cell reception of MBS broadcast reception in Rel-18, so as to reduce the UE implementation complexity. As illustrated in Figure 1, the UE can indicate the non-serving cell reception of MBS broadcast reception in CC_3 and CC_4. However, the UE is only required to use one non-serving cell for MBS broadcast reception. If most companies want to support the more than one non-serving cells receptions for MBS broadcast, we think that RAN2 can further clarify when the multiple non-serving cells receptions for MBS broadcast is TDMed or not, since the simultaneous reception in the time domain would require extra UE implementation complexity (e.g. extra UE buffer).

Proposal 4: The UE can indicate MBS broadcast reception of non-serving cell in multiple CCs.

Proposal 5: The UE is only required to support up-to 1 non-serving cell reception of MBS broadcast.

## Non-serving cell frequencies reported in MBSInterestIndication

As captured in the RRC running CR for Rel-18 MBS [3], the UE can indicate multiple NonServingInfo-r18.

MBS-NonServingInfoList-r18 ::= SEQUENCE (SIZE (1..maxFreqMBS-r17)) OF NonServingInfo-r18 NonServingInfo-r18 ::= SEQUENCE { freqInfoMBS-r18 FreqInfoMBS-r18, OPTIONAL, cfr-InfoMBS-r18 CHOICE { cfr-Bandwidth-r18 INTEGER (1..maxNrofPhysicalResourceBlocks), cfr-LocationAndBW-r18 CFR-LocationAndBW-r18 } OPTIONAL, subcarrierSpacing-r18 SubcarrierSpacing OPTIONAL }

If Proposal 5 is agreed, we think that the UE should not indicate a non-serving cell frequency information which is not to be used for MBS broadcast reception. This means that the UE only needs to report 1 NonServingInfo-r18. Then the ASN.1 IE type of MBS-NonServingInfoList-r18 can be changed from “List” to one-entry single IE.

Proposal 6: If Proposal 5 is agreed, the UE only reports one non-serving cell information in the MBSInterestIndication message.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: broadcastSCell-r17 is not applicable for non-serving cell.

Observation 2: fdm-BroadcastUnicast-r17 is not applicable for non-serving cell.

Observation 3: intraSlotTDM-UnicastGroupCommonPDSCH-r17 is not applicable for non-serving cell.

Proposal 1: When indicating the support of the Rel-18 MBS broadcast reception of non-serving cell, the UE shall also support the Rel-17 basic broadcast reception capability (i.e. optional without UE radio access capability parameters). No extra specification change is expected.

Proposal 2: When indicating the support for the Rel-18 MBS broadcast reception of non-serving cell, the UE can indicate dci-BroadcastWith16Repetitions-r17 for the same CC. No extra specification change is expected.

Proposal 3: When indicating the support for the Rel-18 MBS broadcast reception of non-serving cell, the serving cell reception capability is independently indicated. No extra specification change is expected.

Proposal 4: The UE can indicate MBS broadcast reception of non-serving cell in multiple CCs.

Proposal 5: The UE is only required to support up-to 1 non-serving cell reception of MBS broadcast.

Proposal 6: If Proposal 5 is agreed, the UE only reports one non-serving cell information in the MBSInterestIndication message.

# Reference

[1] RAN2#122 meeting minutes

[2] Post123bis][614][eMBS] UE capabilities CRs update and open issues (vivo)

[3] [Post123bis][611][eMBS] 38.331 CR update and open issues (Huawei)