3GPP TSG-RAN WG2 Meeting #124										       R2-2313387

Chicago, USA, Nov. 13th – 17th, 2023

Source: 	Xiaomi

Title:  	Capability restriction for the proactive approach

Agenda Item:	7.17.2

Document for:	Discussion and Decision

# Introduction

According to the discussion in the RAN2#123bis meeting, RAN2 agreed to allow the UE to report its restricted MIMO-layer per FSPC:

It is confirmed that the previous agreement that Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per CC at least applies for the reactive approach. Baseline for the proactive approach: Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per FSPC (per cc per BC).

In this contribution, we provide further consideration on the capability restriction reported for MUSIM.

# Discussion

## Bandwidth and MIMO-layer for affected band

According to the current UE capability bits reported, the maximum bandwidth and the maximum MIMO-layer are reported as FSPC. According to 38.306, the UE’s total L2 buffer for DL/UL is calculated by counting the maximum MIMO-layer and the maximum bandwidth together. For a MUSIM UE, one possible implementation would be that the L2 buffer would be split between two SIMs. The split of the L2 buffer would impact both the MIMO-layer and the bandwidth to be supported for each SIM. Then, reporting only the MIMO-layer cannot reflect the UE’s real buffer size allocated to each SIM. For example, when the UE is using only SIM-A in band-1, SIM-A-buffer=f(bandwidth=200MHz, MIMO-layer=8). When the UE is using only SIM-A in band-2, SIM-A-buffer=f(bandwidth=100MHz, MIMO-layer=4). When the UE uses SIM-A in band-1 and SIM-B in band-2. The UE would split its buffer to these two bands. Then we could have the following split options:

- Option 1 (only reduce MIMO-layer): SIM-A-buffer=f(bandwidth=200MHz, MIMO-layer=2); SIM-B-buffer=f(bandwidth=100MHz, MIMO-layer=4)

- Option 2 (only reduce bandwidth): SIM-A-buffer=f(bandwidth=100MHz, MIMO-layer=8); SIM-B-buffer=f(bandwidth=100MHz, MIMO-layer=4)

- Option 3 (reduce both bandwidth and MIMO-layer): SIM-A-buffer=f(bandwidth=100MHz, MIMO-layer=4); SIM-B-buffer=f(bandwidth=100MHz, MIMO-layer=4)

Thus, to allow more flexible UE implementation, we think that the UE should be allowed to report its bandwidth restriction.

38.331 running CR: ASN.1 TP included for reactive approach MUSIM-CellToAffect-r18 ::= SEQUENCE { musim-SCellIndex-r18 SCellIndex, musim-MIMO-Layers-DL-r18 INTEGER (1..8) OPTIONAL, musim-MIMO-Layers-UL-r18 INTEGER (1..4) OPTIONAL, musim-BandwidthDL-r18 ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000} OPTIONAL, musim-BandwidthUL-r18 ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000} OPTIONAL }

38.331 running CR: ASN.1 TP included for proactive approach MUSIM-AffectedBandCombList-r18 ::= SEQUENCE (SIZE (1..maxBandComb)) OF MUSIM-AffectedBandComb-r18 MUSIM-AffectedBandComb-r18 ::= SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF MUSIM-CapabilityRestrictedBandParameters MUSIM-CapabilityRestrictedBandParameters SEQUENCE { bandEntryIndex INTEGER(1..maxBands-MUSIM), musim-capabilityRestricted-r18 SEQUENCE { musim-CapabilityDL-PerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF MUSIM-CapabilityDL-r18 OPTIONAL, musim-CapabilityUL-PerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF MUSIM-CapabilityUL-r18 OPTIONAL } OPTIONAL } MUSIM-CapabilityDL SEQUENCE { musim-MIMO-LayersDL-r18 INTEGER (1..8), OPTIONAL musim-BandwidthDL-r18 ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000} OPTIONAL } MUSIM-CapabilityUL SEQUENCE { musim-MIMO-LayersUL-r18 INTEGER (1..8), OPTIONAL musim-BandwidthUL-r18 ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000} OPTIONAL }

The text proposal capturing the UE capability restriction reporting per FSPC is provided above.

Proposal 1: Bandwidth restriction is reported per CC for reactive approach.

Proposal 2: Bandwidth restriction is reported per FSPC for proactive approach.

# Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: Bandwidth restriction is reported per CC for reactive approach.

Proposal 2: Bandwidth restriction is reported per FSPC for proactive approach.

# Reference

[1] RAN2#123bis meeting minutes.

[2] R2-2311933, vivo, “Running RRC CR for NR MUSIM enhancements”.