3GPP TSG-RAN WG2 Meeting #124										       R2-2313392

Chicago, USA, Nov. 13th – 17th, 2023

Source: 	Xiaomi

Title:  	Clarification on the IDLE or INACTIVE mobility with mIAB

Agenda Item:	7.12.2.2

Document for:	Discussion and Decision

# Introduction

According to the RAN2 discussion on keep the on-board UE camping on the mobile IAB cell, RAN2 made the following agreements:

RAN2#122 meeting agreements [1]: Agreements on cell reselection: - R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state. - R2 direction (solution agreements at later stage, no other directions will be considered): - RAN2 acknowledges following two problems to be addressed for idle/inactive UEs: - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low. - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells. - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

RAN2#123 meeting agreements [2]: Mobility enhancements – Idle/Inactive State: Agreements on cell reselection: - For a UE that is “on-board”, irrespective whether it is camped on the mobile IAB cell or a stationary cell, it can prioritize another frequency for which a mobile IAB cell is the best cell). - No enhancement is needed for intra-frequency and equal-priority cell reselection. - The procedure that UE searches and measure for mIAB cells on different frequencies is unspecified. RAN2 assumes that as assistance information, the NW can optionally provide inter-frequency mIAB list in SIB4, details FFS. - It is left to UE implementation to determine whether the UE is physically on a moving vehicle and when it applies mobile IAB cell reselection prioritization for agreed scenarios.

RAN2#123bis meeting agreements [3]: P1: mIAB PCI list is optional present (i.e., not mandatory) for indicated mIAB frequency (confirming that mIAB PCI list is introduced) P7: it is left to UE implementation to determine an actual prioritized frequency among frequencies that can be prioritized for mIAB cell/HSDN/MBS/SL/V2X? P8: Existing Note 0c in TS 38.304 is applicable for the prioritization between mIAB cell/HSDN/MBS/SL/V2X. So, no or marginal additional specification work is needed. FFS: P2: To discuss further if mIAB PCI list is not necessarily exclusive, i.e., the PCI list may or may not include PCIs of non-mIAB cell. P3: To discuss further if mIAB PCI list is not necessarily complete, i.e., the PCI list may or may not include all possible mIAB PCIs.

In this contribution, we provide our understandings on the remaining issues for the IDLE/INACTIVE UE mobility with mIAB node.

# Discussion

## Assistance information for IDLE/INACTIVE UE mobility

38.304 running CR for mIAB [5]: with TP included A UE on a vehicle with a mobile IAB-cell may consider the frequency for which a mobile IAB cell is the best cell to be the highest priority based on the mobileIAB-Cell-r18 indication provided in SIB1 and the mobileIAB-InterFreqCarrierFreqList provided in SIB4. Editor Notes: FFS if mIAB PCI list is not necessarily exclusive, i.e., the PCI list may or may not include PCIs of non-mIAB cell. FFS if mIAB PCI list is not necessarily complete, i.e., the PCI list may or may not include all possible mIAB PCIs.

According to the running RRC CR [4] of mIAB and the RAN2 agreements of introducing frequency and PCI of mIAB via SIB4. However, it seems still unclear how the UE uses the mobility assistance information from SIB4 or SIB1 for the cell reselection.

According to the RAN2 agreements from RAN2#122 meeting, the following two problems need to be address:

- Solution 1 to Problem 1: Assistance information for helping the UE not camping on its mobile IAB-cell to reselect to its mobile IAB-cell

- Solution 2 to Problem 2: Assistance information for keeping the UE on its mobile IAB-cell when the UE is camping on the mobile IAB-cell

For Solution 2, by using mobileIAB-Cell-r18 indication provided in SIB1, the UE is able to know that it is camping on the mobile IAB-cell. Then, the UE can prioritize the current camping frequency.

For Solution 1, the UE needs to know whether a neighbour cell is a mobile IAB cell before cell reselection. As RAN2 agreed that the UE can “prioritize another frequency for which a mobile IAB cell is the best cell”. In order to help the UE to identify that the best cell is a mobile IAB cell, companies agreed in RAN2#123bis meeting that “mIAB PCI list is optional present (i.e., not mandatory) for indicated mIAB frequency”. If the mIAB PCI list is not present, the UE by implementation can read the SIB1 of the neighbour cell before cell reselection, and use the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list provided in SIB4 to prioritize a mIAB frequency. However, since the SIB1 information of the neighbouring cell may not be always available at the UE side before cell reselection, the UE should be allow to use only the mIAB frequency information to prioritize the mIAB frequency. Then after cell reselection, if the prioritized frequency does not provide the mIAB cell, the UE by implementation can remove the highest priority for the frequency prioritized before.

Observation: The UE by implementation can have a proper use of the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list and PCI list provided in SIB4, so as to camp on the mIAB cell. No need to specify the detailed UE behaviours on when/how to read/use the assistance information.

Thus, we consider that TS 38.304 should capture that the UE is allowed to use both the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list and PCI list provided in SIB4 to prioritize an mobile IAB frequency, and the details on when/how the UE reads/uses the information from SIB1 or SIB4 can be left to the UE. The text proposal can be found above. For the TP, we assume that the PCI list (e.g. mobileIAB-PCI-List-r18) would be added for each mIAB frequency (i.e. MobileIAB-InterFreqCarrierFreqInfo-r18).

Proposal 1: The UE is allowed to use the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list and PCI list provided in SIB4, to prioritize the mobile IAB frequency.

Regarding whether the mIAB PCI list is exclusive or complete, we think that 38.304 only needs to capture the required/allowed UE behaviours on prioritizing the mIAB frequency, based on the mIAB assistance information provided for IDLE/INACTIVE mobility. The network by implementation should ensure that the PCI list provided to the UE are mIAB PCI(s). The specification does not need to specify the UE behaviours on the case where a PCI provided in SIB4 is not mIAB PCI.

Proposal 2: No need to specify extra UE/network behaviours on whether the mIAB PCI list provided in SIB4 is exclusive or complete.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation: The UE by implementation can have a proper use of the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list and PCI list provided in SIB4, so as to camp on the mIAB cell. No need to specify the detailed UE behaviours on when/how to read/use the assistance information.

Proposal 1: The UE is allowed to use the mobileIAB-Cell-r18 indication provided in SIB1 and the mIAB frequency list and PCI list provided in SIB4, to prioritize the mobile IAB frequency.

Proposal 2: No need to specify extra UE/network behaviours on whether the mIAB PCI list provided in SIB4 is exclusive or complete.

# Reference

[1] RAN2#122 meeting minutes

[2] RAN2#123 meeting minutes

[3] RAN2#123bis meeting minutes

[4] R2-2310893, Ericsson, “RRC running CR for mobile IAB”

[5] R2-2311608, Intel, “38.304 CR for R18 mIAB”