3GPP TSG-RAN WG2 Meeting #124	R2-2313485

# Chicago, USA, 13th – 17th November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1579 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Koffset handling during RRC connection re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the post meeting discussion [Post123bis][313][NR-NTN], it was agreed in-principle that UE shall clear the configured differential Koffset value upon performing the handover, to eliminate the confusion on which Koffset value the UE shall apply before receiving a new differential Koffset value from the new cell. We think the same confusion also exists when UE is performing a RRC connection re-establishment procedure, and hence needs to be clarified in the same clause as well. |  |  |  |  |  |  |  |  |
| Summary of change: |  | On top of the change agreed in R2-2311597, further specify that UE’s differential Koffset value obtained from the serving cell is not used after triggering the RRC connection re-establishment. Impact Analysis Impacted functionality: Rel-17 IoT-NTN Inter-operability: - If the network is implemented according to the CR and the UE is not, UE and network may have different understanding on which Koffset value is applicable to the UE during the RRC connection re-establishment. - If the UE is implemented according to the CR and the network is not, UE and network may have different understanding on which Koffset value is applicable to the UE during the RRC connection re-establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may use a Koffset value different from that being used by the network during the RRC connection re-establishment procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the Changes

## 5.26	Update of Differential Koffset

The network may provide and update the Differential Koffset of a Serving Cell in a non-terrestrial network by sending the Differential Koffset MAC CE described in clause 6.1.3.21. The configured Differential Koffset is cleared at handover and at RRC connection re-establishment.

The MAC entity shall:

-	if the MAC entity receives a Differential Koffset MAC CE on a Serving Cell:

-	indicate to lower layers the information regarding the Differential Koffset MAC CE.

End of the Changes