3GPP TSG-RAN WG2 Meeting #124	R2-2313508

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SRI use for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, only 16 out of the maximum 64 values which could be configured via SRS-ResourceID are used for NCR UL BH beam indication. This was due to RAN1 initially indicating SRI should be used as beam indication, and SRI has 4 bits. However RAN2 settled on using SRS-ResourceID instead (due to various issues with SRI usage which are captured elsewhere), but limiting it to 16 values which the 4 bits can point to. It has became clear that – once we have settled on use of SRS-ResourceID – any mention or use of SRI is artifical and in fact creates an artifical limit as to how many bits can be used (the relevant field offers 6 bits and therefore the entire SRS-ResourceID and its full range can be indicated, rather than just 4 bits). Analogous change was already agreed for R17 IAB (please see R2-2311269, agreed in Xiamen at RAN2#123-bis). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove mention of SRI. - Expand the useful number of bits from 4 to 6, and indicate SRS-ResourceID is contained in there (rather than have a pointer to a subset of its full ID space, which is what we currently have). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary limitation of the number of beams which can be used for NCR UL BH beam indication. Incorrect use of the term SRI, different from how it is defined in RAN1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.y NCR Uplink Backhaul Link Beam Indication MAC CE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | See Summary of change. |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.1.3.y	NCR Uplink Backhaul Link Beam Indication MAC CE

The NCR Uplink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.y-1):

-	Uplink TCI state ID or SRI: This field is used to indicate the uplink beam to be used for backhaul link transmission. If the dl-OrJointTCI-StateList is configured as specified in TS 38.331 [5] and unifiedTCI-StateType is configured with a value joint, this field contains TCI-StateId (comprising all 7 bits) of a Joint TCI State configured in the active DL BWP, which is used as the uplink beam indication for backhaul link transmission. If the ul-TCI-StateList is configured as specified in TS 38.331 [5], this field contains TCI-UL-State-Id (contained in the 6 rightmost bits) of an UL TCI State configured in the active UL BWP, which is used as the uplink beam indication for backhaul link transmission, with the 1 remaining bit set to zero. Otherwise, 6 rightmost bits of this field contain SRS-ResourceID configured in the active UL BWP which is used as the uplink beam indication for backhaul link transmission, with the 1 remaining bit set to zero;

-	R: Reserved bit, set to 0.

Figure 6.1.3.y-1: NCR Uplink Backhaul Beam Indication MAC CE

End of Changes