TSG RAN WG2 Meeting #124   				    					                               R2-2313535

Chicago, USA, November 13th – November 17th, 2023

Agenda item:		7.3.2 DTX/DRX mechanism

Title:						Key open issues on Cell DTX/DRX

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion and Decision

# Introduction

The ever-increasing need to support more traffic load is putting pressure on the Network Energy consumption with severe financial (Opex) and environmental implications for operators. It is no exaggeration to say that the continued success of 5G NR depends on reducing Network Energy consumption. Naturally, the industry and 3GPP is not unware of such developments and, in Rel-18, 3GPP has been considering Network Energy Saving (NES) techniques in a study phase [1] followed by a standardization phase [2], as a fundamental part of 5G-Advanced.

The guidance of the RAN plenary has been quite clear that the trade-off between energy savings and other KPIs should be carefully balanced [1]:

The study should not only evaluate the potential network energy consumption gains, but also assess and balance the impact on network and user performance, e.g. by looking at KPIs such as spectral efficiency, capacity, user perceived throughput (UPT), latency, UE power consumption, complexity, handover performance, call drop rate, initial access performance, SLA assurance related KPIs, etc. The techniques to be studied should avoid having a large impact to such KPIs.

Cell DTX (Discontinuous Transmission) and Cell DRX (Discontinuous Reception) have been identified as promising NES techniques and have been selected for standardization.

Unfortunately, Cell DTX/DRX still falls short of fulfilling the RAN plenary vision. In particular, as of decisions taken so far, there are some gaps on Cell DTX/DRX design which leads to excessive latency, signaling overhead and UE power consumption in order to bring marginal extra energy savings. This is definitely in contrast to the RAN plenary guidance and, therefore, it is essential to address those gaps.

The Cell DTX/DRX design still has gaps and it does not fulfill the RAN plenary guidance. As of agreements so far, energy saving is achieved by sacrificing latency and/or UE power consumption.

This contribution analyses these key open issues and propose solutions of limited complexity to help Cell DTX/DRX achieve its full potential by improving the trade-off between NES and other KPIs. This will help Cell DTX/DRX to become a successful feature to be widely deployed.

# Cell DTX/DRX gap analysis and solutions

## Key open issue #1 – Handling bursts of traffic

Cell DTX/DRX was always deemed as a feature which will bring benefits at low load. However, it is important to understand the traffic behavior at low load, especially in face of traffic burstiness. For example, 5% load does not mean a continuous 5% resource utilization. Instead, 5% load typically means that on every second we will have (on average) 50 ms of 100% load and 950 ms of 0% load, due to the bursty nature of traffic.

The most efficient way to handle a burst is simply to transmit it as quickly as possible. This is true from latency, UPT and UE power consumption perspectives and, as we show on the following, it is to a great extent also true from Network energy consumption perspective. Unfortunately, this immediate handling of a burst is still missing in Cell DTX/DRX, but it can be addressed with very limited complexity.

Let us consider the network energy consumption (according to TR 38.864) and packet delay in 3 cases:

- Baseline – Cell DTX/DRX is disabled and therefore there is no restriction on PDCCH signaling. Opportunistic symbol shutdown is possible whenever there is no traffic.

- Cell DTX/DRX is enabled and as of current agreement, there is no PDCCH signaling on Cell DTX/DRX inactive time (with very few exceptions).

- Cell DTX/DRX is enabled, but (contrary to case 2) there is a smooth way of continuing PDCCH signaling on a need basis, when a traffic burst arrives (or simply there is still data on the buffer).

These 3 cases are simulated on a TDD scenario (DDDFU, 30 kHz SCS as in set 1 of TR 38.864). The Cell DTX/DRX parameters are set to on-duration of 3 ms (the minimum to include one DL and one UL opportunity on on-duration) and cycle of 20 ms (with SSB aligned to on-duration).

For each case the network power consumption of the first 250 ms after a burst arrival and the corresponding histogram of packet delay are illustrated.

As it can be seen on Figure 1, as a baseline the gNB is always ready to transmit, only doing symbol shutdown opportunistically, but the delay is kept low and the traffic is served as quickly as possible.

In contrast, when Cell DTX/DRX is configured (according to agreements so far) the transmission is interrupted multiple times to abide the inactive time of Cell DTX/DRX. This is illustrated on Figure 2. While this does lead to significant energy savings, it causes a huge increase on packet delay as the transmission is interrupted even when the gNB has more than enough packets to transmit at its buffers. This does not fulfill the RAN plenary guidance as NES techniques should avoid having high impact on KPIs such as latency.

Figure 1 – Baseline: Cell DTX/DRX is not configured and the gNB can opportunistically do symbol shutdown when there is no traffic.

Figure 2 – Cell DTX/DRX activated. With current behavior the transmission is interrupted to save energy on but that leads to increased delay.

Is it possible to have the best of both worlds, attaining low energy consumption and low delay? Absolutely. As shown on Figure 3, if Cell DTX/DRX includes a mechanism to swiftly continue transmissions when there is data on the buffer, then it is possible to have low delay and low energy consumption at the same time. The current high impact on delay can be avoided easily.

Figure 3 – Cell DTX/DRX is activated but the gNB can continue transmission as needed when there is data on the buffer.

Cell DTX/DRX as currently designed has a high impact on delay, but that is absolutely unnecessary. If transmissions can be continued as needed, Cell DTX/DRX can achieve similar energy savings with little to no impact on delay.

Figure 1, 2 and 3 show specific examples aligned to the traffic bursts to facilitate the understanding but the same trend can be seen over a large number of simulations as shown below in Figure 4.

Figure 4 – Cell DTX/DRX modified to bypass PDCCH restrictions on-demand has by far the best trade-off. Little delay added to the baseline and nearly the same energy saving as the current Cell DTX definition (which is very restrictive of PDCCH for new data)

Can we implement the behavior on Figure 3 with the tools which were already defined? Let us analyze the current possibilities to address a traffic burst:

- Use the group DCI signaling to de-activate Cell DTX/DRX whenever a burst needs to be handled. While this solution does provide quickly signaling for de-activation, there is a number of issues with this approach to implement the desired behavior on a burst. First and foremost, this is group signaling. Thus there is only the option to de-activate Cell DTX/DRX for all UEs on the group (cell), even though the traffic burst is typically addressed to a single UE. This will cause unnecessary UE power consumption. Second, and not to be forgotten is that every de-activation would need to be followed by a re-activation to save energy again. It follows more overhead, UE power consumption and network energy consumption. Furthermore, actually a gNB cannot change swiftly between activation and de-activation because the alignment to UE C-DRX is using legacy RRC reconfiguration. If a second UE C-DRX configuration was introduced this problem could have been avoided [3].

- According to current agreements, while the UE does not listen to PDCCH for new assignments during inactive time, the UE can receive PDSCH if it is scheduled on the inactive time. This means that PDCCH can be sent during on-duration to schedule some traffic on the inactive time if the scheduling assignment is done in advance. In the given example, it is completely unrealistic to schedule in advance 17 ms of traffic to fill the inactive time during the 3 ms of on-duration. There would be all types of limitations like reaching the maximum number of HARQ processes, inability to handle further packet arrivals, inflexibility to choose between scheduling new data or retransmissions. All in all this would lead to a very complex gNB scheduler implementation which still would not achieve the desired behavior. Note that if the Cell DTX/DRX cycle is larger, then this approach is even more limited.

- De-activating Cell DTX/DRX by removing the configuration on RRC level. While this has the advantage of being targeted only at the UE which should handle the traffic burst it also has severe disadvantages. First and foremost, the RRC signaling is much slower than what can be done dynamically (e.g. DCI / MAC CE) due to RRC processing delay and RRC level acknowledgment (RRC reconfiguration complete). Second, as this delay is quite significant it would not be fully completed within the on-duration of Cell DTX/DRX. Thus the acknowledgment would need to wait the next on-duration. Because of that it would effectively take at least 2 Cell DTX/DRX cycles to do an RRC reconfiguration for de-activation. During all this time high delay is implied. Last, but not least, after the operation is done Cell DTX/DRX needs to be reconfigured again implying a longer time with excessive network energy consumption.

- Do nothing, keeping Cell DTX/DRX de-activated. This is the behavior shown on Figure 2 and it is shown to have unacceptable delay.

As all already defined possibilities have significant drawbacks, it follows that a targeted enhancement is needed to address this issue and fulfill the RAN plenary vision for Cell DTX/DRX. Fortunately, the solution can be rather simple yet effective. Namely, we only need to specify a mechanism so that the gNB can bypass the Cell DTX/DRX PDCCH restrictions temporarily.

The gNB can send an indication to the UE, so that PDCCH monitoring can be prolonged on-demand during Cell DTX/DRX inactive time.

## Key open issue #2 – Alignment to 5 ms SSB and LTE PSS/SSS on DSS

It was decided to reuse the same cycle duration values for Cell DTX which were already specified for C-DRX long DRX. This means the minimum cycle time is 10 ms. However there can be reasons to have SSB cycles of 5 ms, such as having a DSS scenario where LTE PSS/SSS are anyway transmitted every 5 ms. If the SSB burst period is 5 ms, the gNB needs to wake up for transmission every 5 ms there is no difference in energy consumption between a 5 ms or a 10 ms Cell DTX/DRX cycle, but there is a huge difference on latency and initial access performance.

If SSBs or LTE PSS/SSS (in DSS) need to be transmitted every 5 ms there is no difference in energy consumption between a 5 ms or a 10 ms Cell DTX/DRX cycle, but there is a huge difference on latency and initial access performance.

In fact there was a discussion in RAN 1 to consider the SSB symbols as active time and be able to transmit PDCCH and data. While mandating such behavior would complicate things, we understand it should be possible to set Cell DTX/DRX in a way which makes sure SSB transmissions always lie on the on-duration. With current agreements, that is possible for all SSB configurations except for SSB periods of 5 ms.

Cell DTX/DRX cycle can be set to 5 ms periodicity.

Note that a 5 ms Cell DTX/DRX cycle is compatible with the alignment rule of C-DRX and Cell DTX patterns being a multiple of each other. A 5 ms cycle may be the key to achieve energy savings without latency penalty on DSS scenarios. Eventually, with this smaller cycle a savvy implementation could even achieve energy savings on URLLC scenarios.

## Key open issue #3 – Reducing energy consumption due to retransmissions

Even if only a few UEs are active HARQ retransmission happen very often and it will impact energy saving as a single HARQ retransmission is enough to keep the gNB awake. This leads to severe reduction of the potential energy saving on Cell DTX/DRX In essence, every time there is a retransmission this imply the gNB will not sleep for an extra HARQ-RTT. This is mostly worrisome in TDD as the HARQ-RTT is limited by the TDD period.  An illustration is shown below:

Figure 5 – Every HARQ retransmision delay Cell DTX/DRX true sleeping by an HARQ-RTT. The impact on energy saving is very high as retransmissions occur often.

A detailed analysis in [4] showed that for 5 served UEs 41% of Cell DTX/DRX cycles would have an increase of 15% or higher on the inactive time energy consumption, only due to HARQ retransmissions. If retransmission can be avoided, e.g. by using more robust MCSs, Cell DTX/DRX will achieve much better energy savings. In order to achieve that goal, some improvements may be needed, e.g. that UE CQI measurements can be configured with a different BLER target when Cell DTX/DRX is activated. The exact design and mechanism should be left to RAN 1, but reducing the number of retransmissions is clearly a goal which should be pursued.

RAN 2 to send a LS to RAN 1 to kindly request that RAN 1 designs mechanisms to reduce the number of retransmissions when Cell DTX/DRX is activated.

# Conclusions

In this contribution, we discussed key open issues on the Cell DTX/DRX design for Network Energy Saving. The following observations and proposals have been made:

Observation 1:	The Cell DTX/DRX design still has gaps and it does not fulfill the RAN plenary guidance. As of agreements so far, energy saving is achieved by sacrificing latency and/or UE power consumption.

Observation 2:	Cell DTX/DRX as currently designed has a high impact on delay, but that is absolutely unnecessary. If transmissions can be continued as needed, Cell DTX/DRX can achieve similar energy savings with little to no impact on delay.

Observation 3:	If SSBs or LTE PSS/SSS (in DSS) need to be transmitted every 5 ms there is no difference in energy consumption between a 5 ms or a 10 ms Cell DTX/DRX cycle, but there is a huge difference on latency and initial access performance.

Proposal 1:	The gNB can send an indication to the UE, so that PDCCH monitoring can be prolonged on-demand during Cell DTX/DRX inactive time.

Proposal 2:	Cell DTX/DRX cycle can be set to 5 ms periodicity.

Proposal 3:	RAN 2 to send a LS to RAN 1 to kindly request that RAN 1 designs mechanisms to reduce the number of retransmissions when Cell DTX/DRX is activated.

# References

- RP-220297, "Revised SI: Study on network energy savings for NR", Huawei , 3GPP RAN#95-e, March 2022

- RP-223540, “New WID: Network energy savings for NR ”, Huawei , 3GPP RAN#98-e, December 2022

- R2-2311235, “UE C-DRX configuration used upon cell DTX activation”, vivo, Fraunhofer IIS, Huawei, HiSilicon, LG Electronics Inc., MediaTek Inc., NEC, Xiaomi, ZTE Corporation, Ericsson, Nokia, Nokia Shanghai Bell, BT Plc, NTT DOCOMO INC. , 3GPP RAN2#123-bis, Xiamen, October, 2023

- R1-2307838, “Considerations of Cell DTX/DRX Signaling and Reliability”, Fraunhofer IIS, Fraunhofer HHI, 3GPP RAN1#113, Toulouse, August, 2023