3GPP TSG-RAN WG2 Meeting #124	R2-2313751

# Chicago, USA, November 13-17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1873 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capability for inter-RAT NR FR2 measurements without measurement gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2023-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 in their #123bis meeting agreed to introduce a new UE capability parameter for inter-RAT NR FR2 measurements without gap. This is an addition to the existing UE capability, independentGapConfig-maxCC-r17, defined in TS38.306. The corresponding requirement is defined in the context of per-FR measurement gap in TS38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE capability parameter gaplessMeas-FR2-maxCC-r17, where the UE indicates the maximum number of configured E-UTRA serving cells with which the UE supports inter-RAT NR FR2 measurements without measurement gap, is added. - This UE capability parameter is applicable when only E-UTRA serving cells are configured, i.e. LTE standalone. - The UE reporting this capability and supporting (NG)EN-DC shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC (defined in TS 38.306 [87]). Impact analysis Impacted 5G architecture options: LTE SA Impacted functionality: Inter-RAT NR FR2 measurements without measurement gap Inter-operability: - If the network is implemented according to the CR and the UE is not; No inter-operability issue arises. The network is aware that the UE does not support the inter-RAT NR FR2 measurements without measurement gap ((unless other UE capability parameter the network is aware of indicates the UE supports gapless measurement, e.g. independentGapConfig defined in TS38.306) - If the UE is implemented according to the CR and the network is not; No inter-operability issue arises. The network will configure measurement gap for inter-RAT NR FR2 measurements (unless other UE capability parameter the network is aware of indicates the UE supports gapless measurement, e.g. independentGapConfig defined in TS38.306) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE has no way to convery its capability of inter-RAT NR FR2 measurements without measurement gap depending on the number of configured E-UTRA serving cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.5x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS36.331 CR4968r1 TS38.306 CR0989r1 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.3.6	Measurement parameters

[…]

#### 4.3.6.5x	gaplessMeas-FR2-maxCC-r17

This field defines whether the UE supports inter-RAT NR FR2 measurement without measurement gap as specified in clause 9.1.2 of TS 38.133 [37] while the number of configured serving cells is less than or equal to the indicated number. This field is applicable when only E-UTRA serving cells are configured. The UE reporting this field and supporting (NG)EN-DC shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC (defined in TS 38.306 [32]).