3GPP TSG RAN WG2#124	R2-23113910

Chicago, USA, 13th - 17th November 2023

3GPP TSG RAN WG1 #115		R1-2312371

Chicago, USA, November 13th – November 17th, 2023

Title:	Reply LS on MIMOevo

Response to:	R1-2311004 (R2-2311609)

Release:	Release 18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

Attachments:	None

# 1	Overall description

RAN1 would like to thank RAN2 for the LS on MIMOevo and would like to provide the following response.

- Field applyIndicatedTCI-State in IE CSI-AssociatedReportConfigInfo

In RAN2 current understanding, the field applyIndicatedTCI-State may be configured per resource of per resource set, in which case each resource in a resource set may have different value for the field.

applyIndicatedTCI-State-r18    CHOICE {

perset-r18             ENUMERATED {first, second}

perresource-r18        SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF   ENUMERATED {first, second}

}

Question 1a:

RAN2 would like to confirm if this is the correct understanding?

Answer 1a:

Yes, this is the correct understanding. (there seems to be a typo in the question above – should be “per resource or per resource set”).

In IE CSI-AssociatedReportConfigInfo, the resourcesForChannel2 was restricted not to be used with unified TCI state. However, RAN2 assumes in Release-18 field resourcesForChannel2 is enabled with unified TCI state.

Question 1b:

RAN2 would like to confirm if the above is the correct understanding?

Answer 1b:

Yes, this is the correct understanding.

Question 1c:

If answer to Q1b is yes, RAN2 would like to further ask, whether the parameter applies to both resourcesForChannel and resourcesForChannel2 and whether same value/values are used or these should have separate configurations?

Answer 1c:

Two different parameters applyIndicatedTCI-State and applyIndicatedTCI-State2 can be applied to resourcesForChannel and resourceForChannel2, respectively.

- Simultaneous TCI state update/common TCI state update

In Release-17, there is feature for simultaneous unified TCI state update (based on simultaneousU-TCI-UpdateListx-r17).

Question 2a:

RAN2 would like to ask if this would apply also for Release-18 unified TCI state extension for mTRP? That is, whether the simultaneousU-TCI-UpdateListx-r17 is expected to apply to new Rel-18 MAC CEs?

Answer 2a:

RAN1 is still discussing on the question.

Question 2b:

Is there any restrictions in configuring the serving cells of one list for sDCI mTRP, mDCI mTRP or sTRP operation?

Answer 2b:

It has been agreed in RAN1 that cells operated as sDCI mTRP, mDCI mTRP and sTRP should not be configured in the same list, i.e., they should be configured with separate lists for simultaneous unified TCI state(s) update. Other than this, there is no further/other restriction(s).

Question 2c:

Is it correct understanding that the Rel-18 simultaneous TCI state update applies to both DL-only/Joint TCI state and ul-TCIState?

Answer 2c:

Yes, it is the correct understanding.

- Reference CC/BWP for TCI state list configurations

The reference CC/BWP includes the Rel-17 TCI state pool (a list of TCI states) for PDSCH/PDCCH/PUSCH/PUCCH. This is understood as signalling optimization for UL/DL or joint TCI state list configuration when UE is configured with unified TCI state operation.

Question 3a:

RAN2 would like to ask if this optimization of using reference CC/BWP should be applied to Release-18 features and if so which?

Answer 3a:

Yes, reference CC/BWP should be applied to Rel-18 eUTCI as well.

Question 3b:

If the response to Q3a is yes for 2TA operation for mDCI mTRP, RAN2 would like to ask how the tag_id_ptr per TCI state configuration should be understood here. whether the tag_id_ptr in the TCI state refers to the TAG of the serving cell where the TCI state is configured or to the TAG of the serving cell where the TCI state is applied?

Answer 3b:

The tag_id_ptr in the TCI state refers to the TAG of the serving cell where the TCI state is applied.

- Field n1-n2-codebookSubsetRestrictionList-R18(CJT) and n1-n2-codebookSubsetRestriction-r18(Doppler) in IE CodebookConfig

Furthermore, for both CJT and CJT-PS codebooks, RAN1 indicates in the RRC parameter list and in LS R1-2308396 that only hard amplitude restriction is supported (i.e., no soft amplitude restriction). This restriction has been capture in TS 38.214 clause 5.2.2.2.8 and 5.2.2.2.10 in the CR R1-2308716.

The bitmap parameter n1-n2-codebookSubsetRestriction-CJT-r18 is configured per CSI-RS resource and for at least one of the NTRP CSI-RS resources, and it is configured as described in Clause 5.2.2.2.5, where only the bit values ‘00’ or ‘11’ of Table 5.2.2.2.5-6 are configurable. If parameter n1-n2-codebookSubsetRestriction-CJT-r18 is not configured for a CSI-RS resource, no restriction is applied to the selection of vectors vm1,ji,m2,j(i) corresponding to that resource. The bitmap parameter n1-n2-codebookSubsetRestriction-Doppler-r18 is configured as described in Clause 5.2.2.2.5, where only the bit values ‘00’ or ‘11’ of Table 5.2.2.2.5-6 are configurable.

RAN2 understanding is that the following savings could be possible in IE CBSR-r18 which is used to configure values for Field n1-n2-codebookSubsetRestrictionList-R18(CJT) and n1-n2-codebookSubsetRestriction-r18(Doppler).

For example, for N1=8 and N2=2,  each beam restriction group contains N1N2=16 beams. A total of 139 bits are needed if  2bits are used for each beam as in legacy CBSR, i.e., B1=log2164= 11bits,  B2(k)=2×16=32bits, k=0,1,2,3.

eight-two                              BIT STRING (SIZE (139)),

When 1bit is used for each beam in CBSR for Rel-18 type II for CJT,  only 75 bits are needed, i.e., B1=11 bits,  B2(k)=1×16=16bits, k=0,1,2,3.  A saving of almost 50%

eight-two                              BIT STRING (SIZE (75)),

Question 4:

RAN2 would like to ask RAN1 to provide the exact values for the IE CBSR-r18 which currently has same values as in Release-16 field n1-n2-codebookSubsetRestriction-r16:

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (16)),

two-two                                BIT STRING (SIZE (43)),

four-one                               BIT STRING (SIZE (32)),

three-two                              BIT STRING (SIZE (59)),

six-one                                BIT STRING (SIZE (48)),

four-two                               BIT STRING (SIZE (75)),

eight-one                              BIT STRING (SIZE (64)),

four-three                             BIT STRING (SIZE (107)),

six-two                                BIT STRING (SIZE (107)),

twelve-one                             BIT STRING (SIZE (96)),

four-four                              BIT STRING (SIZE (139)),

eight-two                              BIT STRING (SIZE (139)),

sixteen-one                            BIT STRING (SIZE (128))

}

Answer 4:

If RAN2 decides to use 1 bit (to save RRC signaling overhead), the exact values for the IE CBSR-r18 will be as follows.

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (8)),

two-two                                BIT STRING (SIZE (27)),

four-one                               BIT STRING (SIZE (16)),

three-two                              BIT STRING (SIZE (35)),

six-one                                BIT STRING (SIZE (24)),

four-two                               BIT STRING (SIZE (43)),

eight-one                              BIT STRING (SIZE (32)),

four-three                             BIT STRING (SIZE (59)),

six-two                                BIT STRING (SIZE (59)),

twelve-one                             BIT STRING (SIZE (48)),

four-four                              BIT STRING (SIZE (75)),

eight-two                              BIT STRING (SIZE (75)),

sixteen-one                            BIT STRING (SIZE (64))

}

- CMR configurations for codebooks

The L1 parameter excel has the following rows related to how NZP CSI-RS resources are configured for using the Release-18 codebooks:

Row 21 numberOfCMR-r18 in IE CSI-ReportConfig

Row 33 cmrCJT-K-r18 in IE NZP-CSI-RS-ResourceSet

Row 47 cmrDopplerK-r18 in IE NZP-CSI-RS-ResourceSet

Question 5:

RAN2 would like to ask if the above parameters could be captured as field description restrictions for nzp-CSI-RS-Resources in IE NZP-CSI-RS-ResourceSet as configuration restrictions for the size of NZP-CSI-RS-ResourceSet?

Answer 5:

Yes, numberOfCMR-r18, cmrCJT-K-r18, and cmrDopplerK-r18 can be captured as field description restrictions for nzp-CSI-RS resources in IE NZP-CSI-RS-ResourceSet.

# 2	Actions

To RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above information into account for their future discussion.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1#116		February 26 – March 1, 2024	Athens, GR

TSG RAN WG1#117		April 15 – April 19, 2024 	TBD, CN