3GPP TSG-RAN WG2 Meeting #125	R2-2401777

Athens, Greece, 26th Feb – 1st March 2024

Agenda Item:	7.2.1

Source:	Ericsson

- Title:	 [Post125][409][POS] 38.331 Rel-18 positioning CR (Ericsson)

Document for:	Discussion, Decision

# Introduction

This is to kick off the email discussion.

[Post125][409][POS] 38.331 Rel-18 positioning CR (Ericsson)

Scope: Update and check the CR in R2-2401318.

Intended outcome: Agreed CR in R2-2401632

Deadline:  Short (for RP)

# 2	Discussion

## 2.1	LPHAP

Please provide your comments on the LPHAP changes

| Company Name | Comments |
| --- | --- |
| HW | 1/ if it is running should not be removed 2> if srs-PosRRC-InactiveValidityAreaConfigList is configured: 3> instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT; Rapportuer: right. 2/ The following change should be added to the paragraph for “suspendConfig” Rapporteur: right, done 1> if the RRCRelease provides configuration srs-PosRRC-InactiveValidityAreaPreConfigList: 2> store srs-PosRRC-InactiveValidityAreaConfig; 1> if the RRCRelease indicates release of srs-PosRRC-InactiveValidityAreaPreConfigList: 2> remove all srs-PosRRC-InactiveValidityAreaPreConfigList, if available; 3/ The pre-configured SRS should not be applied immediately after the configuration is received. It is pre-configured Rapporteur: Yes for this now we have: 2> if the RRCRelease provides configuration srs-PosRRC-InactiveValidityAreaPreConfigList: 3> store srs-PosRRC-InactiveValidityAreaConfig and apply when requested by upper layers; 2> if the RRCRelease indicates release of srs-PosRRC-InactiveValidityAreaPreConfigList: 3> remove all srs-PosRRC-InactiveValidityAreaPreConfigList, if available; 2> if srs-PosRRC-Inactive is configured: 3> apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer; 2> if srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 3> apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-ValidityAreaTAT; 4/ Not sure why we need to consider for the emergency services here or what is the correspoding agreement? 2> if the resumption of the RRC connection is triggered for activation of srs-PosRRC-InactiveValidityAreaPreConfigList when the UE is camped in one of the cells indicated in srs-PosConfigValidityArea; 3> if an emergency service is ongoing: 4> select '2' as the Access Category; 4> set the resumeCause to emergency; Rapporteur: This is following legacy approach which prioritizes emergency 5/ The following paragraph is not needed. The procedure is specified in the MAC spec when TAT is received in the RAR and contention resolution is successful. See Section 5.2 of TS 38.321 1> if the resume procedure is initiated with ResumeCause set to srs-PosConfigOrActivationReq for the activation of srs-PosRRC-InactiveValidityAreaPreConfigList: 2> instruct the MAC entity to start inactivePosSRS-ValidityAreaTAT, Rapporteur: Ok, 6/ These two "ifs“ can be merged since the UE procedures are the same 1> if srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 2> instruct the MAC entity to stop inactivePosSRS-ValidityAreaTAT, if it is running; 1> else if field srs-PosRRC-InactiveValidityAreaPreConfigList is configured and if the cell is not listed in srs-PosConfigValidityArea; 2> instruct the MAC entity to stop inactivePosSRS-ValidityAreaTAT, if it is running; Rapporteur: right, thanks. 7/ Editorials: The indentation is wrong The name is wrong 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured and if there is an on-going SRS for positioning transmission procedure in RRC_INACTIVE: 2> if the selected cell is not included in the srs-PosConfigValidityArea 3> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT; 3> initiate RRC connection resume procedure in 5.3.13.2; 2> else if the cell is included in the srs-PosRRC-InactiveValidityArea: 3> if the selected cell and the previously camped cell are in the same srs-PosConfigValidityArea; 4> if autonomousTA-AdjustmentEnabled is configured; 5> indicate to the lower layer to update Timing Advance and stored RSRP provided the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] is met; 4> instruct lower layers to continue transmitting SRS if Timing Advance validation condition as specified in TS 38.321 [3] are satisfied; 3> if the selected cell and previously camped cell are in the different srs-PosConfigValidityArea; 4> initiate RRC connection resume procedure in 5.3.13.2; 4> apply the SRS configuration srs-PosConfigValidityAreaConfig corresponding to the validity area of the selected cell and instruct lower layers to initiate SRS transmission. 8/ The sentence below is not needed. Nothing has changed in this case and the TA validation is performed for each SRS transmission 1> else if cell reselection occurs when srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig is configured and if there is an on-going SRS for positioning transmission procedure in RRC_INACTIVE: 2> if the selected cell is not included in the srs-PosConfigValidityArea 3> indicate to the lower layer to stop inactivePosSRS-ValidityAreaTAT; 3> initiate RRC connection resume procedure in 5.3.13.2; 2> else if the cell is included in the srs-PosRRC-InactiveValidityArea: 3> if the selected cell and the previously camped cell are in the same srs-PosConfigValidityArea; 4> if autonomousTA-AdjustmentEnabled is configured; 5> indicate to the lower layer to update Timing Advance and stored RSRP provided the Timing Advance validation requirements specified in clause 5.6.6.3 of TS 38.133 [14] is met; 4> instruct lower layers to continue transmitting SRS if Timing Advance validation condition as specified in TS 38.321 [3] are satisfied; 3> if the selected cell and previously camped cell are in the different srs-PosConfigValidityArea; 4> initiate RRC connection resume procedure in 5.3.13.2; 4> apply the SRS configuration srs-PosConfigValidityAreaConfig corresponding to the validity area of the selected cell and instruct lower layers to initiate SRS transmission. Rapporteur: If I understand, RAN4 has set the requirement that only if current TA is valid then only UE will do the update so that needs to be checked; this the UE may do after every cell reselection in validity area when autonomous TS adj is enabled. 9/ When SRS is not pre-configured and a cell is reselected out of the validity area, the SRS configuration should be released My general suggestion in the paragraph above is to separate the case of pre-configured and non-preconfigured, with srs-PosRRC-InactiveValidityAreaPreConfig or srs-PosRRC-InactiveValidityAreaNonPreConfig, respectively. Rapporteur: I agree with the general intention; however main goal is that the behaviour should be clearly reflected for these two cases. If there is any ambiguity we can disucss to further have different procedure description. So far in terms of cell reslection and if there is on going SRS Tx then behaviour is aligned. 10/ Prefer to add something like "SRS preiodicity configuration" into the name of the field to indicate that it is for periodicity configuration. This field is only needed when the resource type is configured as “periodic” Rapporteur: ok. srs-PosPeriodicConfigHyperSFN-Index-r18 ..., [[ srs-PosHyperSFN-Index-r18 ENUMERATED {even0, odd1} OPTIONAL, --Need R txHoppingConfig-r18 TxHoppingConfig-r18 OPTIONAL --Need R ]] } |
| ZTE001 | srs-PosRRC-InactiveValidityAreaPreConfigList-r18 SetupRelease { SRS-PosRRC-InactiveValidityAreaPreConfigList-r18 } OPTIONAL, -- Need M Do we agreed that SetupRelease structure should not be used in List? Rapporteur: No this Is fine. Only issue is that list with Need M should not have child field with also need M. |
| CATT001 | 5.3.3.4 Reception of the RRCSetup by the UE 2> if srs-PosRRC-InactiveValidityAreaConfigList is configured: 3> instruct the MAC entity to stop the inactivePosSRS-ValidityAreaTAT; There is no “srs-PosRRC-InactiveValidityAreaConfigList” in the spec. Should be srs-PosRRC-InactiveValidityAreaNonPreConfig? Rapportuer: Yes, right- |
| CATT002 | 5.3.8.3 Reception of the RRCRelease by the UE 1> if the RRCRelease provides configuration srs-PosRRC-InactiveValidityAreaPreConfigList: 2> store srs-PosRRC-InactiveValidityAreaConfig; 1> if the RRCRelease indicates release of srs-PosRRC-InactiveValidityAreaPreConfigList: 2> remove all srs-PosRRC-InactiveValidityAreaPreConfigList, if available; These procedures should be captured under the bullet “1> if the RRCRelease includes suspendConfig:” Additionally, the procedure should follow the setuprelease format, e.g. 1> if srs-PosRRC-InactiveValidityAreaPreConfigList is set to "setup": 2> store srs-PosRRC-InactiveValidityAreaPreConfigList; 1> else (srs-PosRRC-InactiveValidityAreaPreConfigList is set to "release"): 2> release srs-PosRRC-InactiveValidityAreaPreConfigList. Rapporteur: ok, thanks |
| CATT003 | 5.3.8.3 Reception of the RRCRelease by the UE 2> if srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: 3> apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-ValidityAreaTAT; The srs-PosRRC-InactiveValidityAreaNonPreConfig is configured with setuprelease, hence, the procedure should follow the setuprelease format, e.g. 2> if srs-PosRRC-InactiveValidityAreaNonPreConfig is set to "setup": 3> store srs-PosRRC-InactiveValidityAreaNonPreConfig. And apply the SRS for positioning configuration in RRC_INACTIVE and instruct MAC to start the inactivePosSRS-ValidityAreaTAT; 2> else (srs-PosRRC-InactiveValidityAreaNonPreConfig is set to "release"): 3> release srs-PosRRC-InactiveValidityAreaNonPreConfig. |
| CATT004 | 5.3.13.3 Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message 1> if the resume procedure is initiated with ResumeCause set to srs-PosConfigOrActivationReq for the activation of srs-PosRRC-InactiveValidityAreaPreConfigList: 2> instruct the MAC entity to start inactivePosSRS-ValidityAreaTAT, Rapporteur: This is the case when UE has already stored config and wants to activate. For this the UE would apply only after sending RRCResumeReq. However this is captured in MAC also; so we can delete this. This procedure is not align with the following agreement, and should be move to the section “Reception of the RRCRelease by the UE” For preconfigured SRS, when the UE moves to a new validity area, it does not continue transmitting SRS until it has gone through RRCResumeRequest/RRCRelease procedure. No additional acknowledgement message is needed for the activation request, i.e., the UE can apply the preconfiguration after it receives the RRCRelease. |
| CATT005 | 5.3.13.6 Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured 4> instruct lower layers to continue transmitting SRS if Timing Advance validation condition as specified in TS 38.321 [3] are satisfied; RRC cannot know whether the Timing Advance validation condition is satisfied. Rapporteur: yes this is instruction to lower layer and points to lower layer handling TS 38.321 |
| CATT006 | 5.3.13.6 Cell re-selection or cell selection or L2 U2N relay (re)selection while T390, T319 or T302 is running or SDT procedure is ongoing (UE in RRC_INACTIVE) or SRS transmission in RRC_INACTIVE is configured 3> if the selected cell and previously camped cell are in the different srs-PosConfigValidityArea; 4> initiate RRC connection resume procedure in 5.3.13.2; 4> apply the SRS configuration srs-PosConfigValidityAreaConfig corresponding to the validity area of the selected cell and instruct lower layers to initiate SRS transmission. According to the agreement, the UE should apply the preconfiguration after it receives the RRCRelease. |
| CATT007 | AreaValidityTA-Config-r18 ::= SEQUENCE { inactivePosSRS-ValidityAreaTAT-r18 ENUMERATED {ms1280, ms1920, ms2560, ms5120, ms10240, ms20480, ms40960, infinity}, inactivePosSRS-ValidityAreaRSRP-r18 RSRP-ChangeThreshold-r17 OPTIONAL, -- Need M autonomousTA-AdjustmentEnabled-r18 ENUMERATED {true} OPTIONAL -- Need M } We wonder whether there is necessity to configure these parameters as “need M”. Rapporteur: yes, since the parent is a list with need M, good to set it to Need R |
| CATT008 | srs-PosRRC-InactiveValidityAreaPreConfigList Contains the SRS for positioning configurations to be applied when a trigger for an event is met and which is valid across a number of cells comprising a validity area during RRC_INACTIVE state. For each validity area, the UE is preconfigured with only one SRS for positioning configuration. The below fields for the respective IEs are configured commonly in the validity area when srs-PosRRC-InactiveValidityAreaPreConfigList/ srs-PosRRC-InactiveValidityAreaNonPreConfig is configured: IE SRS-PosReseourceSet: srs-PosResourceSetId, srs-PosResourceSetIdList, srs-PosResourceIdList, resourceType, alpha, p0 IE SRS-PosResource: srs-PosResourceId, transmissionComb, resourceMapping, freqDomainShift, freqHopping, resourceType, groupOrSequenceHopping, sequenceID Maybe it is better to copy this text in the field description of srs-PosRRC-InactiveValidityAreaNonPreConfig |
| CATT009 | srs-PosHyperSFN-Index Indicates whether the current SFN is even or odd SFN for SRS for Positioning transmission. If this filed is absent, the UE assumes that SRS for positioning periodictity longer than one SFN is not configured. Should be hyper SFN. Rapportuer: yes, thanks |

## 2.2	Sidelink

Please provide your comments on Sidelink changes.

| Company Name | Comments |
| --- | --- |
| Intel | RAN1 sent LS R1-2401827, ask RAN2 to capture two new parameters „sl-ThreshS- RSSI-PRS-CBR“ and „sl-FilterCoefficient“, would be good to capture them in this version. Rapporteur: The two parameters has been added in SL-PRS-ResourcePool in the RRC. |
| vivo001 | 5.2.2.4.13/5.2.2.4.25/5.8.3.3 3> if configured to receive NR sidelink control information for SL-PRS measurement: From the perspective of RRC layer of UE, a relative higher layer to perform resource control, it cannot understand SCI of SL-PRS. It is PHY to actually receive SCI. Prefer to change it into a high-level description as “to receive SL-PRS”. Rapporteur: In the new version RRC, this has been revised as “to perform SL-PRS measurement” |
| vivo002 | 5.2.2.4.25 RAN2#125 meeting has agreed that “Support segmentation of SIB23”. Thus, UE should ensure having the latest version of SIB23. The following clause should be captured. “Upon receiving SIB23, the UE shall: 1> if the UE has stored at least one segment of SIB23 and the value tag of SIB23 has changed since a previous segment was stored: 2> discard all stored segments; 1> store the segment; 1> if all segments have been received: 2> assemble SIB23-IEs from the received segments; 2> if sl-PosFreqInfoList is included in sl-PosConfigCommonNR: …” Rapporteur: Thanks for the comment, this has been addressed in the new CR. |
| vivo003 | 5.5.3.1 3> if configured with NR sidelink positioning and the cell chosen for NR sidelink positioning provides SIB23 which includes sl-TxPoolSelectedNormal, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for the concerned frequency: 4> perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal, sl-TxPoolExceptional, sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB23; RAN2#125 meeting has agreed that “Configure the SL-PRS shared resource pool under SIB12 and the SL-PRS dedicated resource pool under SIB23.” The pool of resources for SL-PRS dedicated resource pool in SIB23 and shared resource pool in SIB12 should be respectively described. “3> if configured with NR sidelink positioning and the cell chosen for NR sidelink positioning provides SIB23 which includes sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional, or provides SIB12 which includes sl-TxPoolSelectedNormal, sl-TxPoolExceptional for the concerned frequency: 4> perform CBR measurement on pool(s) in sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolExceptional, sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency;” Rapporteur: Thanks for the comment, this has been addressed in the new CR. |
| vivo004 | Editorial change. 5.5.3.1 2> if the UE is in RRC_CONNECTED: 3> if tx-PoolMeasToAddModList is included in VarMeasConfig: 4> perform CBR measurements on each transmission resource pool indicated in the tx-PoolMeasToAddModList; 3> if sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling or sl-PRS-TxPoolExceptional is included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration: 4> perform CBR measurement on pool(s) in sl-DiscTxPoolSelected, sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal, sl-PRS-TxPoolScheduling and sl-PRS-TxPoolExceptional if included in sl-ConfigDedicatedNR for the concerned frequency within RRCReconfiguration; 3> else: 4> if configured with NR sidelink communication and the cell chosen for NR sidelink communication provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency; or 4> if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-TxPoolSelectedNormal or sl-TxPoolExceptional but does not provide sl-DiscTxPoolSelected for the concerned frequency: 5> perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-TxPoolExceptional for the concerned frequency in SIB12; 4> if configured with NR sidelink discovery and the cell chosen for NR sidelink discovery provides SIB12 which includes sl-DiscTxPoolSelected for the concerned frequency: 5> perform CBR measurement on pools in sl-DiscTxPoolSelected and sl-TxPoolExceptional for the concerned frequency in SIB12; 4> if configured with NR sidelink positioning and the cell chosen for NR sidelink positioning provides SIB23 which includes sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional, or provides SIB12 which includes sl-TxPoolSelectedNormal, sl-TxPoolExceptional for the concerned frequency: 5> perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal, sl-TxPoolExceptional, sl-PRS-TxPoolSelectedNormal or sl-PRS-TxPoolExceptional for the concerned frequency. Rapporteur: Thanks for the comment, this has been addressed in the new CR. |
| vivo005 | 5.5.3.1 2> if configured with NR sidelink positioning and sl-TxPoolSelectedNormal or sl-PRS-TxPoolSelectedNormal is included in SL-PreconfigurationNR for the concerned frequency: 3> perform CBR measurement on pool(s) in sl-TxPoolSelectedNormal or sl-PRS-TxPoolSelectedNormal in SidelinkPreconfigNR for the concerned frequency. RAN2#125 meeting(main session) has agreed that “=> rely on SL-PreconfigurationNR only and not define SL-PosPreconfigurationNR”. Therefore, SL-PRS dedicated resource pool should also be included in SidelinkPreconfigNR. Rapporteur: Thanks for the comment, SL-PosPreconfigurationNR has been removed and SL-PRS dedicated resource pool are included in SidelinkPreconfigNR in the new version RRC. |
| vivo006 | 5.8.3.3 5> set the sl-PosQoS-InfoList to include the SL-PRS transmission QoS profile configured by the upper layer; There is no such agreement on positioning QoS report to gNB in SUI. In our view, gNB is of no advantage to obtain the positioning QoS from UE. Prefer to remove it. Rapporteur: we have changed the IE name to sl-PRS-QoS-Infolist and SL-PRS-QoS-Info, to clearly indicate this is the QoS for SL-PRS transmission, and not for SL positioning QoS. |
| vivo007 | #### 5.8.18.3 NR sidelink positioning transmission RAN2#125 meeting has agreed that “Configure the SL-PRS shared resource pool under SIB12 and the SL-PRS dedicated resource pool under SIB23.” In this sense, a UE capable of NR sidelink positioning that is configured by upper layers to transmit SL-PRS shall listen to the frequency used for NR sidelink positioning configured in RRC dedicated msg and SIB12/23, in order to obtain information for both shared/dedicated SL-PRS resource pool. In summary, the description of UE performing SL-PRS transmission using sl-TxPoolSelectedNormal (i.e. shared resource pool) should also be included in this clause. Rapporteur: 5.8.18.3 has been updated to include sl-TxPoolSelectedNormal. |
| vivo008 | #### – SidelinkUEInformationNR SL-PosTxResourceReq-r18 ::= SEQUENCE { sl-PosDestinationIdentity-r18 SL-DestinationIdentity-r16, sl-PosCastType-r18 ENUMERATED {broadcast, groupcast, unicast, spare1}, sl-PosTxInterestedFreqList-r18 SL-TxInterestedFreqList-r16 OPTIONAL, sl-PosTypeTxSyncList-r18 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16 OPTIONAL, sl-PosQoS-InfoList-r18 SEQUENCE (SIZE (1..maxNrofSL-PRS-PerDest-r18)) OF SL-PosQoS-Info-r18 OPTIONAL, sl-PosCapabilityInformationSidelink-r18 OCTET STRING OPTIONAL, ... } Same comment as vivo006. We see no enhancement for gNB to get information of positioning QoS and positioning capability via SUI, since these parameters are transferred and exploited between UE and LMF. Apart from that, seeing the exact content of sl-PosQoS-InfoList, it contains priority and delay budget of SL-PRS, which has been agreed to contain in UAI for request SL grant for SL-PRS. It is redundant to repeatedly included in SUI. Prefer to remove the highlight part. Rapporteur: we have removed'Pos', and use sl-CapabilityInformationSidelink-r18, which is not used for indicating SL positioning capability, while it is already part of legacy which includes UE RRC capability and can be reused. |
| vivo009 | sl-PRS-BW Indicates the desired bandwidth of the requested SL-PRS resources in the unit of MHz. Should keep the name of IE aligned as sl-PRS-Bandwidth in ASN.1. Rapporteur: we have updated the IE name.` |
| HW001 | 1/ not sure why the following section has been removed. This section is the same as 5.3.3.1a, and we have an agreement for this previously. Rapporteur: 5.3.3.1a is for communication and has covered 5.3.3.1b since there is no SL-PRS transmission only UE. For UEs involved in the SL positioning session, SL communication is still needed and therefore 5.3.3.1a already covered the 5.3.3.1b. Suggest to remove 5.3.3.1b. Based upon RIL I151 2/ should use the field name for configured grant on dedicated resource pool (follow the legacy) 5> if T311 is running, configure the lower layers to release the resources indicated by rrc-ConfiguredSidelinkGrant (if any) and CG Type 1 dedicated SL-PRS resource pool; Rapporteur: CG Type1 word is used before and there is no legacy wording here since it is for SL-PRS CG Type1 but as such rrc-ConfiguredSidelinkGrant includes CG Type1 so we could also remove this. 3/ should be SIB12/Sib23 5> else if SIB23 includes sl-PRS-TxPoolExceptional or SIB12 includes sl-TxPoolExceptional for the concerned frequency: 6> from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or 6> if a result of full sensing, if selected and is allowed by sl-PosAllowedResourceSelectionConfig, on the resources configured in sl-PRS-TxPoolSelectedNormal or if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]: 7> configure lower layers to perform the sidelink resource allocation scheme 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-PRS-TxPoolExceptional or sl-TxPoolExceptional for the concerned frequency; 4/ should be r18 SIB23-r18 ::= SEQUENCE { segmentNumber-r16 INTEGER (0..63), segmentType-r18 ENUMERATED {notLastSegment, lastSegment}, segmentContainer-r18 OCTET STRING } Rapporteur: thanks corrected |
| Q001 | sl-PRS-Bandwidth-r18 INTEGER (1..100) OPTIONAL, sl-PRS-BW Indicates the desired bandwidth of the requested SL-PRS resources in the unit of MHz. The minimum bandwidth is 10 PRBs. It needs to be clarified what e.g., 1 MHz would mean. Bandwidth should better be specified in PRBs, as usual. Otherwise, we would need a new MAC-CE when e.g., FR2 or Unlicensed will be introduced. [Samsung]: If we indicate the requested SL-PRS BW in unit of PRBs, the absolute value of requested BW (in unit of MHz) can be interpreted differently with different assumed SCS (Sub-carrier spacing). Thus, the sl-PRS-BW should be indicated in unit of MHz to clearly specify the required BW based on the QoS requirement (e.g., accuracy). Otherwise, when the UE reports the requested BW to gNB in unit of PRB, the gNB would be confused on the size of requested BW since there can be multiple SL-PRS Tx resource pool having different SCS. For the specific value range & granularity for MHz unit, we prefer to leave it as open issue. Companies may internally check it with RAN1 and we can discuss it next meeting based on contributions. In our view, 1MHz-level granurality is not needed and the ENUMERATED type having multiple candidate values and some spare bits can be used to be future-proof. Rapporteur: Agree with Samsung. Lets keep as ENUMERATED with multiple values and can be discussed further next meeting. ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100} OPTIONAL, |
| ZTE001 | #### – UEAssistanceInformation SL-PRS-TxInfo-r18 ::= SEQUENCE { sl-PRS-Periodicity-r18 ENUMERATED {ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000, spare6, spare5, spare4, spare3, spare2, spare1}, sl-PRS-Priority-r18 INTEGER (1..8) OPTIONAL, sl-PRS-DelayBudget-r18 INTEGER (0..1023) OPTIONAL, sl-PRS-Bandwidth-r18 INTEGER (1..100) OPTIONAL, ... } These two is already included in SUI, no need to include in UAI again. Suggest to delete the yellow IEs in UAI Rapporteur: RAN2 agreed to keep in UAI for configured grant request |

## 2.3	Bandwidth Aggregation

Please provide your comments on the bandwidth aggregation changes.

| Company Name | Comments |
| --- | --- |
| HW | 1/ is there an agreement that CA positoining and SRS with validity area cannot be configured at the same time?? Rapporteur: No, but NW can anyway configure it; i.e there is no need to duplicate the BW aggreation IE inside SRS-PosRRC-InactiveValidityAreaConfig SRS-PosRRC-InactiveValidityAreaConfig-r18 ::= SEQUENCE { srs-PosConfigValidityArea-r18 SEQUENCE (SIZE(1..maxNrOfCellsInVA-r18)) OF CellIdentity, srs-PosConfigNUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R srs-PosConfigSUL-r18 SRS-PosConfig-r17 OPTIONAL, -- Need R bwp-NUL-r18 BWP OPTIONAL, -- Need S bwp-SUL-r18 BWP OPTIONAL, -- Need S areaValidityTA-Config-r18 AreaValidityTA-Config-r18 OPTIONAL, -- Need R ... } 2/ according to the feedback from RAN1 colleague, for CONECTED, RAN1 has agreed that the aggregated SRSs are configured under serving cell configuration. Then, the folllowing configuraiton is not needed for CONNECTED, while it is OK for INACITVE? We only need the serving cell index for CONNECTED. Rapporteur: Yes cell config can configure SRS config in multiple CC; but then RRC Reconfiguration should provide which SRS resource sets across the CC can be aggregated #### – SRS-PosResourceSetLinkedForAggBW The IE SRS-PosResourceSetLinkedForAggBW provides the SRS Positioning Resource Sets that are linked for bandwidth aggregation. SRS-PosResourceSetLinkedForAggBW information element -- ASN1START -- TAG- SRS-POSRESOURCESETLINKEDFORAGGBW-START SRS-PosResourceSetLinkedForAggBW-r18 ::= SEQUENCE { srs-PosResourceSetLinked-r18 SRS-PosResourceSetId-r16, freqInfo-r18 ARFCN-ValueNR OPTIONAL, -- Need R ul-bwp-ID-r18 BWP-Id OPTIONAL, -- Cond ConnectedMode scs-SpecificCarrier-r18 SCS-SpecificCarrier OPTIONAL, -- Need R ... } -- TAG- SRS-POSRESOURCESETLINKEDFORAGGBW-STOP -- ASN1STOP |
| ZTE001 | SRS-PosResourceSetLinkedForAggBWList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18 maxNrOfLinkedSRS-PosResourceSet-r18 INTEGER ::= 32 -- Maximum number of linked SRSPosResourceSets that can be aggregated across CCs In Athens meeting, RAN1 has already agreed with maximum set linkage per UE should be 32. However the current interpretation is not correct. It should be two level structure, see the following TP as example: RRCReconfiguration-v1800-IEs ::= SEQUENCE { srs-PosResourceSetLinkedGroupList-r18 SetupRelease { SRS-PosResourceSetLinkedGroupList-r18 } OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } SRS-PosResourceSetLinkedGroupList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResourceSetGroup-r18)) OF SRS-PosResourceSetLinkedForAggBWList-r18 SRS-PosResourceSetLinkedForAggBWList-r18 ::= SEQUENCE (SIZE(1..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18 maxNrOfLinkedSRS-PosResourceSet-r18 INTEGER ::= 3 -- Value is FFS Maximum number of linked SRSPosResourceSets per SRSPosResourceSets linked combinations maxNrOfLinkedSRS-PosResourceSetGroup-r18 INTEGER ::= 32 -- Value is Maximum number of SRSPosResourceSets linked combinations across CCs. Rapporteur: I had the impression that in RRC we will specify all combination then in MAC we will specify the exact one that needs to be activated. We can discuss in April meeting. |
| ZTE002 | SRS-PosRRC-AggBW-InactiveConfigList-r18 ::= SEQUENCE (SIZE (1..2)) OF SRS-PosResourceSetLinkedForAggBW-r18 This structure of size{1..2} strictly follows RAN1’s agreement that introduce additional 1 or 2 carriers, that have the linkage with the previous carrier (i.e., Rel-17 SRS carrier in RRC_INACTIVE). srs-PosRRC-AggBW-InactiveConfigList SRS for positioning configuration with bandwidth aggregation for RRC_INACTIVE state.. So in the above field description, it should be clarified that, if this IE is present, SRS-PosRRC-InactiveConfig-r17 should also be present, and the bwp-SUL-r17 of SRS-PosRRC-InactiveConfig-r17 should not be present. Rapporteur: ok updates as below: srs-PosRRC-AggBW-InactiveConfigList SRS for positioning configuration with additional one or two carrier(s) configuration where the primary carrier is provided by srs-PosRRC-Inactive-r17 for bandwidth aggregation and to be used in RRC_INACTIVE state. |

## 2.4	REDCAP CR

Please provide your comments on the RedCap changes

| Company Name | Comments |
| --- | --- |

## 2.5	Any other comments

Please provide any other comments below.

| Company Name | Comments |
| --- | --- |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References