3GPP TSG-RAN2 Meeting #125	R2-2401891

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4994 | rev | 3 | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE location information in NB-IoT RLF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the RAN2#124 meeting, RAN2 made the following agreement: - RAN2 will define a flag to indicate NB-IOT UEs to report location information (at the time of the failure) in the RLF report In the RAN2#124 meeting, two solutions were discussed for indicating NB-IOT UEs to report location information (at the time of the failure) in the RLF report, as follows: - Indication in the RRCConnectionReconfiguration-NB, - Indication in the UEInformationRequest-NB. This CR provides relevant changes to indicate NB-IOT UEs to report location information (at the time of the failure) in the RLF report in the RRCConnectionReconfiguration-NB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are summarized as follows: - Change in section 5.3.3.2 to release the indication (obtainLocationNB) for NB-IoT UEs upon RRC connection establishment initiation. - Change in section 5.3.3.4b for UE procedures when an indication (for location reporting in the RLF report) is sent in the RRCConnectionResume-NB. - Change in section 5.3.5.3 for UE procedures when an indication (for location reporting in the RLF report) is sent in the RRCConnectionReconfiguration-NB - Change in section 5.3.7.2 Release the indication (obtainLocationNB) for NB-IoT UEs upon RRC connection re-establishment - ASN.1 Change in section 6.7.2 for introducing a new flag in RRCConnectionReconfiguration-NB and added field description for the new IE. - ASN.1 Change in section 6.7.2 for introducing the flag in RRCConnectionResume-NB. - UE capability in section 6.7.3.6. Impact analysis Impacted functionality: RLF report for NB-IoT Inter-operability analysis: If the change is implemented by the UE, and not by the network, then NB-IoT UE will not report location information while network may expect UE to provide location information. If the change is implemented by the NW, and not by the UE, then UE may be reporting location in RLF report even though network is not expecting it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For NB-IoT UEs, there would be no indication from the network if UE should report location information in SON reports (i.e., RLF report). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2, 5.3.3.4a, 5.3.5.3, 5.3.7.2, 6.7.2, 6.7.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.306 CR 1879 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE or when upper layers request resume of an RRC connection or RRC layer requests resume of an RRC connection for, e.g. RNAU or reception of RAN paging while the UE is in RRC_INACTIVE.

Except for NB-IoT, upon initiation of the procedure, if the UE is connected to EPC, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1>	if SystemInformationBlockType2 contains acdc-BarringPerPLMN-List and the acdc-BarringPerPLMN-List contains an ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected ACDC-BarringPerPLMN entry for ACDC barring check (i.e. presence or absence of access barring parameters in this entry) irrespective of the acdc-BarringForCommon parameters included in SystemInformationBlockType2;

1>	else:

2>	in the remainder of this procedure use the acdc-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType2 for ACDC barring check;

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that EAB is applicable, upon which the procedure ends;

1>	if upper layers indicate that the RRC connection is subject to ACDC (see TS 24.301 [35]), SystemInformationBlockType2 contains BarringPerACDC-CategoryList, and acdc-HPLMNonly indicates that ACDC is applicable for the UE:

2>	if the BarringPerACDC-CategoryList contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers:

3>	select the BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers;

2>	else:

3>	select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList;

2>	stop timer T308, if running;

2>	perform access barring check as specified in 5.3.3.13, using T308 as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable due to ACDC, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating CS fallback:

2>	if SystemInformationBlockType2 includes ac-BarringForCSFB:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForCSFB as "AC barring parameter";

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback is applicable, due to ac-BarringForCSFB, upon which the procedure ends;

2>	else:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

3>	if access to the cell is barred:

4>	if timer T303 is not running, start T303 with the timer value of T306;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback and mobile originating calls is applicable, due to ac-BarringForMO-Data, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating MMTEL voice, mobile originating MMTEL video, mobile originating SMSoIP or mobile originating SMS; or

1>	if the UE is establishing the RRC connection after EPS fallback for IMS voice (see TS 23.502 [102]) was triggered in NR via RRCRelease with voiceFallbackIndication (see TS 38.331 [82]):

2>	if the UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVoice; or

2>	if the UE is establishing the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVideo; or

2>	if the UE is establishing the RRC connection for mobile originating SMSoIP or SMS and SystemInformationBlockType2 includes ac-BarringSkipForSMS:

3>	consider access to the cell as not barred;

2>	else:

3>	if establishmentCause received from higher layers is set to mo-Signalling (including the case that mo-Signalling is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the clause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

4>	if access to the cell is barred:

5>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

3>	if establishmentCause received from higher layers is set to mo-Data (including the case that mo-Data is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the clause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

4>	if access to the cell is barred:

5>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

5>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

6>	if timer T306 is not running, start T306 with the timer value of T303;

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

Upon initiation of the procedure, if the UE is connected to 5GC, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNAU:

2>	if an emergency service is ongoing:

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [95];

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

1>	if the UE is resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

2>	if the UE was configured with (NG)EN-DC:

3>	if the UE does not support maintaining SCG configuration upon connection resumption:

4>	perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

4>	release p-MaxEUTRA, if configured;

4>	release p-MaxUE-FR1, if configured;

4>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

3>	release otherConfig associated with the SCG, if configured;

3>	stop timers T346a, T346b, T346c, T346d and T346e associated with the SCG (see TS 38.331 [82], clause 7.1.1), if running;

2>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

3>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

2>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

2>	release reportProximityConfig and clear any associated proximity status reporting timer;

2>	release obtainLocationConfig, if configured;

2>	release bt-NameListConfig, if configured;

2>	release wlan-NameListConfig, if configured;

2>	release idc-Config, if configured;

2>	release sps-AssistanceInfoReport, if configured;

2>	release measSubframePatternPCell, if configured;

2>	if the UE was configured with DC:

3>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

2>	release naics-Info for the PCell, if configured;

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

2>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release ailc-BitConfig, if configured;

2>	release uplinkDataCompression, if configured;

2>	release overheatingAssistanceConfig and overheatingAssistanceConfigForSCG, if configured and stop timer T345, if running;

NOTE 1a:	The parameters and configurations are released from the UE Inactive AS context if the UE is resuming an RRC connection from RRC_INACTIVE.

1>	if the UE is establishing or resuming an RRC connection from a suspended RRC connection:

2>	if the UE has a stored pur-Config and the cell is different from the cell where pur-Config was provided:

3>	if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

3>	release pur-Config;

3>	discard previously stored pur-Config;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	if the UE is resuming an RRC connection from a suspended RRC connection:

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else if the UE is resuming an RRC connection from RRC_INACTIVE:

2>	set the variable pendingRnaUpdate to 'FALSE';

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else:

2>	if stored, discard the UE AS context, UE Inactive AS context and resumeIdentity;

2>	release rrc-InactiveConfig, if configured;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b; or

2>	if the UE is initiating CP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

1>	if stored, discard mt-EDT;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state or UEs in RRC_INACTIVE. However, the UE needs to perform system information acquisition upon cell re-selection.

For NB-IoT, upon initiation of the procedure, the UE shall:

1>	if the UE is connected to EPC:

2>	if the UE is establishing or resuming the RRC connection for mobile originating exception data; or

2>	if the UE is establishing or resuming the RRC connection for mobile originating data; or

2>	if the UE is establishing or resuming the RRC connection for delay tolerant access; or

2>	if the UE is establishing or resuming the RRC connection for mobile originating signalling;

3>	perform access barring check as specified in 5.3.3.14;

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable, upon which the procedure ends;

1>	if the UE is connected to 5GC:

2>	if the Access Category provided by the upper layers is different from '0':

3>	perform access barring check for per-NRSRP barring as specified in 5.3.3.14;

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

3>	else:

4>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

1>	if the UE is establishing or resuming an RRC connection:

2>	if the UE has a stored pur-Config and the cell is different from the cell where pur-Config was provided:

3>	if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

3>	release pur-Config;

3>	discard previously stored pur-Config;

2> release obtainLocationNB, if configured;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	start timer T300;

1>	if the UE is establishing an RRC connection:

2>	if stored, discard the UE AS context and resumeIdentity;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b; or

2>	if the UE is initiating CP transmission using PUR in accordance with conditions in 5.3.3.1c:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

1>	else if the UE is resuming an RRC connection:

2>	release schedulingRequestConfig, if configured;

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	if stored, discard mt-EDT;

NOTE 3:	Upon initiating the connection establishment or resumption procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

NOTE 4:	For EDT and transmission using PUR, upon initiating the connection establishment or resumption procedure, it is up to UE implementation whether to continue cell re-selection related measurements as well as cell re-selection evaluation and, if the conditions for cell re-selection are fulfilled, whether to perform cell re-selection as specified in 5.3.3.5.

NEXT CHANGE

#### 5.3.3.4a	Reception of the RRCConnectionResume by the UE

The UE shall:

1>	stop timer T300;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	stop T380 if running;

1>	if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT or for transmission using PUR:

2>	discard the stored UE AS context and resumeIdentity;

2>	if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for transmission using PUR:

3>	instruct the associated MAC entity to start timeAlignmentTimer;

1>	else:

2>	if resuming an RRC connection from a suspended RRC connection in EPC; or

2>	for NB-IoT, if resuming an RRC connection from a suspended RRC connection in 5GC and fullConfig is not present in the RRCConnectionResume message:

3>	restore the PDCP state and re-establish PDCP entities for SRB2, if configured with E-UTRA PDCP, and for all DRBs that are configured with E-UTRA PDCP;

3>	if drb-ContinueROHC is included:

4>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

4>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

3>	else:

4>	indicate to lower layers that stored UE AS context is used;

4>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

3>	if restoreMCG-SCells is included:

4>	restore the MCG SCell(s) configuration, if stored;

3>	else:

4>	release the MCG SCell(s) from the UE AS context, if stored;

3>	if restoreSCG is included:

4>	restore nr-SecondaryCellGroupConfig, if stored;

3>	else if the UE was configured with EN-DC:

4>	perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

4>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

3>	discard the stored UE AS context and resumeIdentity;

3>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

2>	else if the RRCConnectionResume message includes the fullConfig (i.e., for resuming an RRC connection from RRC_INACTIVE or for resuming a suspended RRC connection in 5GC):

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	else if resuming an RRC connection from RRC_INACTIVE:

3>	restore the following from the stored UE Inactive AS context:

-	MCG physical layer configuration,

-	MCG MAC configuration,

-	MCG RLC configuration,

-	PDCP configuration;

3>	if restoreMCG-SCells is included:

4>	restore the MCG SCell(s) configuration, if stored;

3>	else:

4>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

3>	if restoreSCG is included:

4>	restore nr-SecondaryCellGroupConfig, if stored;

3>	else if the UE was configured with NGEN-DC:

4>	perform MR-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

4>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

3>	discard the stored UE Inactive AS context;

3>	configure lower layers to consider the restored MCG and SCG SCell(s) (if any) to be in deactivated state;

3>	release the rrc-InactiveConfig, except ran-NotificationAreaInfo;

2>	else (i.e., except for NB-IoT for resuming a suspended RRC connection in 5GC):

3>	restore the physical layer configuration, the MAC configuration, the RLC configuration and the PDCP configuration from the stored UE AS context;

3>	discard the stored UE AS context and resumeIdentity;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10.0;

NOTE 1:	When performing the radio resource configuration procedure, for the physical layer configuration and the MAC Main configuration, the restored RRC configuration from the stored UE AS context is used as basis for the reconfiguration.

1>	if the received RRCConnectionResume includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionResume includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	if the received RRCConnectionResume includes the sCellGroupToReleaseList:

2>	perform SCell group release as specified in 5.3.10.3d;

1>	if the received RRCConnectionResume includes the sCellGroupToAddModList:

2>	perform SCell group addition or modification as specified in 5.3.10.3e;

1>	if the received RRCConnectionResume message includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3;

1>	if the received RRCConnectionResume message includes the sk-Counter:

2>	perform key update procedure as specified in TS 38.331 [82], clause 5.3.5.8;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	except if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT or for transmission using PUR:

2>	resume SRB2, SRB3 (if configured), and all DRBs, if any, including RBs configured with NR PDCP;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	if stored, discard the altFreqPriorities provided by the RRCConnectionRelease;

1>	if stored, discard the dedicated offset provided by the redirectedCarrierOffsetDedicated;

1>	if the RRCConnectionResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if T302 is running:

2>	stop timer T302;

2>	if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	stop timer T323, if running;

1>	if timer T331 is running:

2>	stop timer T331;

2>	perform the actions as specified in 5.6.20.3;

1>	if the UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18 or RRCConnectionResume is received in response to an RRCConnectionResumeRequest from RRC_INACTIVE:

2>	ignore the nextHopChainingCount value indicated in the RRCConnectionResume message;

1>	else:

2>	if resuming an RRC connection from a suspended RRC connection in EPC:

3>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionResume message, as specified in TS 33.401 [32];

3>	store the nextHopChainingCount value;

3>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

3>	request lower layers to verify the integrity protection of the RRCConnectionResume message, using the previously configured algorithm and the KRRCint key;

3>	if the integrity protection check of the RRCConnectionResume message fails:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

3>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

3>	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

3>	configure lower layers to resume ciphering and to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionResumeComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35] for E-UTRA/EPC and TS 24.501 [95] for E-UTRA/5GC) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	except for NB-IoT:

3>	if resuming an RRC connection from a suspended RRC connection:

4>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

5>	include rlf-InfoAvailable;

4>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableMBSFN;

4>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailable;

5>	if Bluetooth measurement results are included in the logged measurements the UE has available:

6>	include logMeasAvailableBT;

5>	if WLAN measurement results are included in the logged measurements the UE has available:

6>	include logMeasAvailableWLAN;

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

4>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

4>	if the UE has flight path information available:

5>	include flightPathInfoAvailable;

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include mobilityHistoryAvail;

3>	if the idleModeMeasurementReq is included in the RRCConnectionResume message:

4>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

5>	set the measResultListIdle-r16 in the RRCConnectionResumeComplete message to the value of measReportIdle-r15 in the VarMeasIdleReport;

5>	set the measResultListExtIdle in the RRCConnectionResumeComplete message to the value of measReportIdle-r16 in the VarMeasIdleReport, if available;

5>	set the measResultListIdleNR in the RRCConnectionResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

5>	discard the VarMeasIdleReport upon successful delivery of the RRCConnectionResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB2 contains idleModeMeasurements and the UE has E-UTRA idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB2 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

3>	if the RRCConnectionResume message includes nr-SecondaryCellGroupConfig:

4>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

2>	for NB-IoT:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

3>	if the UE is connected to EPC:

4>	if the UE has radio link failure information available in VarRLF-Report-NB and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report-NB:

5>	include rlf-InfoAvailable;

4>	if the UE has ANR measurements information available in VarANR-MeasReport-NB and if the RPLMN is included in plmn-IdentityList stored in VarANR-MeasReport-NB:

5>	include anr-InfoAvailable;

1>	if the UE is configured to operate in EN-DC as result of this procedure, forward upperLayerIndication to upper layers as if the UE has received this field from SIB2, otherwise indicate to upper layers the absence of this field;

1>	submit the RRCConnectionResumeComplete message to lower layers for transmission;

1>	for NB-IoT:

2>	if the received RRCConnectionResume message includes the obtainLocationNB:

3>	attempt to have detailed location information available for any RLF report;

NOTE x:	The UE is requested to attempt to have valid detailed location information available at the time of RLF. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

- the procedure ends.

NEXT CHANGE

#### 5.3.5.3	Reception of an RRCConnectionReconfiguration not including the mobilityControlInfo by the UE

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

1>	if the received RRCConnectionReconfiguration includes the daps-SourceRelease:

2>	reset source MCG MAC and release the source MCG MAC configuration;

2>	for each DAPS bearer:

3>	re-establish the RLC entity or entities for the source PCell;

3>	release the RLC entity or entities and the associated DTCH logical channel for the source PCell;

3>	reconfigure the PDCP entity to release DAPS, as specified in TS 36.323 [8];

2>	for each SRB:

3>	release the PDCP entity for the source PCell;

3>	release the RLC entity and the associated DCCH logical channel for the source PCell;

2>	release the physical channel configuration for the source PCell;

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	re-establish PDCP for SRB2 configured with E-UTRA PDCP entity and for all DRBs that are established and configured with E-UTRA PDCP, if any;

2>	re-establish RLC for SRB2 and for all DRBs that are established and configured with E-UTRA RLC, if any;

2>	if the RRCConnectionReconfiguration message includes the fullConfig:

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10.0;

NOTE 1:	Void

NOTE 2:	Void

1>	else:

2>	if the RRCConnectionReconfiguration message includes the radioResourceConfigDedicated:

3>	perform the radio resource configuration procedure as specified in 5.3.10.0;

NOTE 3:	If the RRCConnectionReconfiguration message includes the establishment of radio bearers other than SRB1, the UE may start using these radio bearers immediately, i.e. there is no need to wait for an outstanding acknowledgment of the SecurityModeComplete message.

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToReleaseList:

2>	perform SCell group release as specified in 5.3.10.3d;

1>	if the received RRCConnectionReconfiguration includes the sCellGroupToAddModList:

2>	perform SCell group addition or modification as specified in 5.3.10.3e;

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs configured with pdcp-Config and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

1>	if the received RRCConnectionReconfiguration includes the nr-Config and it is set to release: or

1>	if the received RRCConnectionReconfiguration includes endc-ReleaseAndAdd and it is set to TRUE:

2>	perform MR-DC release as specified in TS 38.331 [82], clause 5.3.5.10;

1>	if the received RRCConnectionReconfiguration includes the sk-Counter:

2>	perform key update procedure as specified in TS 38.331 [82], clause 5.3.5.7;

1>	if the received RRCConnectionReconfiguration includes the nr-SecondaryCellGroupConfig:

2>	perform NR RRC Reconfiguration as specified in TS 38.331 [82], clause 5.3.5.3;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionReconfiguration includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if this is the first RRCConnectionReconfiguration message after successful completion of the RRC connection re-establishment procedure:

2>	resume SRB2 and all DRBs that are suspended, if any, including RBs configured with NR PDCP;

NOTE 4:	The handling of the radio bearers after the successful completion of the PDCP re-establishment, e.g. the re-transmission of unacknowledged PDCP SDUs (as well as the associated status reporting), the handling of the SN and the HFN, is specified in TS 36.323 [8].

NOTE 5:	The UE may discard SRB2 messages and data that it receives prior to completing the reconfiguration used to resume these bearers.

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType1Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType1 message as specified in 5.2.2.7;

1>	if the received RRCConnectionReconfiguration includes the systemInformationBlockType2Dedicated:

2>	perfom the actions upon reception of the SystemInformationBlockType2 message as specified in 5.2.2.9;

1>	if the RRCConnectionReconfiguration message includes the dedicatedInfoNASList:

2>	forward each element of the dedicatedInfoNASList to upper layers in the same order as listed;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	if the RRCConnectionReconfiguration message includes the otherConfig:

2>	perform the other configuration procedure as specified in 5.3.10.9;

1>	if the received RRCConnectionReconfiguration message includes the obtainLocationNB:

2>	attempt to have detailed location information available for any RLF report;

NOTE x:	The UE is requested to attempt to have valid detailed location information available at the time of RLF. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the RRCConnectionReconfiguration message includes the sl-DiscConfig or sl-CommConfig:

2>	perform the sidelink dedicated configuration procedure as specified in 5.3.10.15;

1>	if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated:

2>	perform the V2X sidelink communication dedicated configuration procedure as specified in 5.3.10.15a;

NOTE 5a:	If the sl-V2X-ConfigDedicated was received embedded within an NR RRCReconfiguration message, the UE does not build an E-UTRA RRCConnectionReconfigurationComplete message for the received sl-V2X-ConfigDedicated.

1>	if the RRCConnectionReconfiguration message includes the sl-ConfigDedicatedForNR:

2>	perform the related procedures for NR sidelink communication in accordance with TS 38.331 [82], clause 5.3.5.14 and clause 5.5.2;

1>	if the RRCConnectionReconfiguration message includes wlan-OffloadInfo:

2>	perform the dedicated WLAN offload configuration procedure as specified in 5.6.12.2;

1>	if the RRCConnectionReconfiguration message includes rclwi-Configuration:

2>	perform the WLAN traffic steering command procedure as specified in 5.6.16.2;

1>	if the RRCConnectionReconfiguration message includes lwa-Configuration:

2>	perform the LWA configuration procedure as specified in 5.6.14.2;

1>	if the RRCConnectionReconfiguration message includes lwip-Configuration:

2>	perform the LWIP reconfiguration procedure as specified in 5.6.17.2;

1>	upon RRC connection establishment, if UE does not need UL gaps during continuous uplink transmission:

2>	configure lower layers to stop using UL gaps during continuous uplink transmission in FDD for RRCConnectionReconfigurationComplete message and subsequent uplink transmission in RRC_CONNECTED except for UL transmissions as specified in TS36.211 [21];

1>	if the RRCConnectionReconfiguration message includes the conditionalReconfiguration:

2>	perform conditional reconfiguration as specified in 5.3.5.9;

NOTE 6:	In case of conditional reconfiguration the text "if the received RRCConnectionReconfiguration. . ." corresponds to applying the stored RRCConnectionReconfiguration message (according to 5.3.5.9.5).

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest:

3>	include perCC-GapIndicationList and numFreqEffective;

2>	if the frequencies are configured for reduced measurement performance:

3>	include numFreqEffectiveReduced;

2>	if the received RRCConnectionReconfiguration message included nr-SecondaryCellGroupConfig:

3>	include scg-ConfigResponseNR in accordance with TS 38.331 [82], clause 5.3.5.3;

1>	if the UE is configured to operate in EN-DC as result of this procedure, forward upperLayerIndication, as if the UE receives this field from SIB2, to upper layers, otherwise indicate upper layers absence of this field;

1>	if the UE is configured with NE-DC:

2>	if the received RRCConnectionReconfiguration message was included in an NR RRCResume message:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCResumeComplete as specified in TS 38.331 [82], clause 5.3.13.4;

2>	else:

3>	transfer the RRCConnectionReconfigurationComplete message via SRB1 embedded in NR RRC message RRCReconfigurationComplete as specified in TS 38.331 [82], clause 5.3.5.3;

1>	else:

2>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration, upon which the procedure ends;

NEXT CHANGE

#### 5.3.7.2	Initiation

The UE shall only initiate the procedure either when AS security has been activated or for a NB-IoT UE supporting RRC connection re-establishment for the Control Plane CIoT EPS/5GS optimisation. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure and T316 is not configured, in accordance with 5.3.11; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.11; or

1>	upon detecting radio link failure of the MCG while NR PSCell change or PSCell addition is ongoing, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	except when resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18, upon integrity check failure indication from lower layers concerning SRB1 or SRB2; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5; or

1>	upon an RRC connection reconfiguration failure, in accordance with TS38.331 [82], clause 5.3.5.8; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with TS 38.331 [82] clause 5.3.10.3 in (NG)EN-DC; or

1>	upon SCG change failure while MCG transmission is suspended, in accordance with TS 38.331 [82] clause 5.3.5.8.3 in (NG)EN-DC; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with TS 38.331 [82] clause 5.3.5.8.2 in (NG)EN-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG transmission is suspended; or

1>	upon T316 expiry, in accordance with clause 5.6.26.5.

NOTE:	When resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18, integrity check failure indication from lower layers is handled in accordance with clause 5.3.3.16.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T313, if running;

1>	stop timer T316, if running;

1>	stop timer T307, if running;

1>	start timer T311;

1>	stop timer T370, if running;

1>	if the UE is not configured with conditionalReconfiguration:

2>	release uplinkDataCompression, if configured;

2>	suspend all RBs, including RBs configured with NR PDCP, except SRB0;

2>	reset MAC;

2>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

2>	release the SCell group(s), if configured, in accordance with 5.3.10.3d;

2>	apply the default physical channel configuration as specified in 9.2.4;

2>	except for NB-IoT, for the MCG, apply the default semi-persistent scheduling configuration as specified in 9.2.3;

2>	for NB-IoT, release schedulingRequestConfig, if configured;

2> for NB-IoT, release obtainLocationNB, if configured;

2>	for the MCG, apply the default MAC main configuration as specified in 9.2.2;

2>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

2>	release reportProximityConfig, if configured and clear any associated proximity status reporting timer;

2>	release obtainLocationConfig, if configured;

2>	release idc-Config, if configured;

2>	release sps-AssistanceInfoReport, if configured;

2>	release measSubframePatternPCell, if configured;

2>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

2>	if (NG)EN-DC is configured:

3>	perform MR-DC release, as specified in TS 38.331[82], clause 5.3.5.10;

3>	release p-MaxEUTRA, if configured;

3>	release p-MaxUE-FR1, if configured;

3>	release tdm-PatternConfig or tdm-PatternConfig2, if configured;

2>	release naics-Info for the PCell, if configured;

2>	if connected as an RN and configured with an RN subframe configuration:

3>	release the RN subframe configuration;

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

2>	release overheatingAssistanceConfig and overheatingAssistanceConfigForSCG, if configured and stop timer T345, if running;

2>	release ailc-BitConfig, if configured;

2>	if the UE has a stored pur-Config and the cell is different from the cell where pur-Config was provided:

3> if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

3>	release pur-Config;

3>	discard previously stored pur-Config.

1>	if any DAPS bearer is configured:

2>	release the MAC entity for the source PCell;

2>	for each DAPS bearer:

3>	re-establish the RLC entity for the source PCell;

3>	release the RLC entity and the associated DTCH logical channel for the source PCell;

3>	reconfigure the PDCP entity to release DAPS, as specified in TS 36.323 [8];

2>	for each SRB:

3>	release the PDCP entity for the source PCell;

3>	release the RLC entity and the associated DCCH logical channel for the source PCell;

2>	release the physical channel configuration for the source PCell;

1>	perform cell selection in accordance with the cell selection process as specified in TS 36.304 [4];

NEXT CHANGE

### 6.7.2	NB-IoT Message definitions

*/Omitted

#### –	RRCConnectionReconfiguration-NB

The RRCConnectionReconfiguration-NB message is the command to modify an RRC connection. It may convey information for resource configuration (including RBs, MAC main configuration and physical channel configuration) including any associated dedicated NAS information.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionReconfiguration-NB message

-- ASN1START

RRCConnectionReconfiguration-NB ::=	SEQUENCE {

rrc-TransactionIdentifier				RRC-TransactionIdentifier,

criticalExtensions						CHOICE {

c1										CHOICE{

rrcConnectionReconfiguration-r13		RRCConnectionReconfiguration-NB-r13-IEs,

spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionReconfiguration-NB-r13-IEs ::= SEQUENCE {

dedicatedInfoNASList-r13			SEQUENCE (SIZE(1..maxDRB-NB-r13)) OF

DedicatedInfoNAS		OPTIONAL,	-- Need ON

radioResourceConfigDedicated-r13	RadioResourceConfigDedicated-NB-r13	OPTIONAL,	-- Need ON

fullConfig-r13						ENUMERATED {true}					OPTIONAL,	-- Cond Reestab

lateNonCriticalExtension			OCTET STRING						OPTIONAL,

nonCriticalExtension				RRCConnectionReconfiguration-NB-v16f0-IEs	OPTIONAL

}

RRCConnectionReconfiguration-NB-v16f0-IEs ::=		SEQUENCE {

obtainLocationNB-r16			ENUMERATED {setup}						OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

RRCConnectionReconfiguration-NB field descriptions

dedicatedInfoNASList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

fullConfig Indicates the full configuration option is applicable for the RRC Connection Reconfiguration message.

| Conditional presence | Explanation |
| --- | --- |
| Reestab | This field is optionally present, need ON upon the first reconfiguration after RRC connection re-establishment; otherwise the field is not present. |

*/Omitted

#### –	RRCConnectionResume-NB

The RRCConnectionResume-NB message is used to resume the suspended RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionResume-NB message

-- ASN1START

RRCConnectionResume-NB ::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionResume-r13				RRCConnectionResume-NB-r13-IEs,

spare1								NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionResume-NB-r13-IEs ::=		SEQUENCE {

radioResourceConfigDedicated-r13		RadioResourceConfigDedicated-NB-r13	OPTIONAL,		-- Need ON

nextHopChainingCount-r13				NextHopChainingCount,

drb-ContinueROHC-r13					ENUMERATED {true}				OPTIONAL,	-- Need OP

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					RRCConnectionResume-NB-v1610-IEs	OPTIONAL

}

RRCConnectionResume-NB-v1610-IEs ::=	SEQUENCE {

fullConfig-r16							ENUMERATED {true}		OPTIONAL,	-- Cond 5GC

nonCriticalExtension					RRCConnectionResume-NB-v16f0-IEs				OPTIONAL

}

RRCConnectionResume-NB-v16f0-IEs ::=	SEQUENCE {

obtainLocationNB-r16					ENUMERATED {setup}		OPTIONAL,	-- Need OR

nonCriticalExtension					SEQUENCE {}				OPTIONAL

}

-- ASN1STOP

RRCConnectionResume-NB field descriptions

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues while absence indicates that the header compression protocol context is reset.

fullConfig Indicates that the full configuration option is applicable for the RRCConnectionResume-NB message.

| Conditional presence | Explanation |
| --- | --- |
| 5GC | The field is optionally present, Need ON, if the UE is connected to 5GC; otherwise the field is not present. |

NEXT CHANGE

#### 6.7.3.6	NB-IoT Other information elements

*/Omitted

#### –	UE-Capability-NB

The IE UE-Capability-NB is used to convey the NB-IoT UE Radio Access Capability Parameters, see TS 36.306 [5]. The IE UE-Capability-NB is transferred in NB-IoT only.

UE-Capability-NB information element

-- ASN1START

UE-Capability-NB-r13 ::=		SEQUENCE {

accessStratumRelease-r13		AccessStratumRelease-NB-r13,

ue-Category-NB-r13				ENUMERATED {nb1}					OPTIONAL,

multipleDRB-r13					ENUMERATED {supported}				OPTIONAL,

pdcp-Parameters-r13				PDCP-Parameters-NB-r13				OPTIONAL,

phyLayerParameters-r13			PhyLayerParameters-NB-r13,

rf-Parameters-r13				RF-Parameters-NB-r13,

dummy							SEQUENCE {}							OPTIONAL

}

UE-Capability-NB-Ext-r14-IEs ::=		SEQUENCE {

ue-Category-NB-r14					ENUMERATED {nb2}				OPTIONAL,

mac-Parameters-r14					MAC-Parameters-NB-r14			OPTIONAL,

phyLayerParameters-v1430			PhyLayerParameters-NB-v1430		OPTIONAL,

rf-Parameters-v1430					RF-Parameters-NB-v1430,

nonCriticalExtension				UE-Capability-NB-v1440-IEs		OPTIONAL

}

UE-Capability-NB-v1440-IEs ::=		SEQUENCE {

phyLayerParameters-v1440			PhyLayerParameters-NB-v1440		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v14x0-IEs		OPTIONAL

}

UE-Capability-NB-v14x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-14 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1530-IEs		OPTIONAL

}

UE-Capability-NB-v1530-IEs ::=		SEQUENCE {

earlyData-UP-r15					ENUMERATED {supported}			OPTIONAL,

rlc-Parameters-r15					RLC-Parameters-NB-r15,

mac-Parameters-v1530				MAC-Parameters-NB-v1530,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

tdd-UE-Capability-r15				TDD-UE-Capability-NB-r15		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v15x0-IEs		OPTIONAL

}

UE-Capability-NB-v15x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-15 extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v1610-IEs		OPTIONAL

}

UE-Capability-NB-v1610-IEs ::=		SEQUENCE {

earlySecurityReactivation-r16		ENUMERATED {supported}			OPTIONAL,

earlyData-UP-5GC-r16				ENUMERATED {supported}			OPTIONAL,

pur-Parameters-r16					PUR-Parameters-NB-r16			OPTIONAL,

mac-Parameters-v1610				MAC-Parameters-NB-v1610,

phyLayerParameters-v1610			PhyLayerParameters-NB-v1610		OPTIONAL,

son-Parameters-r16					SON-Parameters-NB-r16		OPTIONAL,

measParameters-r16					MeasParameters-NB-r16,

tdd-UE-Capability-v1610				TDD-UE-Capability-NB-v1610		OPTIONAL,

nonCriticalExtension				UE-Capability-NB-v16x0-IEs	OPTIONAL

}

UE-Capability-NB-v16x0-IEs ::=		SEQUENCE {

-- Following field is only to be used for late REL-16 extensions

lateNonCriticalExtension			OCTET STRING (CONTAINING UE-EUTRA-Capability-v16f0-IEs)					OPTIONAL,

nonCriticalExtension				SEQUENCE	{}					OPTIONAL

}

-- Late non-critical extensions

UE-EUTRA-Capability-v16f0-IEs ::=	SEQUENCE {

son-Parameters-v16f0				SON-Parameters-NB-v16f0,

nonCriticalExtension				SEQUENCE	{}					OPTIONAL

}

TDD-UE-Capability-NB-r15 ::=		SEQUENCE {

ue-Category-NB-r15					ENUMERATED {nb2}				OPTIONAL,

phyLayerParametersRel13-r15			PhyLayerParameters-NB-r13		OPTIONAL,

phyLayerParametersRel14-r15			PhyLayerParameters-NB-v1430		OPTIONAL,

phyLayerParameters-v1530			PhyLayerParameters-NB-v1530		OPTIONAL,

...

}

TDD-UE-Capability-NB-v1610 ::=		SEQUENCE {

slotSymbolResourceResvDL-r16			ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-r16				ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL

}

AccessStratumRelease-NB-r13 ::=		ENUMERATED {rel13, rel14, rel15, rel16, spare4, spare3, spare2, spare1, ...}

PDCP-Parameters-NB-r13		::= SEQUENCE {

supportedROHC-Profiles-r13			SEQUENCE {

profile0x0002						BOOLEAN,

profile0x0003						BOOLEAN,

profile0x0004						BOOLEAN,

profile0x0006						BOOLEAN,

profile0x0102						BOOLEAN,

profile0x0103						BOOLEAN,

profile0x0104						BOOLEAN

},

maxNumberROHC-ContextSessions-r13	ENUMERATED {cs2, cs4, cs8, cs12}	DEFAULT cs2,

...

}

RLC-Parameters-NB-r15		::=		SEQUENCE {

rlc-UM-r15							ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-r14		::=		SEQUENCE {

dataInactMon-r14					ENUMERATED {supported}					OPTIONAL,

rai-Support-r14						ENUMERATED {supported}				OPTIONAL

}

MAC-Parameters-NB-v1530		::=		SEQUENCE {

sr-SPS-BSR-r15						ENUMERATED {supported}			OPTIONAL

}

MAC-Parameters-NB-v1610		::=		SEQUENCE {

rai-SupportEnh-r16					ENUMERATED {supported}			OPTIONAL

}

MeasParameters-NB-r16		::=		SEQUENCE {

dl-ChannelQualityReporting-r16		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-r13	::=		SEQUENCE {

multiTone-r13						ENUMERATED {supported}			OPTIONAL,

multiCarrier-r13						ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1430	::=		SEQUENCE {

multiCarrier-NPRACH-r14				ENUMERATED {supported}			OPTIONAL,

twoHARQ-Processes-r14				ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1440	::=		SEQUENCE {

interferenceRandomisation-r14		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1530	::=		SEQUENCE {

mixedOperationMode-r15				ENUMERATED {supported}			OPTIONAL,

sr-WithHARQ-ACK-r15					ENUMERATED {supported}			OPTIONAL,

sr-WithoutHARQ-ACK-r15				ENUMERATED {supported}			OPTIONAL,

nprach-Format2-r15					ENUMERATED {supported}			OPTIONAL,

additionalTransmissionSIB1-r15		ENUMERATED {supported}			OPTIONAL,

npusch-3dot75kHz-SCS-TDD-r15		ENUMERATED {supported}			OPTIONAL

}

PhyLayerParameters-NB-v1610	::=		SEQUENCE {

npdsch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npdsch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-r16					ENUMERATED {supported}			OPTIONAL,

npusch-MultiTB-Interleaving-r16		ENUMERATED {supported}			OPTIONAL,

multiTB-HARQ-AckBundling-r16		ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvDL-r16			ENUMERATED {supported}			OPTIONAL,

slotSymbolResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL,

subframeResourceResvDL-r16				ENUMERATED {supported}			OPTIONAL,

subframeResourceResvUL-r16			ENUMERATED {supported}			OPTIONAL

}

PUR-Parameters-NB-r16	::=			SEQUENCE {

pur-CP-EPC-r16						ENUMERATED {supported}			OPTIONAL,

pur-CP-5GC-r16						ENUMERATED {supported}			OPTIONAL,

pur-UP-EPC-r16						ENUMERATED {supported}			OPTIONAL,

pur-UP-5GC-r16						ENUMERATED {supported}			OPTIONAL,

pur-NRSRP-Validation-r16			ENUMERATED {supported}			OPTIONAL,

pur-CP-L1Ack-r16					ENUMERATED {supported}			OPTIONAL

}

RF-Parameters-NB-r13	::=			SEQUENCE {

supportedBandList-r13				SupportedBandList-NB-r13,

multiNS-Pmax-r13					ENUMERATED {supported}		OPTIONAL

}

RF-Parameters-NB-v1430 ::=			SEQUENCE {

powerClassNB-14dBm-r14				ENUMERATED {supported}		OPTIONAL

}

SupportedBandList-NB-r13 ::=		SEQUENCE (SIZE (1..maxBands)) OF SupportedBand-NB-r13

SupportedBand-NB-r13	::=			SEQUENCE {

band-r13							FreqBandIndicator-NB-r13,

powerClassNB-20dBm-r13				ENUMERATED {supported}		OPTIONAL

}

SON-Parameters-NB-r16 ::=			SEQUENCE {

anr-Report-r16						ENUMERATED {supported}		OPTIONAL,

rach-Report-r16						ENUMERATED {supported}		OPTIONAL

}

SON-Parameters-NB-v16f0 ::=			SEQUENCE {

locationInfo-r16					ENUMERATED {supported}				OPTIONAL

}

-- ASN1STOP

| UE-Capability-NB field descriptions | FDD/TDD appl | FDD/TDD diff |
| --- | --- | --- |
| accessStratumRelease Set to rel16 in this version of the specification. | FDD/TDD | No |
| additionalTransmissionSIB1 Indicates whether the UE supports additional SIB1 transmission as specified in TS 36.213 [23]. | FDD | - |
| anr-Report Indicates whether the UE supports ANR measurements in RRC_IDLE. | FDD/TDD | No |
| dataInactMon Indicates whether the UE supports the data inactivity monitoring as specified in TS 36.321 [6]. | FDD/TDD | No |
| dl-ChannelQualityReporting-r16 Indicates whether the UE supports DL channel quality reporting in connected mode as specified in TS 36.321 [6]. | FDD | - |
| dummy This field is not used in the specification. It shall not be sent by the UE. | NA | NA |
| earlyData-UP, earlyData-UP-5GC Indicates whether the UE supports EDT for User plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and 24.501 [95] respectively. | FDD | - |
| earlySecurityReactivation Indicates whether the UE supports early security reactivation when resuming a suspended RRC connection. | FDD/TDD | No |
| interferenceRandomisation For FDD: Indicates whether the UE supports interference randomisation in connected mode as defined in TS.36.211 [21]. | FDD | - |
| locationInfo Indicates whether the UE supports reporting of locationInfo in RLF report. | FDD/TDD | No |
| maxNumberROHC-ContextSessions Set to the maximum number of concurrently active ROHC contexts supported by the UE, excluding context sessions that leave all headers uncompressed. cs2 corresponds with 2 (context sessions), cs4 corresponds with 4 and so on. The network ignores this field if the UE supports none of the ROHC profiles in supportedROHC-Profiles. | FDD/TDD | No |
| mixedOperationMode Defines whether the UE supports multi-carrier operation with mixed operation mode, standalone or inband/guardband, between the anchor carrier and the non-anchor carrier for unicast, paging, and random access as specified in TS 36.300 [9]. | FDD | - |
| multiCarrier Defines whether the UE supports multi -carrier operation. | FDD/TDD | Yes |
| multicarrier-NPRACH Defines whether the UE supports NPRACH on non-anchor carrier as specified in TS 36.321 [6]. | FDD/TDD | Yes |
| multipleDRB Defines whether the UE supports multiple DRBs. | FDD/TDD | No |
| multiNS-Pmax Defines whether the UE supports the mechanisms defined for NB-IoT cells broadcasting NS-PmaxList-NB. | FDD/TDD | No |
| multiTB-HARQ-AckBundling Indicates whether the UE supports HARQ ACK bundling for interleaved transmission for DL. If multiTB-HARQ-AckBundling is included, the UE shall also indicate support for npdsch-MultiTB-Interleaving. | FDD | - |
| multiTone Defines whether the UE supports UL multi-tone transmissions on NPUSCH. | FDD/TDD | Yes |
| npdsch-MultiTB Indicates whether the UE supports multiple TBs scheduling in RRC_CONNECTED for DL. If npdsch-MultiTB is included, the UE shall also indicate support for twoHARQ-Processes. | FDD | - |
| npdsch-MultiTB-Interleaving Indicates whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for DL. | FDD | - |
| nprach-Format2 Defines whether the UE supports NPRACH resources using preamble format 2. | FDD | - |
| npusch-3dot75kHz-SCS-TDD Indicates whether the UE supports NPUSCH with 3.75kHz SCS for TDD. | TDD | - |
| npusch-MultiTB Indicates whether the UE supports multiple TBs scheduling in RRC_CONNECTED for UL. If npusch-MultiTB is included, the UE shall also indicate support for twoHARQ-Processes. | FDD | - |
| npusch-MultiTB-Interleaving Indicates whether the UE supports interleaved transmission when multiple TBs is scheduled in RRC_CONNECTED for UL. | FDD | - |
| powerClassNB-14dBm Defines whether the UE supports power class 14dBm in all the bands supported by the UE as specified in TS 36.101 [42]. If powerClassNB-20dBm is included, the UE shall not include the field powerClassNB-14dBm. | FDD/TDD | No |
| powerClassNB-20dBm Defines whether the UE supports power class 20dBm in NB-IoT for the band, as specified in TS 36.101 [42]. If neither powerClassNB-14dBm nor powerClassNB-20dBm is included, UE supports power class 23 dBm in the NB-IoT band. | FDD/TDD | No |
| pur-CP-EPC, pur-CP-5GC Indicates whether the UE supports transmission using PUR for Control plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and TS 24.501 [95] respectively. | FDD | - |
| pur-CP-L1Ack Indicates whether UE supports L1 acknowledgement in response to CP transmission using PUR. If pur-CP-L1Ack is included, the UE shall also indicate support for pur-CP-EPC or pur-CP-5GC. | FDD | - |
| pur-NRSRP-Validation Indicates whether UE supports serving cell NRSRP for TA validation for transmission using PUR. If pur-NRSRP-Validation is included, the UE shall also indicate support for pur-CP-EPC, pur-CP-5GC, pur-UP-EPC or pur-CP-5GC. | FDD | - |
| pur-UP-EPC, pur-UP-5GC Indicates whether the UE supports transmission using PUR for User plane CIoT EPS/5GS optimisations, as defined in TS 24.301 [35] and TS 24.501 [95] repectively. | FDD | - |
| rach-Report Indicates whether the UE supports delivery of rach-Report. | FDD/TDD | No |
| rai-Support Defines whether the UE supports release assistance indication (RAI) as specified in TS 36.321 [6]. | FDD/TDD | No |
| rai-SupportEnh Indicates whether the UE supports AS Release Assistance Indication via the DCQR and AS RAI MAC CE when connected to EPC as specified in TS 36.321 [6]. | FDD/TDD | No |
| rlc-UM Defines whether the UE supports RLC UM as specified in TS 36.322 [7]. | FDD/TDD | No |
| slotSymbolResourceResvDL Indicates whether the UE supports slot/symbol-level time-domain DL resource reservation, e.g. for NB-IoT coexistence with NR. If slotSymbolResourceResvDL is included, the UE shall also indicate support for subframeResourceResvDL. | FDD/TDD | Yes |
| slotSymbolResourceResvUL Indicates whether the UE supports slot/symbol-level time-domain UL resource reservation, e.g. for NB-IoT coexistence with NR. If slotSymbolResourceResvUL is included, the UE shall also indicate support for subframeResourceResvUL. | FDD/TDD | Yes |
| supportedBandList Includes the supported NB-IoT bands as defined in TS 36.101 [42]. | FDD/TDD | No |
| sr-SPS-BSR Defines whether the UE supports SR using SPS BSR as specified in TS 36.321 [6]. | FDD | - |
| sr-withHARQ-ACK Defines whether the UE supports physical layer SR with HARQ ACK as specified in TS 36.213 [23]. | FDD | - |
| sr-withoutHARQ-ACK Defines whether the UE supports physical layer SR without HARQ ACK as specified in TS 36.211 [21] and TS 36.213 [23]. | FDD | - |
| subframeResourceResvDL Indicates whether the UE supports subframe-level time-domain DL resource reservation, e.g. for NB-IoT coexistence with NR. | FDD/TDD | Yes |
| subframeResourceResvUL Indicates whether the UE supports subframe-level time-domain UL resource reservation, e.g. for NB-IoT coexistence with NR. | FDD/TDD | Yes |
| supportedROHC-Profiles List of supported ROHC profiles as defined in TS 36.323 [8]. | FDD/TDD | No |
| twoHARQ-Processes Defines whether the UE supports two HARQ processes operation in DL and UL as specified in TS 36.212 [22] and TS 36.213 [23]. | FDD/TDD | Yes |
| ue-Category-NB UE category as defined in TS 36.306 [5]. Value nb1 corresponds to UE category NB1, value nb2 corresponds to UE category NB2. A UE shall always include the field ue-Category-NB-r13 in this version of the specification. | FDD/TDD | Yes |

NOTE 1:	The IE UE-Capability-NB does not include AS security capability information, since these are the same as the security capabilities that are signalled by NAS. Consequently AS need not provide "man-in-the-middle" protection for the security capabilities.

NOTE 2:	The column 'FDD/TDD appl' indicates the applicability to the xDD mode: 'FDD' means applicable to FDD only, 'TDD' means applicable to TDD only and 'FDD/TDD' means applicable to FDD and TDD.

NOTE 3:	The column 'FDD/TDD diff' indicates if the UE is allowed to signal a different value for FDD and TDD when the capability applies to both FDD and TDD modes. '-' is used when the capability applies to one mode only, 'No' is used for dual mode capabilities where a common value is signalled for both modes, and 'Yes' is used for dual mode capabilities where a separate value is signalled for each mode. Common capabilities and FDD capabilities are reported in the fields of UE-Capability-NB except field tdd-UE-Capability. TDD capabilities are reported in tdd-UE-Capability.

END OF CHANGE