3GPP TSG-RAN2 Meeting #125	R2-2402018

Athens, GR, Feb. 26th – Mar. 1st, 2024	was R3-241199

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0131 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 37.320 on the user consent for trace reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Huawei, CMCC, Ericsson, Interdigtal, ZTE, Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to local regulation/law, some MDT data type may not require user consent, while current user consent mechanism in MDT doesn’t provide a possibility for the operators to collect and handle a specific type of information/data, which is not subject to use consent, from a subscriber for a particular purpose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that, a UE is configured with an MDT PLMN List if user consent is valid for the RPLMN, or if the data types of the configured MDT measurements are not subject to user consent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User consent is mandatory for all data type in MDT collection, which might be no necessary since local regulation/law doesn’t require user consent for every data type in MDT collection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-241199. |  |  |  |  |  |  |  |  |

Start of Change

### 5.1.3	MDT Initiation

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422 [6].

For signalling based MDT within a PLMN, the CN shall not initiate MDT towards a particular user unless it is allowed.

For management based MDT within a PLMN, the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user considering e.g. user consent and roaming status (see TS 32.422 [6]), by providing management based MDT allowed information. For MDT measurements that are not subject to user consent, the RAN may initiate MDT towards a particular UE independently of user consent. For E-UTRAN/UTRAN, the MDT allowed information consists of the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List. For NR, the MDT allowed information only consists of the Management Based MDT PLMN List. The management based MDT allowed information propagates during inter-PLMN handover or inter-PLMN UE context retrieval if the Management Based MDT PLMN List is available and includes the target PLMN.

A UE is configured with an MDT PLMN List if user consent is valid for the RPLMN, or if none of the MDT measurements in the MDT configuration are subject to user consent.

User consent does not apply if the UE is served by an SNPN.

End of Change