3GPP TSG-RAN WG2 Meeting #125	R2-2402028

# Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4445 | rev | 3 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability for Enhanced channel raster |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In LSs in R2-2400056/R4-2321730 and R4-2403636, RAN4 indicated that a new UE capability for enhanced UE channel raster with 10 kHz granularity should be specified: - The capability should be indicated per band and be applicable for all TN and NTN FR1 bands below 3GHz with a 100 kHz channel raster. - The capability is not applicable for bands within FR2-1/FR2-2. - RAN4 also prefers that the capability be considered for early implementation from Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New capability enhancedChannelRaster-r18 is introduced per NR band. Implementation of this CR from Rel-16 will not cause interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot indicate support of enhanced channel raster. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.306 CR0994r3 |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2312819, R4-2400841, R2-2401603 |  |  |  |  |  |  |  |  |

### 6.3.3	UE capability information elements

<cut>

#### –	RF-Parameters

The IE RF-Parameters is used to convey RF-related capabilities for NR operation.

RF-Parameters information element

-- ASN1START

-- TAG-RF-PARAMETERS-START

RF-Parameters ::=                                   SEQUENCE {

supportedBandListNR                                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList                        BandCombinationList                         OPTIONAL,

appliedFreqBandListFilter                           FreqBandList                                OPTIONAL,

...,

[[

supportedBandCombinationList-v1540                  BandCombinationList-v1540                   OPTIONAL,

srs-SwitchingTimeRequested                          ENUMERATED {true}                           OPTIONAL

]],

[[

supportedBandCombinationList-v1550                  BandCombinationList-v1550                   OPTIONAL

]],

[[

supportedBandCombinationList-v1560                  BandCombinationList-v1560                   OPTIONAL

]],

[[

supportedBandCombinationList-v1610                  BandCombinationList-v1610                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-r16    BandCombinationListSidelinkEUTRA-NR-r16     OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-r16     BandCombinationList-UplinkTxSwitch-r16      OPTIONAL

]],

[[

supportedBandCombinationList-v1630                  BandCombinationList-v1630                   OPTIONAL,

supportedBandCombinationListSidelinkEUTRA-NR-v1630  BandCombinationListSidelinkEUTRA-NR-v1630   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1630   BandCombinationList-UplinkTxSwitch-v1630    OPTIONAL

]],

[[

supportedBandCombinationList-v1640                  BandCombinationList-v1640                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1640   BandCombinationList-UplinkTxSwitch-v1640    OPTIONAL

]],

[[

supportedBandCombinationList-v1650                  BandCombinationList-v1650                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1650   BandCombinationList-UplinkTxSwitch-v1650    OPTIONAL

]],

[[

extendedBand-n77-r16                                ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-UplinkTxSwitch-v1670   BandCombinationList-UplinkTxSwitch-v1670    OPTIONAL

]],

[[

supportedBandCombinationList-v1680                  BandCombinationList-v1680                   OPTIONAL

]],

[[

supportedBandCombinationList-v1690                  BandCombinationList-v1690                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1690   BandCombinationList-UplinkTxSwitch-v1690    OPTIONAL

]],

[[

supportedBandCombinationList-v1700                  BandCombinationList-v1700                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1700   BandCombinationList-UplinkTxSwitch-v1700    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-r17   OCTET STRING                                OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-NonRelayDiscovery-r17 OCTET STRING                               OPTIONAL,  -- Contains PC5 BandCombinationListSidelinkNR-r16

supportedBandCombinationListSidelinkEUTRA-NR-v1710  BandCombinationListSidelinkEUTRA-NR-v1710   OPTIONAL,

sidelinkRequested-r17                               ENUMERATED {true}                           OPTIONAL,

extendedBand-n77-2-r17                              ENUMERATED {supported}                      OPTIONAL

]],

[[

supportedBandCombinationList-v1720                  BandCombinationList-v1720                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1720   BandCombinationList-UplinkTxSwitch-v1720    OPTIONAL

]],

[[

supportedBandCombinationList-v1730                  BandCombinationList-v1730                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1730   BandCombinationList-UplinkTxSwitch-v1730    OPTIONAL,

supportedBandCombinationListSL-RelayDiscovery-v1730 BandCombinationListSL-Discovery-r17         OPTIONAL,

supportedBandCombinationListSL-NonRelayDiscovery-v1730 BandCombinationListSL-Discovery-r17      OPTIONAL

]],

[[

supportedBandCombinationList-v1740                  BandCombinationList-v1740                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1740   BandCombinationList-UplinkTxSwitch-v1740    OPTIONAL

]],

[[

supportedBandCombinationList-v1760                  BandCombinationList-v1760                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1760   BandCombinationList-UplinkTxSwitch-v1760    OPTIONAL

]],

[[

supportedBandCombinationList-v1770                  BandCombinationList-v1770                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1770   BandCombinationList-UplinkTxSwitch-v1770    OPTIONAL

]],

[[

supportedBandCombinationList-v1800                  BandCombinationList-v1800                   OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v1800   BandCombinationList-UplinkTxSwitch-v1800    OPTIONAL,

supportedBandCombinationListSL-U2U-Relay-r18        SEQUENCE {

supportedBandCombinationListSL-U2U-RelayDiscovery-r18  OCTET STRING             OPTIONAL,  -- Contains PC5

-- BandCombinationListSidelinkNR-r16

supportedBandCombinationListSL-U2U-DiscoveryExt BandCombinationListSL-Discovery-r17         OPTIONAL

}                                                                                               OPTIONAL

]]

}

RF-Parameters-v15g0 ::=                   SEQUENCE {

supportedBandCombinationList-v15g0        BandCombinationList-v15g0                   OPTIONAL

}

RF-Parameters-v16a0 ::=                            SEQUENCE {

supportedBandCombinationList-v16a0                 BandCombinationList-v16a0                    OPTIONAL,

supportedBandCombinationList-UplinkTxSwitch-v16a0  BandCombinationList-UplinkTxSwitch-v16a0     OPTIONAL

}

RF-Parameters-v16c0 ::=                            SEQUENCE {

supportedBandListNR-v16c0                          SEQUENCE (SIZE (1..maxBands)) OF BandNR-v16c0

}

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL                       CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]],

[[

channelBWs-DL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL,

channelBWs-UL-v1590                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (16))              OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (16))              OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (8))               OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (8))               OPTIONAL

}

}                                                                               OPTIONAL

]],

[[

asymmetricBandwidthCombinationSet     BIT STRING (SIZE (1..32))           OPTIONAL

]],

[[

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-r16 SharedSpectrumChAccessParamsPerBand-r16 OPTIONAL,

-- R1 11-7b: Independent cancellation of the overlapping PUSCHs in an intra-band UL CA

cancelOverlappingPUSCH-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-1: Multiple LTE-CRS rate matching patterns

multipleRateMatchingEUTRA-CRS-r16       SEQUENCE {

maxNumberPatterns-r16               INTEGER (2..6),

maxNumberNon-OverlapPatterns-r16    INTEGER (1..3)

}                                                                               OPTIONAL,

-- R1 14-1a: Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier

overlapRateMatchingEUTRA-CRS-r16        ENUMERATED {supported}                  OPTIONAL,

-- R1 14-2: PDSCH Type B mapping of length 9 and 10 OFDM symbols

pdsch-MappingTypeB-Alt-r16              ENUMERATED {supported}                  OPTIONAL,

-- R1 14-3: One slot periodic TRS configuration for FR1

oneSlotPeriodicTRS-r16                  ENUMERATED {supported}                  OPTIONAL,

olpc-SRS-Pos-r16                        OLPC-SRS-Pos-r16                        OPTIONAL,

spatialRelationsSRS-Pos-r16             SpatialRelationsSRS-Pos-r16             OPTIONAL,

simulSRS-MIMO-TransWithinBand-r16       ENUMERATED {n2}                         OPTIONAL,

channelBW-DL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                          SEQUENCE {

scs-60kHz                           ENUMERATED {supported}              OPTIONAL,

scs-120kHz                          ENUMERATED {supported}              OPTIONAL

}

}                                                                               OPTIONAL,

channelBW-UL-IAB-r16                    CHOICE {

fr1-100mhz                              SEQUENCE {

scs-15kHz                               ENUMERATED {supported}          OPTIONAL,

scs-30kHz                               ENUMERATED {supported}          OPTIONAL,

scs-60kHz                               ENUMERATED {supported}          OPTIONAL

},

fr2-200mhz                              SEQUENCE {

scs-60kHz                               ENUMERATED {supported}          OPTIONAL,

scs-120kHz                              ENUMERATED {supported}          OPTIONAL

}

}                                                                               OPTIONAL,

rasterShift7dot5-IAB-r16                ENUMERATED {supported}                  OPTIONAL,

ue-PowerClass-v1610                     ENUMERATED {pc1dot5}                    OPTIONAL,

condHandover-r16                        ENUMERATED {supported}                  OPTIONAL,

condHandoverFailure-r16                 ENUMERATED {supported}                  OPTIONAL,

condHandoverTwoTriggerEvents-r16        ENUMERATED {supported}                  OPTIONAL,

condPSCellChange-r16                    ENUMERATED {supported}                  OPTIONAL,

condPSCellChangeTwoTriggerEvents-r16    ENUMERATED {supported}                  OPTIONAL,

mpr-PowerBoost-FR2-r16                  ENUMERATED {supported}                  OPTIONAL,

-- R1 11-9: Multiple active configured grant configurations for a BWP of a serving cell

activeConfiguredGrant-r16               SEQUENCE {

maxNumberConfigsPerBWP-r16                  ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 11-9a: Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell

jointReleaseConfiguredGrantType2-r16    ENUMERATED {supported}                  OPTIONAL,

-- R1 12-2: Multiple SPS configurations

sps-r16                                 SEQUENCE {

maxNumberConfigsPerBWP-r16                  INTEGER (1..8),

maxNumberConfigsAllCC-r16                   INTEGER (2..32)

}                                                                               OPTIONAL,

-- R1 12-2a: Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell

jointReleaseSPS-r16                     ENUMERATED {supported}                  OPTIONAL,

-- R1 13-19: Simultaneous positioning SRS and MIMO SRS transmission within a band across multiple CCs

simulSRS-TransWithinBand-r16            ENUMERATED {n2}                         OPTIONAL,

trs-AdditionalBandwidth-r16             ENUMERATED {trs-AddBW-Set1, trs-AddBW-Set2}  OPTIONAL,

handoverIntraF-IAB-r16                  ENUMERATED {supported}                  OPTIONAL

]],

[[

-- R1 22-5a: Simultaneous transmission of SRS for antenna switching and SRS for CB/NCB /BM for intra-band UL CA

-- R1 22-5c: Simultaneous transmission of SRS for antenna switching and SRS for antenna switching for intra-band UL CA

simulTX-SRS-AntSwitchingIntraBandUL-CA-r16  SimulSRS-ForAntennaSwitching-r16            OPTIONAL,

-- R1 10: NR-unlicensed

sharedSpectrumChAccessParamsPerBand-v1630   SharedSpectrumChAccessParamsPerBand-v1630   OPTIONAL

]],

[[

handoverUTRA-FDD-r16                      ENUMERATED {supported}                       OPTIONAL,

-- R4 7-4: Report the shorter transient capability supported by the UE: 2, 4 or 7us

enhancedUL-TransientPeriod-r16            ENUMERATED {us2, us4, us7}                   OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1640 SharedSpectrumChAccessParamsPerBand-v1640    OPTIONAL

]],

[[

type1-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

type2-PUSCH-RepetitionMultiSlots-v1650    ENUMERATED {supported}                       OPTIONAL,

pusch-RepetitionMultiSlots-v1650          ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType1-v1650             ENUMERATED {supported}                       OPTIONAL,

configuredUL-GrantType2-v1650             ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1650 SharedSpectrumChAccessParamsPerBand-v1650    OPTIONAL

]],

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

-- R1 36-1: Support of 1024QAM for PDSCH for FR1

pdsch-1024QAM-FR1-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R4 22-1 support of FR2 HST operation

ue-PowerClass-v1700                       ENUMERATED {pc5, pc6, pc7}                   OPTIONAL,

-- R1 24: NR extension to 71GHz (FR2-2)

fr2-2-AccessParamsPerBand-r17             FR2-2-AccessParamsPerBand-r17                OPTIONAL,

rlm-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

bfd-Relaxation-r17                        ENUMERATED {supported}                       OPTIONAL,

cg-SDT-r17                                ENUMERATED {supported}                       OPTIONAL,

locationBasedCondHandover-r17             ENUMERATED {supported}                       OPTIONAL,

timeBasedCondHandover-r17                 ENUMERATED {supported}                       OPTIONAL,

eventA4BasedCondHandover-r17              ENUMERATED {supported}                       OPTIONAL,

mn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

sn-InitiatedCondPSCellChangeNRDC-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3a: PDCCH skipping

pdcch-SkippingWithoutSSSG-r17             ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3b: 2 search space sets group switching

sssg-Switching-1BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3c: 3 search space sets group switching

sssg-Switching-2BitInd-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3d: 2 search space sets group switching with PDCCH skipping

pdcch-SkippingWithSSSG-r17                ENUMERATED {supported}                       OPTIONAL,

-- R1 29-3e: Support Search space set group switching capability 2 for FR1

searchSpaceSetGrp-switchCap2-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 26-1: Uplink Time and Frequency pre-compensation and timing relationship enhancements

uplinkPreCompensation-r17                 ENUMERATED {supported}                       OPTIONAL,

-- R1 26-4: UE reporting of information related to TA pre-compensation

uplink-TA-Reporting-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-5: Increasing the number of HARQ processes

max-HARQ-ProcessNumber-r17                ENUMERATED {u16d32, u32d16, u32d32}          OPTIONAL,

-- R1 26-6: Type-2 HARQ codebook enhancement

type2-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6a: Type-1 HARQ codebook enhancement

type1-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-6b: Type-3 HARQ codebook enhancement

type3-HARQ-Codebook-r17                   ENUMERATED {supported}                       OPTIONAL,

-- R1 26-9: UE-specific K_offset

ue-specific-K-Offset-r17                  ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1f: Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1

multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R1 24-1g: Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1

multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 14-4: Parallel PRS measurements in RRC_INACTIVE state, FR1/FR2 diff

parallelPRS-MeasRRC-Inactive-r17          ENUMERATED {supported}                       OPTIONAL,

-- R1 27-1-2: Support of UE-TxTEGs for UL TDOA

nr-UE-TxTEG-ID-MaxSupport-r17             ENUMERATED {n1, n2, n3, n4, n6, n8}          OPTIONAL,

-- R1 27-17: PRS processing in RRC_INACTIVE

prs-ProcessingRRC-Inactive-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-3-2: DL PRS measurement outside MG and in a PRS processing window

prs-ProcessingWindowType1A-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType1B-r17            ENUMERATED {option1, option2, option3}       OPTIONAL,

prs-ProcessingWindowType2-r17             ENUMERATED {option1, option2, option3}       OPTIONAL,

-- R1 27-15: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP

srs-AllPosResourcesRRC-Inactive-r17       SRS-AllPosResourcesRRC-Inactive-r17          OPTIONAL,

-- R1 27-16: OLPC for positioning SRS in RRC_INACTIVE state - gNB

olpc-SRS-PosRRC-Inactive-r17              OLPC-SRS-Pos-r16                             OPTIONAL,

-- R1 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB

spatialRelationsSRS-PosRRC-Inactive-r17   SpatialRelationsSRS-Pos-r16                  OPTIONAL,

-- R1 30-1: Increased maximum number of PUSCH Type A repetitions

maxNumberPUSCH-TypeA-Repetition-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-2: PUSCH Type A repetitions based on available slots

puschTypeA-RepetitionsAvailSlot-r17       ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3: TB processing over multi-slot PUSCH

tb-ProcessingMultiSlotPUSCH-r17           ENUMERATED {supported}                       OPTIONAL,

-- R1 30-3a: Repetition of TB processing over multi-slot PUSCH

tb-ProcessingRepMultiSlotPUSCH-r17        ENUMERATED {supported}                       OPTIONAL,

-- R1 30-4: The maximum duration for DM-RS bundling

maxDurationDMRS-Bundling-r17              SEQUENCE {

fdd-r17                                   ENUMERATED {n4, n8, n16, n32}            OPTIONAL,

tdd-r17                                   ENUMERATED {n2, n4, n8, n16}             OPTIONAL

}                                                                                      OPTIONAL,

-- R1 30-6: Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

pusch-RepetitionMsg3-r17                  ENUMERATED {supported}                       OPTIONAL,

sharedSpectrumChAccessParamsPerBand-v1710 SharedSpectrumChAccessParamsPerBand-v1710    OPTIONAL,

-- R4 25-2: Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions

-- on normal operations with the serving cell

parallelMeasurementWithoutRestriction-r17 ENUMERATED {supported}                       OPTIONAL,

-- R4 25-5: Parallel measurements on multiple NGSO satellites within a SMTC

maxNumber-NGSO-SatellitesWithinOneSMTC-r17 ENUMERATED {n1, n2, n3, n4}                 OPTIONAL,

-- R1 26-10: K1 range extension

k1-RangeExtension-r17                     ENUMERATED {supported}                       OPTIONAL,

-- R1 35-1: Aperiodic CSI-RS for tracking for fast SCell activation

aperiodicCSI-RS-FastScellActivation-r17   SEQUENCE {

maxNumberAperiodicCSI-RS-PerCC-r17        ENUMERATED {n8, n16, n32, n48, n64, n128, n255},

maxNumberAperiodicCSI-RS-AcrossCCs-r17    ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}

}                                                                                      OPTIONAL,

-- R1 35-2: Aperiodic CSI-RS bandwidth for tracking for fast SCell activation for 10MHz UE channel bandwidth

aperiodicCSI-RS-AdditionalBandwidth-r17   ENUMERATED {addBW-Set1, addBW-Set2}          OPTIONAL,

-- R1 28-1a: RRC-configured DL BWP without CD-SSB or NCD-SSB

bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17    ENUMERATED {supported}                       OPTIONAL,

-- R1 28-3: Half-duplex FDD operation type A for (e)RedCap UE

halfDuplexFDD-TypeA-RedCap-r17            ENUMERATED {supported}                       OPTIONAL,

-- R1 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 OPTIONAL,

-- R4 15-3 UE support of CBW for 480kHz SCS

channelBWs-DL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-480kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 15-4 UE support of CBW for 960kHz SCS

channelBWs-DL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

channelBWs-UL-SCS-960kHz-FR2-2-r17        BIT STRING (SIZE (8))                        OPTIONAL,

-- R4 17-1 UL gap for Tx power management

ul-GapFR2-r17                             ENUMERATED {supported}                       OPTIONAL,

-- R1 25-4: One-shot HARQ ACK feedback triggered by DCI format 1_2

oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 ENUMERATED {supported}                      OPTIONAL,

-- R1 25-5: PHY priority handling for one-shot HARQ ACK feedback

oneShotHARQ-feedbackPhy-Priority-r17      ENUMERATED {supported}                       OPTIONAL,

-- R1 25-6: Enhanced type 3 HARQ-ACK codebook feedback

enhancedType3-HARQ-CodebookFeedback-r17   SEQUENCE {

enhancedType3-HARQ-Codebooks-r17          ENUMERATED {n1, n2, n4, n8},

maxNumberPUCCH-Transmissions-r17          ENUMERATED {n1, n2, n3, n4, n5, n6, n7}

}                                                                                      OPTIONAL,

-- R1 25-7: Triggered HARQ-ACK codebook re-transmission

triggeredHARQ-CodebookRetx-r17              SEQUENCE {

minHARQ-Retx-Offset-r17                     ENUMERATED {n-7, n-5, n-3, n-1, n1},

maxHARQ-Retx-Offset-r17                     ENUMERATED {n4, n6, n8, n10, n12, n14, n16, n18, n20, n22, n24}

}                                                                                      OPTIONAL

]],

[[

-- R4 22-2 support of one shot large UL timing adjustment

ue-OneShotUL-TimingAdj-r17                        ENUMERATED {supported}               OPTIONAL,

-- R1 25-2: Repetitions for PUCCH format 0, and 2 over multiple slots with K = 2, 4, 8

pucch-Repetition-F0-2-r17                         ENUMERATED {supported}               OPTIONAL,

-- R1 25-11a: 4-bits subband CQI for NTN and unlicensed

cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17   ENUMERATED {supported}               OPTIONAL,

-- R1 25-16: HARQ-ACK with different priorities multiplexing on a PUCCH/PUSCH

mux-HARQ-ACK-DiffPriorities-r17                   ENUMERATED {supported}               OPTIONAL,

-- R1 25-20a: Propagation delay compensation based on Rel-15 TA procedure for NTN and unlicensed

ta-BasedPDC-NTN-SharedSpectrumChAccess-r17        ENUMERATED {supported}               OPTIONAL,

-- R1 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17  ENUMERATED {supported}               OPTIONAL,

-- R1 33-2e: Multiple G-RNTIs for group-common PDSCHs

maxNumberG-RNTI-r17                               INTEGER (2..8)                       OPTIONAL,

-- R1 33-2f: Dynamic multicast with DCI format 4_2

dynamicMulticastDCI-Format4-2-r17                 ENUMERATED {supported}               OPTIONAL,

-- R1 33-2i: Supported maximal modulation order for multicast PDSCH

maxModulationOrderForMulticast-r17                CHOICE {

fr1-r17                                           ENUMERATED {qam256, qam1024},

fr2-r17                                           ENUMERATED {qam64, qam256}

}                                                                                                                          OPTIONAL,

-- R1 33-3-1: Dynamic Slot-level repetition for group-common PDSCH for TN and licensed

dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17  ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-3-1a: Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed

dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-4-1: DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17                ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

maxNumberG-CS-RNTI-r17                                          INTEGER (2..8)                                             OPTIONAL,

-- R1 33-10: Support group-common PDSCH RE-level rate matching for multicast

re-LevelRateMatchingForMulticast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 36-1a: Support of 1024QAM for PDSCH with maximum 2 MIMO layers for FR1

pdsch-1024QAM-2MIMO-FR1-r17                                     ENUMERATED {supported}                                     OPTIONAL,

-- R4 14-3 PRS measurement without MG

prs-MeasurementWithoutMG-r17                                    ENUMERATED {cpLength, quarterSymbol, halfSymbol, halfSlot} OPTIONAL,

-- R4 25-7: The number of target LEO satellites the UE can monitor per carrier

maxNumber-LEO-SatellitesPerCarrier-r17                          INTEGER (3..4)                                             OPTIONAL,

-- R1 27-3-3 DL PRS Processing Capability outside MG - buffering capability

prs-ProcessingCapabilityOutsideMGinPPW-r17    SEQUENCE (SIZE(1..3)) OF PRS-ProcessingCapabilityOutsideMGinPPWperType-r17   OPTIONAL,

-- R1 27-15a: Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS

srs-SemiPersistent-PosResourcesRRC-Inactive-r17                 SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17                       ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17                ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

}                                                                                                                          OPTIONAL,

-- R2: UE support of CBW for 120kHz SCS

channelBWs-DL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL,

channelBWs-UL-SCS-120kHz-FR2-2-r17                              BIT STRING (SIZE (8))                                      OPTIONAL

]],

[[

-- R1 30-4a: DM-RS bundling for PUSCH repetition type A

dmrs-BundlingPUSCH-RepTypeA-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4b: DM-RS bundling for PUSCH repetition type B

dmrs-BundlingPUSCH-RepTypeB-r17                                 ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4c: DM-RS bundling for TB processing over multi-slot PUSCH

dmrs-BundlingPUSCH-multiSlot-r17                                ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4d: DMRS bundling for PUCCH repetitions

dmrs-BundlingPUCCH-Rep-r17                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4e: Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH

interSlotFreqHopInterSlotBundlingPUSCH-r17                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4f: Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling

interSlotFreqHopPUCCH-r17                                       ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4g: Restart DM-RS bundling

dmrs-BundlingRestart-r17                                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 30-4h: DM-RS bundling for non-back-to-back transmission

dmrs-BundlingNonBackToBackTX-r17                                ENUMERATED {supported}                                     OPTIONAL

]],

[[

-- R1 33-5-1e: Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

maxDynamicSlotRepetitionForSPS-Multicast-r17                    ENUMERATED {n8, n16}                                       OPTIONAL,

-- R1 33-5-1g: DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-1i: Multicast SPS scheduling with DCI format 4_2

sps-MulticastDCI-Format4-2-r17                                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-5-2: Multiple SPS group-common PDSCH configuration on PCell

sps-MulticastMultiConfig-r17                                    INTEGER (1..8)                                             OPTIONAL,

-- R1 33-6-1: DL priority indication for multicast in DCI

priorityIndicatorInDCI-Multicast-r17                            ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-1a: DL priority configuration for SPS multicast

priorityIndicatorInDCI-SPS-Multicast-r17                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-2: Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities

-- for unicast and multicast at a UE

twoHARQ-ACK-CodebookForUnicastAndMulticast-r17                  ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-6-3: More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 33-9: Supporting unicast PDCCH to release SPS group-common PDSCH

releaseSPS-MulticastWithCS-RNTI-r17                             ENUMERATED {supported}                                     OPTIONAL

]],

[[

-- R1 41-3-1a  UE automomous TA adjustment when cell-reselection happens

posUE-TA-AutoAdjustment-r18                                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-4-6a   support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers

-- for SRS bandwidth aggregation in RRC_CONNECTED state

posJointTriggerBySingleDCI-RRC-Connected-r18                    ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-5-1a PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs

dl-PRS-MeasurementWithRxFH-RRC-InactiveforRedCap-r18            ENUMERATED {supported}                                     OPTIONAL,

-- R1 41-5-1b PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs

dl-PRS-MeasurementWithRxFH-RRC-IdleforRedCap-r18                ENUMERATED {supported}                                     OPTIONAL,

-- R1 42-4: Cell DTX and/or DRX operation based on RRC configuration

nes-CellDTX-DRX-r18                                             ENUMERATED {cellDTXonly, cellDRXonly, both}                OPTIONAL,

-- R1 42-5: Cell DTX/DRX operation triggered by DCI format 2_9

nes-CellDTX-DRX-DCI2-9-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 45-6: UE-based TA measurement

ue-TA-Measurement-r18                                           INTEGER (1..8)                                             OPTIONAL,

-- R1 45-7: TA indication in cell switch command

ta-IndicationCellSwitch-r18                                     ENUMERATED {supported}                                     OPTIONAL,

-- R1 50-1: Multi-PUSCHs for Configured Grant

multiPUSCH-CG-r18                                               ENUMERATED {n16, n32}                                      OPTIONAL,

-- R1 50-1a: Multiple active multi-PUSCHs configured grant configurations for a BWP of a serving cell

multiPUSCH-ActiveConfiguredGrant-r18                            SEQUENCE {

maxNumberConfigsPerBWP                                          ENUMERATED {n1, n2, n4, n8, n12},

maxNumberConfigsAllCC-FR1                                       INTEGER (2..32),

maxNumberConfigsAllCC-FR2                                       INTEGER (2..32)

}                                                                                                                          OPTIONAL,

-- R1 50-2: UCI indication of unused CG-PUSCH transmission occasions

cg-PUSCH-UTO-UCI-Ind-r18                                        ENUMERATED {supported}                                     OPTIONAL,

-- R1 50-3: PDCCH monitoring resumption after UL NACK

pdcch-MonitoringResumptionAfterUL-NACK-r18                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 51-1: support for 3MHz channel bandwidth

support-3MHz-ChannelBW-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 51-2: support 12 PRB CORESET0

support-12PRB-CORESET0-r18                                      ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-1: Reception of NR PDCCH candidates overlapping with LTE CRS REs

nr-PDCCH-OverlapLTE-CRS-RE-r18                                  SEQUENCE {

overlapInRE-r18                                                 ENUMERATED {oneSymbolNoOverlap, someOrAllSymOverlap},

overlapInSymbol-r18                                             ENUMERATED {symbol2,symbol1And2}

}                                                                                                                          OPTIONAL,

-- Editor's Note: someOrAllSymOverlap considers to be supported in overlapInRE-r18 only if RAN4 performance requirements for

-- someOrAllSymOverlap are not defined

-- R1 52-1a: Reception of NR PDCCH candidates overlapping with LTE CRS REs with multiple non-overlapping CRS rate matching patterns

nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18                    ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-1b: NR PDCCH reception that overlaps with LTE CRS within a single span of 3 consecutive OFDM symbols that is within the

-- first 4 OFDM symbols in a slot

nr-PDCCH-OverlapLTE-CRS-RE-Span-3-4-r18                         ENUMERATED {supported}                                     OPTIONAL,

-- R1 52-2: Two LTE-CRS overlapping rate matching patterns within NR 15 kHz carrier overlapping with LTE carrier (regardless of

-- support or configuration of multi-TRP)

twoRateMatchingEUTRA-CRS-patterns-3-4-r18                       SEQUENCE {

maxNumberPatterns-r18                                           INTEGER (2..6),

maxNumberNon-OverlapPatterns-r18                                INTEGER (1..3)

}                                                                                                                          OPTIONAL,

-- R1 52-2a: Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within NR 15 kHz carrier

-- overlapping with LTE carrier

overlapRateMatchingEUTRA-CRS-Patterns-3-4-Diff-CS-Pool-r18      ENUMERATED {supported}                                     OPTIONAL,

-- R1 53-3: Support RLM/BM/BFD measurements based on NCD-SSB within active BWP

ncd-SSB-BWP-Wor-r18                                             ENUMERATED {supported}                                     OPTIONAL,

-- R1 53-4: Support Support RLM/BM/BFD measurements based on CSI-RS when CD-SSB is outside active BWP

rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18                          ENUMERATED {supported}                                     OPTIONAL,

-- R1 55-3: Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1

multiPUSCH-SingleDCI-NonConsSlots-r18                           ENUMERATED {supported}                                     OPTIONAL,

-- R1 55-2d: single-symbol DL-PRS used in RTT-based Propagation delay compensation

pdc-maxNumberPRS-ResourceProcessedPerSlot-r18                   SEQUENCE {

fr1-r18 SEQUENCE {

scs-15kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-30kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-60kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL

},

fr2-r18 SEQUENCE {

scs-60kHz-r18                                   ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL,

scs-120kHz-r18                                  ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64}     OPTIONAL

}

}                                                                                                                         OPTIONAL,

-- R4 27-2: LowerMSD for inter-band NR CA and EN-DC

lowerMSD-r18                                                    SEQUENCE (SIZE (1..maxLowerMSD-r18)) OF LowerMSD-r18      OPTIONAL,

-- R4 28-1: Enhanced channel raster

enhancedChannelRaster-r18                                       ENUMERATED {supported}                                     OPTIONAL,

-- R4 31-2 Beam sweeping factor reduction for FR2 unknown SCell activation

beamSweepingFactorReduction-r18                                 SEQUENCE {

reduceForCellDetection                                          ENUMERATED {n1, n2, n4, n6},

reduceForSSB-L1-RSRP-Meas                                       INTEGER (0..7)

}                                                                                                                         OPTIONAL,

-- R4 35-2: the requirements defined for ATG UE with antenna array or omni-direction antenna requirements.

antennaArrayType-r18                                           ENUMERATED {supported}                                     OPTIONAL,

locationBasedCondHandoverATG-r18                               ENUMERATED {supported}                                     OPTIONAL,

-- R4 35-3: rated maximum output power value range from 23dBm to 40dBm with 1dB as granularity at maximum modulation order and full

-- PRB configurations.

maxOutputPowerATG-r18                                          INTEGER (1..18)                                            OPTIONAL,

eventA4BasedCondHandoverNES-r18                                ENUMERATED {supported}                                     OPTIONAL,

nesBasedCondHandoverWithDCI-r18                                ENUMERATED {supported}                                     OPTIONAL,

rachLessHandoverNTN-r18                                        ENUMERATED {supported}                                     OPTIONAL,

locationBasedCondHandoverEMC-r18                               ENUMERATED {supported}                                     OPTIONAL,

mt-CG-SDT-r18                                                  ENUMERATED {supported}                                     OPTIONAL,

posSRS-RRC-InactiveInitialUL-BWP-r18                           ENUMERATED {supported}                                     OPTIONAL,

posSRS-RRC-InactiveOutsideInitialUL-BWP-r18                    ENUMERATED {supported}                                     OPTIONAL,

cg-SDT-PeriodicityExt-r18                                      ENUMERATED {supported}                                     OPTIONAL

]]

}

BandNR-v16c0 ::=                                                SEQUENCE {

pusch-RepetitionTypeA-v16c0                                     ENUMERATED {supported}                                     OPTIONAL,

...

}

LowerMSD-r18 ::=           SEQUENCE {

aggressorband1-r18         FreqBandIndicatorNR,

aggressorband2-r18         FreqBandIndicatorNR                                                                             OPTIONAL,

msd-Information-r18        SEQUENCE (SIZE (1..maxLowerMSDInfo-r18)) OF MSD-Information-r18

}

MSD-Information-r18 ::=    SEQUENCE {

msd-Type-r18               ENUMERATED {harmonic, harmonicMixing, crossBandIsolation, imd2, imd3, imd4, imd5, all, spare8, spare7,

spare6, spare5,spare4, spare3, spare2, spare1},

msd-PowerClass-r18         ENUMERATED {pc1dot5, pc2, pc3},

msd-Class-r18              ENUMERATED {classI, classII, classIII, classIV, classV, classVI, classVII, classVIII }

}

-- Editor note: The power class related part can be updated further pending RAN4 discussion.

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

RF-Parameters field descriptions

appliedFreqBandListFilter In this field the UE mirrors the FreqBandList that the NW provided in the capability enquiry, if any, as described in clause 5.6.1.4. The UE filtered the band combinations in the supportedBandCombinationList in accordance with this appliedFreqBandListFilter. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationList A list of band combinations that the UE supports for NR (and NR-DC, if requested). The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandCombinationListSidelinkEUTRA-NR A list of band combinations that the UE supports for NR sidelink communication only, for joint NR sidelink communication and V2X sidelink communication, or for V2X sidelink communication only. The UE does not include this field if the UE capability is requested by E-UTRAN (see TS 36.331[10]) and the network request includes the field eutra-nr-only.

supportedBandCombinationListSL-NonRelayDiscovery A list of band combinations that the UE supports for NR sidelink non-relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-RelayDiscovery A list of band combinations that the UE supports for NR sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationListSL-U2U-DiscoveryExt This field indicates the band parameter in BandCombinationListSL-Discovery-r17 that the UE supports for NR U2U sidelink relay discovery in a band included in supportedBandCombinationListSL-U2U-RelayDiscovery.

supportedBandCombinationListSL-U2U-RelayDiscovery A list of band combinations that the UE supports for NR U2U sidelink relay discovery. The encoding is defined in PC5 BandCombinationListSidelinkNR-r16.

supportedBandCombinationList-UplinkTxSwitch A list of band combinations that the UE supports dynamic uplink Tx switching for NR UL CA and SUL. The FeatureSetCombinationId:s in this list refer to the FeatureSetCombination entries in the featureSetCombinations list in the UE-NR-Capability IE. The UE does not include this field if the UE capability is requested by E-UTRAN and the network request includes the field eutra-nr-only [10].

supportedBandListNR A list of NR bands supported by the UE. If supportedBandListNR-v16c0 is included, the UE shall include the same number of entries, and listed in the same order, as in supportedBandListNR (without suffix).

Annex C (normative):	List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table C-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-200335: Correction on usage of access category 2 for UAC for RNA update | 1141 | 2 | Release 15 |  |
| RP-201185: Introduction of signalling for high-speed train scenarios | 1464 | 5 | Release 15 |  |
| RP-201216: Release-16 UE capabilities based on RAN1, RAN4 feature lists and RAN2 | 1665 | 2 | Release 15 | Early implementation part is referring to the aspect covered by - R2-2006203: Extension of CSI-RS capabilities per codebook type - R2-2006360: Intraband EN_DC power class expansion for 29 dBm |
| RP-202768: UE behaviour when UL 7.5KHz shift is not supported | 2107 | 2 | Release 15 |  |
| RP-202790: Correction on uac-AccessCategory1-SelectionAssistanceInfo | 2130 | 1 | Release 15 |  |
| RP-211483: Clarification on the initiation of RNA update | 2581 | 1 | Release 15 |  |
| RP-201190: Introduction of eCall over IMS for NR | 1670 | - | Release 15 |  |
| RP-212598: Distinguishing support of extended band n77 | 2810 | 2 | Release 15 |  |
| RP-213342: Duty cycle signalling for power class 1.5 | 2817 | 1 | Release 15 |  |
| RP-213345: CR on 38.331 for introducing UE capability of txDiversity | 2859 | 1 | Release 15 |  |
| RP-220497: Introduction of function for RRM enhancements for Rel-17 NR FR1 HST | 2898 | 2 | Release 16 |  |
| RP-220838: Release-17 UE capabilities based on R1 and R4 feature lists (TS38.331) | 2901 | 1 | Release 15 | Early implementation part is referring to the aspect covered by: - R2-2203898: Introduction of BCS4 and BCS5 - R2-2203836: Introducing UE capability for power class 5 for FR2 FWA |
| RP-221721: CR on the CBM/IBM reporting-38331 | 2916 | 2 | Release 16 |  |
| RP-221736: Distinguishing support of band n77 restrictions in Canada [n77 Canada] | 3078 | 2 | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for the US | 3476 | - | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for Canada | 3478 | - | Release 15 |  |
| RP-232570: Addition of extended number range for NS value | 3900 | 6 | Release 16 |  |
| RP-233888: Introduction of FR2 FBG2 CA BW classes | 2867 | 6 | Release 15 |  |
| RP-233882: Enhancing SCell A2 event reporting | 4375 | 2 | Release 15 |  |
| RP-233890: PTM retransmission reception for multicast DRX with HARQ feedback disabled [PTM_ReTx_Mcast_HARQ_Disb] | 4504 | - | Release 17 |  |
| RP-23xxxxx: UE capability for Enhanced channel raster | 4445 | 2 | Release 16 |  |

<End of changes>