3GPP TSG RAN WG2#125	R2-2400013

Athens, Greece, 26th February - 1st March 2024

3GPP TSG RAN WG1 #115			R1-2312382

Chicago, USA, November 13th – 17th, 2023

Title:	LS to RAN2 on TDCP for Rel-18 MIMO

Response to:

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Eko Onggosanusi

E-mail Address:	eko.o@samsung.com

1. Overall Description:

In relation to the following text from the running CR for TS 38.331:

nzp-CSI-RS-ResourceSetList List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set. If resourceType is set to ‘aperiodic’, the network configures up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets. If resourceType is is set to ‘periodic’ or ‘semiPersistent’ and groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig, the network configures 1 resource set. If resourceType is set to ‘periodic’ or ‘semiPersistent’ and groupBasedBeamReporting-v1710 is configured, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the following applies: - if the list has one NZP CSI-RS resource set, this resource set is indicated by a resource set indicator set to 0; - if the list has two NZP CSI-RS resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

RAN1 would like to inform RAN2 about the following two agreements pertaining to TDCP:

[RAN1#114] Agreement

For the Rel-18 TRS-based TDCP reporting, the supported values of KTRS (number of configured TRS resource sets) are {1,2,3}

- The candidate values {2,3} are UE optional

[RAN1#115] Agreement

For the Rel-18 TRS-based TDCP reporting, in relation to the following text in TS 38.331, send an LS to RAN2 that the nzp-CSI-RS-ResourceSetList in CSI-ResourceConfig can be configured with up to 3 periodic CSI-RS resource set for TDCP report:

nzp-CSI-RS-ResourceSetList

List of references to NZP CSI-RS resources used for beam measurement and reporting in a CSI-RS resource set.

If resourceType is set to ‘aperiodic’, the network configures up to maxNrofNZP-CSI-RS-ResourceSetsPerConfig resource sets. If resourceType is is set to ‘periodic’ or ‘semiPersistent’ and groupBasedBeamReporting-v1710 is not configured in IE CSI-ReportConfig, the network configures 1 resource set. If resourceType is set to ‘periodic’ or ‘semiPersistent’ and groupBasedBeamReporting-v1710 is configured, the network configures 2 resource sets, which may be two NZP CSI-RS resource sets, two CSI SSB resource sets or one NZP CSI-RS resource set and one CSI-SSB resource set (see TS 38.214 [19], clause 5.2.1.2 and 5.2.1.4.2). In this case, in TS 38.212 [17] Table 6.3.1.1.2-8B, the following applies:

- if the list has one NZP CSI-RS resource set, this resource set is indicated by a resource set indicator set to 0;

- if the list has two NZP CSI-RS resource sets, the first resource set is indicated by a resource set indicator set to 0 and the second resource set by a resource set indicator set to 1.

2. Actions:

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into account in drafting the description for nzp-CSI-RS-ResourceSetList pertaining to the number of configured resource sets.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #116	26th February – 1st March, 2024	Athens, Greece