3GPP TSG RAN WG2#125	R2-2400026

Athens, Greece, 26th February - 1st March 2024

3GPP TSG RAN WG1 Meeting #115		R1-2312621

Chicago, USA, November 13th – November 17th, 2023

Title:	LS on Rel-17 URLLC/IIoT required RRC parameter description change in 38.331

Response to:	-

Release:	Release 17

Work Item:	NR_IIOT_URLLC_enh-Core

Source:	RAN1

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Klaus Hugl

E-mail Address:	klaus.hugl@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

As part of maintenance discussions on Rel-17 URLLC/IIoT, RAN1 recognized that the RRC parameter description of cg-UCI-Multiplexing in Rel-17 requires some updates as additional functionalities are supported for Rel-17 URLLC/IIoT multiplexing UCI of different PHY priorities on PUSCH in connection with the Rel-16 RRC parameter cg-UCI-Multiplexing where the RRC parameter indicates the possible multiplexing of HARQ-ACK of different PHY priority but the HARQ-ACK and CG-UCI are not jointly coded.

Therefore, RAN1 would like to request RAN2 to consider the following proposed changes to the RRC parameter description of cg-UCI-Multiplexing for Rel-17:

cg-UCI-Multiplexing If present, this field indicates that in the case of PUCCH overlapping with CG-PUSCH(s) including CG-UCI within a PUCCH group, the CG-UCI and HARQ-ACK is multiplexed on the CG-PUSCH including CG-UCI are jointly encoded (see TS 38.213 [13], clause 9).

# 2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above into consideration in their future work.

# 3	Dates of next TSG RAN 1 meetings

TSG RAN WG1 Meeting #116	Feb. 26th –  March 1st, 2024	Athens, GR

TSG RAN WG1 Meeting #116bis	April 14th – 19th, 2024	China (TBC), CN