3GPP TSG-SA5 Meeting #153 	S5-240846

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0421 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS32.422 User Consent for Trace Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As described in the incoming LS R3-237964 on Support for the user consent for trace reporting, RAN3 has agreed on a solution: - For Management Based MDT, RAN select UEs on the basis of not only the received Management Based MDT PLMN List IE but also the configured MDT data. If the configured MDT data are not subject to user consent, the RAN does not need to consider user consent when selecting UEs and it can select UEs including those for which the Management Based MDT PLMN List was not received. - For Signalling Based MDT, AMF/UDM triggers a Trace Activation for the UE based on not only the user consent availability but also the configured MDT data. If the configured MDT data are NOT subject to user consent, the AMF/UDM shall trigger a Trace Activation, even if user consent is not available for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defining a new user consent procedures for Management Based MDT and For Signalling Based MDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user consent procedures for Management Based MDT and For Signalling Based MDT cannot be supported as RAN3 specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.1, 4.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 1139 |  |  |
| Other comments: |  | TS32.422 CR0421, TS28.541 CR1139 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-240025 |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.9.1	Signalling based MDT

In case of signalling based MDT getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI.

Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned to the UDM database.

The following figure summarizes the functionality.

# Figure 4.9.1.1: Example for delivering user consent information in signalling based MDT

When the IMSI/IMEI(SV)/SUPI based MDT is activated, it is targeted the UDM. The UDM shall check the MDT user consent requirements configured by OAM. The UDM shall consider user consent when activating an MDT trace session for the UE only if the MDT measurement name specified in MDT activation is subject to user consent. If the MDT measurement name specified in MDT activation is not subject to user consent, the UDM shall not consider user consent when activating an MDT trace session for the UE.

If one or more MDT measurement name(s) specified in MDT activation is/are subject to user consent, the UDM shall check the user consent availability before starting a Trace Session for the given subscriber. If there is no user consent given by the specific user for the user-consent-required MDT measurement name(s), the UDM should not start a Trace Session for the given subscriber.

As the user consent availability information is stored as part of the subscription data it should also be transferred to the AMF during update location procedure. This is required if the signalling based MDT is started from AMF. In that case, same user consent checking is required as in the UDM case.

It should also be possible to handle user consent revocation. The process of user consent revocation shall be done also via customer care process and the user consent availability information should be updated in the UDM DB when a user consent revocation happens.

If the user consent revocation happens during an ongoing Trace Session with MDT, it is not required to stop and deactivate the Trace Recording Session, Trace Session respectively immediately i.e. to stop an ongoing Trace Recording Session in case of Immediate MDT. A notification to the management system should be sent and the management system should deactivate the Trace Session.

*** START OF NEXT CHANGE ***

### 4.9.2	Management  based MDT

The following figure summarizes the functionality.

Figure 4.9.2.1: Example for delivering user consent information in management based MDT

When UE attaches to the network, the UDM shall forward the user consent information, stored in the UDM database, to the corresponding AMF. When the AMF receives the user consent information it shall store it in its subscriber database.

The AMF shall also check the roaming status of the user. If the user is within its home operator’s PLMNs and the user has given its consent, the AMF shall send the Management based MDT PLMN List IE to the gNB during the UE context setup procedure (INITIAL CONTEXT SETUP REQUEST message, see TS 38.413 [49]). Otherwise the AMF shall not send the Management based MDT PLMN List IE  to the gNB.

If the result of the roaming status check indicates a home subscriber, AMF shall forward the already stored user consent information to the corresponding gNB as part of Management based MDT PLMN List IE .

When the management based MDT activation is sent to gNB, the gNB shall check the MDT user consent requirements configured by OAM. The gNB shall consider user consent when selecting UEs only if the MDT measurement name specified in MDT collection is subject to user consent. If the MDT measurement name specified in MDT collection is not subject to user consent, the gNB shall not consider user consent when selecting UEs.

If one or more MDT measurement name(s) specified in MDT collection is/are subject to user consent, gNB shall check the availability of the Management based MDT PLMN List IE before making the UE selection. In case the Management based MDT PLMN List IE is not available, the gNB shall not select the UE. The gNB shall forward the received Management based MDT PLMN List IE during Xn based handovers to the target node. The Management based MDT PLMN List IE is stored in the gNB as part of the UE context. If the user consent information is updated while a UE context is already set up in the gNB, the changed user consent should be taken into account in the next call/session setup.

The AMF shall forward the received Management based MDT PLMN List IE during N2 based handovers to the target node in the HANDOVER REQUEST message. Furthermore, in case of path switch request procedure, the AMF shall forward the received Management based MDT PLMN List IE to the corresponding gNB in PATH SWITCH REQUEST ACKNOWLEDGE message, see TS 38.413 [49].

AMF subscribes to UDM  SDM API to get notification on any changes of the UE AM subscription data, including the changes on MDT user consent information which is part of the UE AM subscription data. When being notified by the UDM about an MDT user consent information change, the AMF shall subsequently signal the updated Management Based MDT PLMN Modification List IE to the gNB in UE CONTEXT MODIFICATION REQUEST message, see TS 38.413 [49].

*** END OF CHANGE ***