3GPP TSG RAN WG2#125	R2-2401703

Athens, Greece, 26th February - 1st March 2024

3GPP TSG RAN WG1 #116	R1-2401716

Athens, Greece, February 26th – March 1st, 2024

Title:	LS on TS38.300 TP for Multi-cell scheduling in Rel-18

Response to:

Release:	Rel-18

Work Items:	NR_MC_enh-Core

Source:	RAN WG1

To:	RAN WG2

CC:

Contact Person:

#### Name:	Hiroki Harada

E-mail Address:	hiroki.harada.sv@nttdocomo.com

Attachment:

1. Overall Description:

RAN1 discussed on necessary updates for Rel-18 Multi-cell scheduling by a single DCI in TS38.300, and following text proposal was agreed as RAN1 recommendation.

## 10.X Multi-cell scheduling by a single DCI Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions: - When a serving cell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on a cell set, the PUSCH/PDSCH on serving cells in the cell set is always scheduled by a PDCCH on the serving cell; - When PCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, that PCell’s PDSCH and PUSCH cannot be scheduled by a PDCCH on an SCell; - When an SCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, PCell is not included in the cell set; - The scheduling PDCCH and the scheduled PDSCH(s)/PUSCH(s) can use the same or different numerologies; - The co-scheduled PDSCH(s) with a PDCCH use the same numerology. - The co-scheduled PUSCH(s) with a PDCCH use the same numerology.

2. Actions:

To RAN WG2

ACTION: RAN1 kindly asks RAN2 to take the above RAN1 recommendation into account.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #116bis	April 15 to 19, 2024	Changsha, China

TSG-RAN WG1 Meeting #117	May 20 to 24, 2024	Fukuoka, Japan