3GPP TSG-RAN WG2 Meeting #125                    	R2-2401722

# Athens, Greece, 26 Feb – 1 Mar, 2024

Title:	Reply LS on skipping UL transmission and R17 TBoMS

Response to:	R1-2312651, LS on skipping UL transmission and R17 TBoMS

Release:	Rel-17

Work Item:	NR_cov_enh-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name:	Chong Lou

E-mail Address:	louchong@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 would like to thank RAN1 for the LS on skipping UL transmission and R17 TBoMS.

RAN1 agreed that the Rel-16 skipping UL transmission and the Rel-17 TBoMS (TB processing over multiple slots) features are not configured at the same time to a UE. And RAN1 asked RAN2 to check if there is any issues identified to the agreement.

RAN2 answer is that RAN2 does not identify issues if the Rel-16 skipping UL transmission and the Rel-17 TBoMS features are not configured at the same time to a UE. And RAN2 will add the following description in RRC spec to capture the agreement: “If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, numberOfSlotsTBoMS (as specified in TS 38.214, clause 6.1.2.1) is not configured”.

In addition, RAN1 asked RAN2 to clarify the relationship between the higher layer parameter REPETITION_NUMBER specified in clause 5.4.2.1 of TS 38.321 and the number of slots scheduled for a TBoMS PUSCH specified in clause 6.1.2.1 of TS 38.214. RAN2 clarification is that the higher layer parameter REPETITION_NUMBER is “the number of repetitions K” for a TBoMS PUSCH as described in clause 6.1.2.1 of TS 38.214 and has no relation to the number of slots used for TBS determination N given by numberOfSlotsTBoMS.

2. Actions:

To RAN1: RAN2 respectfully ask RAN1 to take the above information into account for their future discussion.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#125-bis	15 Apr – 19 Apr 	Changsha, China

RAN2#126	20 May – 24 May 	Fukuoka, Japan