3GPP TSG-RAN WG2 Meeting #125	R2-2401741

Athens, Greece, Feb 26th – Mar 1st, 2024

Title:	LS on Paralle Tx Capability

Response to:

Release:	Release 15/16/17

Work Item:	TEI17, NR_newRAT-Core,NR_2step_RACH, NR_IIOT_URLLC_enh

Source:	TSG RAN WG2

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Wenting Li

E-mail Address:	Li.wenting@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

For the capability of “Simultaneous PUSCH and PUCCH transmissions of same priority on different inter-band cells”,  there is a Note2 as below in the RAN1 LS  (R1-2312456)

Agreement Simultaneous PUSCH and PUCCH transmissions of same priority on different inter-band cells is supported. - Note 1: Above applies since Rel-17. - Note 2: Above applies only for inter-band CA - Note 3: Above is subject to one new RRC parameter and one new UE capability (per BC). When the new RRC parameter is provided, simultaneous PUSCH and PUCCH transmission of same priority is always applied on different cells belonging to different bands.

During RAN2 discussion, there are 2 different understanding on this Note:

- Understanding #1: This capability can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG

- Understanding #2: This capability can only be applied to the inter-band CA band combination

Thus RAN2 would like to confirm which understanding is aligned with RAN1’s intention.

Q1: For the application of  “Simultaneous PUSCH and PUCCH transmissions of same priority on different inter-band cells” capability, which understanding is aligned with RAN1’s intention?

- Understanding #1: This capability can also be applied to the NR-DC band combination with the inter-band CA operation on the MCG/SCG;

- Understanding #2: This capability can only be applied to the inter-band CA band combination

Based on the Q1, RAN2 also went though the other parallex Tx capabilities and would like to confirm the below 2 further questions:

Q2: Whether Features listed in the Table 1 follow understanding #1 or understanding #2?

Table 1: Parallel Tx Capability with Inter-band CA

| FG Index | Ran1 Description in the Feature list Table | Current Ran2 Field description |
| --- | --- | --- |
| 4-25(R15) | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | parallelTxSRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination. |
| 4-26(R15) | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination. |
| 9-3(R16) | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | parallelTxMsgA-SRS-PUCCH-PUSCH-r16 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an inter-band CA band combination. A UE supporting this feature shall also indicate support of parallelTxPRACH-SRS-PUCCH-PUSCH. |
| 25-18(R17) | Support simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA. | parallelTxPUCCH-PUSCH-r17 Indicates whether the UE supports simultaneous PUCCH and PUSCH transmissions of different priority on different cells for inter-band CA. |

Q3: Whether Features listed in the Table 2 can also be applied to the NR-DC band combination with the intra-band non-contiguous CA operation on the MCG/SCG or only to intra-band non-contiguous CA band combination?

Table 2: Parallel Tx Capability with Intra-band non-contiguous CA

| FG Index | Ran1 Description in the Feature list Table | Current Ran2 Field description |
| --- | --- | --- |
| 39-2(R17) | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA | parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an intra-band non-contiguous CA band combination. |
| 39-4(R17) | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in intra-band non-contiguous CA(Prerequisite 9-3) | parallelTxMsgA-SRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of MsgA and SRS/ PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination. The UE indicating support of this field shall also indicate support of parallelTxMsgA-SRS-PUCCH-PUSCH-r16 and parallelTxPRACH-SRS-PUCCH-PUSCH-intraBand-r17. |
| 39-1(R17) | Parallel SRS and PUCCH/PUSCH transmission across CCs in intra-band non-contiguous CA | parallelTxSRS-PUCCH-PUSCH-intraBand-r17 Indicates whether the UE supports parallel transmission of SRS and PUCCH/ PUSCH across CCs in an intra-band non-contiguous CA band combination. |

Therefore, RAN2 respectfully asks RAN1 to provide feedback on above questions

2. Actions

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to provide feedback on above questions.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#125-bis	from 2024-04-15	to 2024-04-19		Changsha, China

RAN2#126	from 2024-05-20	to 2024-05-24		Fukuoka City, JP