3GPP TSG RAN WG2#125bis	R2-2402131

Changsha, China, 15th - 19th April 2024

3GPP TSG-RAN WG4 Meeting #110	R4-2403549

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Title:	LS on R18 mobility - Improvement on SCell/SCG setup delay

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact person:	Qiming Li

Li_qiming@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment: None

1. Overall Description:

In RAN4#110 RAN4 agreed the following UE feature list for UE supporting R18 Improvement on SCell/SCG setup delay:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-x1 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup | idleInactiveNR-MeasReport-r16 | Yes | N/A | UE does not support EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
|  | 39-x2 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on non-EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | Yes | N/A | UE does not support non-EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | FFS | N/A |  | Optional with capability signaling |

2. Actions:

To RAN2

ACTION: RAN4 respectfully asks RAN2 to take the above information into account.

3. Date of Next RAN WG4 Meetings:

TSG RAN WG4 Meeting #110-bis	15 – 19 April 2024	Changsha, China

TSG RAN WG4 Meeting #111	20 - 24 May 2024 	Fukuoka, Japan