3GPP TSG RAN WG2#125bis	R2-2402135

Changsha, China, 15th - 19th April 2024

3GPP TSG-RAN WG4 Meeting #110	R4-2403659

# Athens, Greece, Feb 26 – Mar 1, 2024

# Title:	LS on power class capability for NR coverage enhancement

Release:	Rel-18

Work Item:	8.19.1

Source:	TSG WG RAN4

To:	TSG WG RAN2

Cc:

Contact Person:

#### Name:  	Jung-Geun Chi

#### E-mail Address: 	junggeun.chi@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 has discussed the PowerClassPerBandPerBC-17 capability in power class for inter-band CA section 6.2A.1 of TS38.101-1 (highlighted below) and CR(R4-2318030) was agreed.

The HPUE operation including PowerClassPerBandPerBC-17 was defined in NOTE 8 of Table 6.2A.1.3-1, however, the PowerClassPerBandPerBC-r17 does not include Power Class 5. To clarify high power operation for inter-band CA. RAN4 would like to ask RAN2 that per-band per BC power class capability include power class 5 since Rel-18.

Table 6.2A.1.3-1 UE Power Class for uplink inter-band CA (two bands) Uplink CA Configuration; Class 1 (dBm); Tolerance (dB); Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB); Class 4 (dBm); Tolerance (dB). CA_n1A-n3A; 23; +2/-3. unnecessary rows are omitted. NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise..

2. Actions:

To RAN WG2 group:

ACTION: RAN4 respectfully ask RAN2 to take the above information into consideration and to include power class 5 in per-band per BC power class capability since Rel-18.

3. Date of Next TSG WG RAN4 Meetings:

RAN4#110-bis	Apr 15th – 19th	Changsha, China

RAN4#111	May 20th – 24th	Fukuoka, Japan