3GPP TSG RAN WG2#125bis	R2-2402183

Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4643 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Redcap 1 Rx and 2 Rx barring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2024-03-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the last RAN2 meeting, correction to RedCap UE regarding to 1RX/2RX barring according to R2-2400828 was not agreed. Current RRC spec states that a cell can be barred for the RedCap UE which is equipped with 1Rx branch or 2Rx branches, but the RedCap UE’s support of 1Rx or 2Rx operation may vary for different frequency bands. Therefore, the correct statement is not accurate |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the cell is considered as barred if the cellBarredRedCap1Rx is present and set to barred and the UE supports only 1 Rx branch operation on the selected band. Clarify that the cell is considered as barred if the cellBarredRedCap2Rx is present and set to barred and the UE supports 2Rx branch operation on the selected band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE’s barring behavior is not clear as RedCap UE’s suport of 1Rx or 2Rx may be different for different frequency bands. Impact analysis Impacted functionality: Barring of 1 Rx and 2 Rx RedCap UEs. Inter-operability: - If the network is implemented according to the CR and the UE is not, RedCap UE supporting 1 Rx for some frequency bands may consider a cell as barred/not barred although the cell does not indicate barred/not barred for 1 Rx, or RedCap UE supporting 2 Rx for some frequency bands may consider a cell as barred/not barred although the cell does not indicate barred/not barred for 2 Rx. - If the UE is implemented according to the CR and the network is not, there is no problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .... TS/TR ... CR .... TS/TR ... CR .... |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselectionRedCap is set to allowed, upon which the procedure ends;

2> else:

3>	if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE supports only 1 Rx branch operation on the selected frequency band from the frequencyBandList; or

3>	if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE supports 2 Rx branches operation on the selected frequency band from the frequencyBandList; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;