3GPP TSG-RAN WG2 #125bis	R2-2402207

Changsha, China, April 2024

Agenda Item:	7.9.6

Source:	OPPO

Title:	Discussion on duplicated PDU submitted to indirect path RLC

Document for:	Discussion, Decision

Introduction

This paper will discuss when the duplicated PDCP PDU is submitted to indirect path RLC.

Discussion

We have agreed to use Duplication RLC Activation/Deactivation MAC CE to indicate whether the RLC entity on indirect path in is activated or not, so there may be a case when PDCP duplication is activated in MP relay but the RLC entity on indirect path is deactivated by Duplication RLC Activation/Deactivation MAC CE, in that case, the data should only be submitted to the associated activated Uu RLC entities.

There may be a case when PDCP duplication is activated in MP relay but the RLC entity on indirect path is deactivated by Duplication RLC Activation/Deactivation MAC CE, in that case, the data should only be submitted to the associated activated Uu RLC entities.

Currently, whether the RLC entity on indirect path is activated for duplication is not considered when submitting data to both paths, which means the above case is missing in the specification (clause 5.2.1). The proposed correction is as follows:

### 5.2.1 Transmit operation At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall: - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured). NOTE 0: Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation. For a PDCP SDU received from upper layers, the transmitting PDCP entity shall: - associate the COUNT value corresponding to TX_NEXT to this PDCP SDU; NOTE 1: Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation. - perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4; - perform uplink data compression of the PDCP SDU as specified in clause 5.14.4; - perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively; - set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL]; - increment TX_NEXT by one; - submit the resulting PDCP Data PDU to lower layer as specified below. When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall: - if the transmitting PDCP entity is associated with one SRAP entity: - submit the PDCP PDU to the associated SRAP entity; - else, if the transmitting PDCP entity is associated with one RLC entity: - submit the PDCP PDU to the associated RLC entity; - else, if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C: - if PDCP duplication is activated for the RB: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to both the MP primary path and MP secondary path, including any associated RLC entities or SL RLC entity associated with the SRAP entity activated for PDCP duplication; - else: - submit the PDCP Control PDU to the MP primary path; - else (i.e., PDCP duplication is deactivated for the RB): - if the total amount of PDCP data volume, RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the RLC entity, and data volume pending for either transmission in the N3C (if available) or mapped SL RLC entity associated with the SRAP entity, is equal to or larger than ul-DataSplitThreshold: - submit the PDCP PDU to either the MP primary path or MP secondary path; - else: - submit the PDCP PDU to the MP primary path; - else, if the transmitting PDCP entity is associated with at least two RLC entities: - if the PDCP duplication is activated for the RB: - if the PDCP PDU is a PDCP Data PDU: - duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication; - else: - submit the PDCP Control PDU to the primary RLC entity; - else (i.e. the PDCP duplication is deactivated for the RB or the RB is a DAPS bearer): - if the split secondary RLC entity is configured; and - if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold: - submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity; - else, if the transmitting PDCP entity is associated with the DAPS bearer: - if the uplink data switching has not been requested: - submit the PDCP PDU to the RLC entity associated with the source cell; - else: - if the PDCP PDU is a PDCP Data PDU: - submit the PDCP Data PDU to the RLC entity associated with the target cell; - else: - if the PDCP Control PDU is associated with source cell: - submit the PDCP Control PDU to the RLC entity associated with the source cell; - else: - submit the PDCP Control PDU to the RLC entity associated with the target cell; - else: - submit the PDCP PDU to the primary RLC entity. NOTE 2: If the transmitting PDCP entity is associated with two RLC entities, or with one or more RLC entities and either an SRAP entity or the N3C, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities, or to the one or more RLC entities and either the SRAP entity or the N3C, to minimize PDCP reordering delay in the receiving PDCP entity.

In clause 5.2.1, when submit data to both paths, whether the PC5 RLC entity on indirect path is activated should also be considered as the above TP.

Conclusion

We have the following observations:

Observation 1	There may be a case when PDCP duplication is activated in MP relay but the RLC entity on indirect path is deactivated by Duplication RLC Activation/Deactivation MAC CE, in that case, the data should only be submitted to the associated activated Uu RLC entities.

We have the following proposals:

Proposal 1	In clause 5.2.1, when submit data to both paths, whether the PC5 RLC entity on indirect path is activated should also be considered as the above TP.