3GPP TSG-RAN WG2 #125bis								R2-2402225

Changsha, China, April 2024

Agenda Item:	7.15.1

Source:	OPPO

Title:	RRC Open Issue list for R18 SL-Evo

Document for:	Discussion, Information

Introduction

This is for the open issue list of R18 SL-Evo.

Discussion

| Open Issue | Rapporteur View |
| --- | --- |
| Issue-1: Editor's note: The value range of sl-PSFCH-PowerOffset may need to be updated based on RAN4 reply LS. It comes from the NOTE in R1 RRC parameter list, i.e., “Note: this value range may need to be updated based on RAN4 reply LS”. | Based on R4 LS reply in R4-2321767, that o P_common < P_dedicated is feasible given that the evaluated MPR value is no higher than that for P_common= P_dedicated for all PSFCH transmissions with Alt 1-1b based on RAN4’s current understanding in Rel-18. There is no need to change the value range of sl-PSFCH-PowerOffset. |

Reference

- 3GPP RP-230077, WID revision: NR sidelink evolution, OPPO