3GPP TSG-RAN WG2 Meeting #125                                                                  R2-2402258

Changsha, China, April 15th – 19th, 2024

Title:	Questions on PRS and SRS bandwidth aggregation

Response to:

Release:	Release 18

Work Item:	NR_pos_enh2-Core

Source:	RAN WG2

To:	RAN WG1

Cc:	RAN WG3, RAN WG4

Contact Person:

Name:	Jianxiang Li

E-mail Address:	lijianxiang@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the measurement report in case of measurement is based on aggregated PRS bandwidth configuration, and also the activation/deactivation MAC CE for SP SRS with bandwidth aggregation, and the following questions need further to check with RAN1.

On PRS bandwidth aggregation:

From RAN2 perspective, UE also need to indicate the used PRS resource for measurement within each PRS aggregated resource sets to LMF within the measurement report. However, there is not such indication in the RAN1 parameter list. So RAN2 would like to check with RAN1:

Question 1: Is the indication of associated PRS resource for each aggregated PRS resource set required for joint measurements when UE report the measurement results to LMF?

On SRS bandwidth aggregation:

RAN2 discussed the design of the SP SRS bandwidth aggregation MAC CE, and agreed that the spatial relation is not needed to be indicated in the MAC CE, from RAN2 perspective.  The reason is that the spatial relationship is already indicated in SRS configuration, so UE knows which aggregated SRS resources have the same spatial relationship. Besides, the activation/deactivation is per SRS resource set, and all SRS resources in a set are activated at the same time. When MAC CE activates/deactivates a CA combination, all the linked SRS resources are activated, including the spatial relations, which is known to the UE.

Question 2: RAN2 would like RAN1 to confirm whether the above understanding is correct.

2. Actions:

To RAN1

ACTION:	 RAN2 kindly asks RAN1 to answer the above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#126                            2024-05-20 to 2024-05-24		Fukuoka City, Fukuoka , JP

TSG-RAN WG2#127                            2024-08-19 to 2024-08-23		Maastricht , NL