3GPP TSG-RAN WG2 Meeting #125bis	R2-2402329

Changsha, China, April 15th – 19th, 2024

Agenda Item:	7.24.2.2

Source: 	vivo, Samsung

Title:  	Discussion on specification modification of redirection to GERAN

Document for:	Discussion and Decision

Introduction

In the last meeting, the issue of redirection to GERAN was raised in R2-2400167 [1]. We understand that the companies think the issue of redirection to GERAN was valid according to the online discussion. However, how to modify the specification to fix this issue did not reach a consensus in the last meeting. The chairman notes regarding the discussion on this issue are excerpted as follows [2].

Redirection to GERAN R2-2400167 Discussion on redirection to GERAN vivo discussion Rel-17 TEI18 R2-2400168 Correction on redirection to GERAN vivo CR Rel-17 36.331 17.7.0 4982 - F TEI18 - Nokia thinks the last procedure text is not needed - Qualcomm wonders why we don’t remove the full content. => Remove last change => update to proper formatting and change to Rel-18 CR => The CR will be updated in R2-2401932 with all the changes above Aftercomeback R2-2401932 Correction on redirection to GERAN vivo CR Rel-17 36.331 17.7.0 4982 1 F TEI18 - Nokia is concerned that part of this container has been sent to the NAS, RRCEarlyDataComplete, so how do you ignore it. => Postponed

In this contribution, we will give the way we expect to modify the specification and give our reasons.

Discussion

According to ASN.1 of RRCEarlyDataComplete below [3], there are four parts in RRCEarlyDataComplete which are highlighted by different colors, these parameters can be divided into two types, i.e., generated by eNB and generated by CN.

RRCEarlyDataComplete-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataComplete-r15			RRCEarlyDataComplete-r15-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCEarlyDataComplete-r15-IEs ::=	SEQUENCE {

dedicatedInfoNAS-r15				DedicatedInfoNAS					OPTIONAL,	-- Need ON

extendedWaitTime-r15				INTEGER (1..1800)					OPTIONAL,	-- Need ON

idleModeMobilityControlInfo-r15		IdleModeMobilityControlInfo			OPTIONAL,	-- Need OP

idleModeMobilityControlInfoExt-r15	IdleModeMobilityControlInfo-v9e0	OPTIONAL,	-- Cond IdleInfoEUTRA

redirectedCarrierInfo-r15			RedirectedCarrierInfo-r15-IEs		OPTIONAL,	-- Need ON

nonCriticalExtension				RRCEarlyDataComplete-v1590-IEs	OPTIONAL

}

RRCEarlyDataComplete-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

RedirectedCarrierInfo-r15-IEs ::=	CHOICE {

eutra						ARFCN-ValueEUTRA-r9,

geran						CarrierFreqsGERAN,

utra-FDD					ARFCN-ValueUTRA,

cdma2000-HRPD				CarrierFreqCDMA2000,

cdma2000-1xRTT				CarrierFreqCDMA2000,

utra-TDD					CarrierFreqListUTRA-TDD-r10

}

UE shall not apply the parameters generated by eNB if UE determines that eNB is a fake base station. If redirectedCarrierInfo indicating redirection to geran, utra-FDD, utra-TDD or utra-TDD-r10 or idleModeMobilityControlInfo including freqPriorityListGERAN, freqPriorityListUTRA-FDD or freqPriorityListUTRA-TDD under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed, the eNB is probably a fake base station. So RRC parameters (i.e., extendedWaitTime, idleModeMobilityControlInfo and redirectedCarrierInfo) should be ignored under such case.

Observation 1: extendedWaitTime, idleModeMobilityControlInfo and redirectedCarrierInfo in RRCEarlyDataComplete should be ignored if the NW configures UE redirect to GERAN or UTRAN under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed.

Next, whether to forward the parameters generated by CN (i.e., dedicatedInfoNAS) from AS to NAS should be discussed if eNB is a fake base station. One understanding is that a fake base station may tamper with the content of dedicatedInfoNAS or send a fake dedicatedInfoNAS to the UE, so AS should not forward dedicatedInfoNAS to NAS. The other understanding is that it should be up to the NAS layer to determine whether to use or discard this information since dedicatedInfoNAS is used to transfer UE specific NAS layer information between the network and the UE.

Observation 2: RAN2 should discuss whether to forward dedicatedInfoNAS from AS to NAS upon reception of the RRCEarlyDataComplete, if the NW configures UE redirect to GERAN or UTRAN under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed.

Considering there are two understandings of whether to forward dedicatedInfoNAS from AS to NAS, there are two options for specification modification of the procedure of reception of the RRCEarlyDataComplete to avoid UE applying unsecured configuration if redirectedCarrierInfo indicating redirection to geran, utra-FDD, utra-TDD or utra-TDD-r10 or idleModeMobilityControlInfo including freqPriorityListGERAN, freqPriorityListUTRA-FDD or freqPriorityListUTRA-TDD under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed.

- Option 1: UE does not forward the dedicatedInfoNAS from AS to NAS, and ignores the whole content of RRCEarlyDataComplete;

- Option 2: UE ignores the whole content of RRCEarlyDataComplete except dedicatedInfoNAS, but still forwards the dedicatedInfoNAS from AS to NAS.

Proposal 1: RAN2 discuss which options shall be adapted.

Option 1: UE does not forward the dedicatedInfoNAS from AS to NAS, and ignores the whole content of RRCEarlyDataComplete;

Option 2: UE ignores the whole content of RRCEarlyDataComplete except dedicatedInfoNAS, but still forwards the dedicatedInfoNAS from AS to NAS.

Conclusions

Based on the analyses given above, we have the following observations and proposals:

Observation 1: extendedWaitTime, idleModeMobilityControlInfo and redirectedCarrierInfo in RRCEarlyDataComplete should be ignored if the NW configures UE redirect to GERAN or UTRAN under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed.

Observation 2: RAN2 should discuss whether to forward dedicatedInfoNAS from AS to NAS upon reception of the RRCEarlyDataComplete, if the NW configures UE redirect to GERAN or UTRAN under the case that upper layers indicate that redirect to GERAN or UTRAN without AS security is not allowed.

Proposal 1: RAN2 discuss which options shall be adapted.

Option 1: UE does not forward the dedicatedInfoNAS from AS to NAS, and ignores the whole content of RRCEarlyDataComplete;

Option 2: UE ignores the whole content of RRCEarlyDataComplete except dedicatedInfoNAS, but still forwards the dedicatedInfoNAS from AS to NAS.

Reference

- R2-2400167 Discussion on redirection to GERAN

- Chairman_notes of 3GPP TSG RAN WG2 meeting #125

- TS 36.331 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)