3GPP TSG-RAN WG2 Meeting #125bis	R2-2402356

Changsha, China, April 15th – 19th, 2024

Agenda Item:	8.8.5

Source: 	CATT, China Broadnet

Title: 	Discussion on support of regenerative payload in Rel-19 NR NTN

Document for: 	Discussion and Decision

Introduction

According to the latest WID for NR NTN Phase 3 [1], one of the objectives is to support regenerative payload:

- Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In the document, we analyze potential RAN2 impacts to support regenerative payload, and provide our observations and proposals.

Discussion

We analyse the main L2 and RRC functionalities to see whether/what RAN higher-layer impacts are needed in NR Uu to support the regenerative payload, compared with the transparent payload already supported in Rel-17/18. Our observations are summarized in the below table. One can refer to the Appendix for the detailed technical analyses on how these observations (in the "Conclusion" column) are derived.

Table 1: Impacts analysis on the case of gNB on board for regenerative payload

|  | Impact analysis |  |  |  | Conclusion |
| --- | --- | --- | --- | --- | --- |
| Architecture |  |  |  |  | OA.1: The propagation delay on Uu in regenerative payload of gNB on board is smaller than that in transparent payload. |
| RRC (RAN L3) | (C)HO | Based on distance between UE and reference location or based on t-Service, all required (C)HO related functionalities in NR NTN have been supported by the satellite ephemeris information, reference location/t-Service and UE position based on Rel-17/18 NR NTNdesign. |  |  | OA.2: No impact on RAN L3 is identified for enhancements on regenerative payload of gNB on board on top of Rel-17 and Rel-18 for NTN. |
|  | Measurement | The SMTC offset is adjusted according to the PDD between service satellite and neighbor satellite. It is derived based on satellite assistance information which can already have been supported by current SIB19 with the k-mac, TA common set to be 0 |  |  |  |
|  | Cell reselection | Similar situation as to (C)HO. |  |  |  |
|  | Access control | Based on above observation and the following observation, current stage 3 spec can support regenerative payload without additional UE capability, so regenerative payload specific access control infomration e.g. cell bar, UAC, is not needed. |  |  |  |
| L2 | SDAP | No impacts on SDAP in NTN. |  |  | OA.3: The regenerative payload can be supported in SDAP based on current specification for the case of gNB on board |
|  | PDCP | The discardTimer is extended handle the long propagation delay in Rel-17 NTN. But for gNB on board, the RTT between the UE and gNB decreased, which can already have been supported by current value range specified. |  |  | OA.4: The regenerative payload can be supported in PDCP/RLC based on current specification for the case of gNB on board. |
|  | RLC | Timer t-Reassembly is extended handle the long propagation delay in Rel-17 NTN. But for gNB on board, the RTT between the UE and gNB decreased, which can already have been supported by current value range specified. |  |  |  |
|  | MAC | HARQ | HARQ number does NOT need to be extended to avoid HARQ stalling. |  | OA.7: The regenerative payload can be supported in MAC based on current specification for the case of gNB on board. |
|  |  | CG/SPS | Further extension of configuredGrantTimer and harq-ProcId-Offset2 are NOT needed. |  |  |
|  |  | SR | The value of sr-ProhibitTimer is NOT necessarily further extended. |  |  |
|  |  | TA | TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc, where NTA,adjcommon=0 for gNB on board | OA.5: For regenerative payload, the equation of TA can be reused. |  |
|  |  | UE-gNB-RTT | UE-gNB-RTT = TA+Kmac, where Kmac can be configured to be 0. | OA.6: The definition of UE-gNB RTT can be reused for regenerative payload. |  |
|  |  | RACH | No impacts on RACH considering UE-gNB-RTT can be reused based on OA.6. |  |  |
|  |  | DRX | No impacts on DRX considering UE-gNB-RTT can be reused based OA.6. |  |  |
|  |  | TAR | No impacts since the trigger events can be reused for gNB on board. |  |  |
|  |  | Koffset | No impacts based on OA.1. |  |  |

According to above analysis, regenerative payload is mainly to support new architecture without additional UE capability, meaning that stage 3 impact is not needed. Therefore, we have the following observation and proposals for RAN2 to confirm:

Observation 1: For the case of gNB on board, current spec can already support regenerative payload from NR Uu perspective.

Proposal 1: From RAN2’s perspective, no stage-3 impact on NR Uu is needed to support regenerative payload, unless potential enhancement is identified based on future inputs from RAN3/SA2.

By contrast, some Stage-2 Spec impact may be needed to incorporate the regenerative payload deployment into the RAN2 Spec, and this can be done by RAN2. Also, some high-level clarification for regenerative payload may also be added in the Stage-2 Spec, e.g. Common TA and kmac = 0, NR Uu only concerns service link, etc..

Proposal 2: RAN2 incorporates the regenerative payload deployment into Stage-2 Spec, including some high-level clarifications on regenerative specific aspects in NR Uu (e.g. Common TA and kmac = 0, NR Uu only concerns service link, etc.).

- 3.	Conclusion

In this document, we analyse the potential impacts brought by regenerative mode of gNB on board, and find the following observations and proposals:

Observation 1: For the case of gNB on board, current spec can already support regenerative payload from NR Uu perspective.

Proposal 1: From RAN2’s perspective, no stage-3 impact on NR Uu is needed to support regenerative payload, unless potential enhancement is identified based on future inputs from RAN3/SA2.

Proposal 2: RAN2 incorporates the regenerative payload deployment into Stage-2 Spec, including some high-level clarifications on regenerative specific aspects in NR Uu (e.g. Common TA and kmac = 0, NR Uu only concerns service link, etc.).

- 4.	Reference

- RP-234078, Non-Terrestrial Networks (NTN) for NR Phase 3;

- Appendix: Detailed analyses for Table 1

When gNB is on-board, the NTN payload plays a role to process the radio protocol received from the UE and regenerates the radio protocol to the UE, and vice versa. Meanwhile, it keeps the connection with the core network. The whole architecture is depicted in Figure 1.

Figure 1: Illustration of architecture for regenerative payload of gNB on board

And the Timing information is shown in Figure 2.

Figure 2: Timing relationship in gNB on board case for regenerative payload

As illustrated in Figure 1 and 2, radio protocol including L3 and L2 will be processed in NTN payload. Hence, the propagation delay on Uu will be smaller than that in transparent payload.

OA.1: The propagation delay on Uu in regenerative payload of gNB on board is smaller than that in transparent payload.

In the following we analyze the potential impacts and identify whether the impact may bring spec impact.

## A.1	Analysis on RRC (RAN L3)

For L3 analysis, we focus on the timing related enhancements introduced in Rel-17 and Rel-18 NTN and access control, including (C)HO enhancements, Measurement cell reselection and access control.

(C)HO enhancement

In NTN, (cond)EventD1, (cond)EventD2 and condEventT1 are introduced to facilitate the UE and the network to make handover decisions considering the UE can obtain the serving time of the satellite or estimate the distance between the UE and satellite based on the satellite ephemeris information and reference location. For gNB on-board in generative payload, it mainly impacts the RTT which does not have the impact the configuration for these events which mainly relate to satellite ephemeris information.

Measurement

In Rel-17, the offset of SMTC is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. So, the offset is related to the PDD between serving cell and neighbor cells, not for the service link which is between gNB and UE for regenerative payload.

Cell reselection

In NTN, time-based and location-based criteria for cell reselection were adopted. Like HO, satellite ephemeris information other than RTT is essential for deriving the cell reselection criteria.

Access control

Based on above analysis on RRC and the analysis on L2, we can find the regenerative payload doesn’t need to introduce additional UE capability, it is mainly impact on the architecture. Current stage 3 spec can supports it already. Hence regenerative payload specific access control information e.g. cell bar, UAC etc, does not need to be introduced.

Based on the analysis above, it can be concluded that there is no impact to L3 for the scenario that gNB is on board.

OA.2: No impact on RAN L3 is identified for enhancements on regenerative payload of gNB on board on top of Rel-17 and Rel-18 for NTN.

## A.2	Analysis on L2

### A.2.1	Analysis on SDAP

As there is no impact on SDAP brought by NTN in Rel-17/18, the regenerative payload of gNB on board has no impact on the protocol either.

OA.3: The regenerative payload can be supported in SDAP based on current specification for the case of gNB on board.

### A.2.2	Analysis on PDCP/RLC

In Rel-17, in order to handle the long propagation delay between UE and the gNB which consists of the service link and the feeder link in NTN, the value for timer t-Reassembly in RLC was extended. And the discardTimer in PDCP which mainly reflects the QoS requirements of the packets belonging to a service was extended in Rel-17 similarly.

However, since the gNB is on board, the RTT between the UE and gNB decreased. Hence, the values defined in Rel-17 are sufficient.

Therefore, it can be seen that:

OA.4: The regenerative payload can be supported in PDCP/RLC based on current specification for the case of gNB on board.

### A.2.3	Impacts on MAC

Value extension

    - HARQ process number

In order to avoid HARQ stalling considering enough TBs are unsuccessfully decoded (resulting in the associated HARQ PIDs not being reusable for a long time) due to the large propagation delay in NTN, the HARQ process number are extended in NTN.

    - CG

In Rel-17 NTN, configuredGrantTimer and harq-ProcId-Offset2 were both extended considering the propagation delay.

    - SR

In Rel-17 NTN, the value of sr-ProhibitTimer is extended keep running in the time period that the UE does not have a chance to receive the NW scheduling in response to receive the SR from UE.

However, the extension of these values or timers is due to the large propagation delay in NTN. According to OA.1, i.e., the propagation delay in regenerative payload is smaller than that in transparent payload, there is no need to extend the values or timers above. Hence, no impacts on these procedures by regenerative payload of gNB on board are identified.

Delay the start of some specific timers

One terminology UE-gNB RTT was introduced when the timers are required to delay being started considering the long propagation delay in NTN. UE-gNB RTT is the sum of the UE's Timing Advance (TA) value and kmac.

For TA, according to the spec, it can be calculated based on the following equation: TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc where

-	NTA and NTA,offset are derived based on the network indication or using default values;

-	NTA,adjcommon is derived from the parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

For regenerative payload, the propagation delay of feeder link is zero. Hence, NTA,adjcommon is 0 for regenerative payload. Therefore, the Equation for TA still works with assumption that parameters TACommon, TACommonDrift, and TACommonDriftVariation are not configured.

For kmac, value 0 has already been specified in the spec.

kmac Scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. If the field is absent UE assumes value 0. In FR1, the unit of kmac is number of slots for a given subcarrier spacing of 15 kHz.

Therefore, the definition of UE-gNB RTT can be reused for regenerative payload.

OA.5: For regenerative payload, the equation of TA can be reused.

OA.6: The definition of UE-gNB RTT can be reused for regenerative payload.

    - Random access

In Rel-17 NTN, RAR/MSGB window and contention resolution window will be started after one offset. For RAR/MSGB window, this procedure has been specified in 38.213:

## 8.2 Random access response - Type-1 random access procedure In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321] if the PRACH transmission is not triggered by a PDCCH order that includes a Cell Indicator field with non-zero value; otherwise, the UE does not attempt to detect the DCI format 1_0. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the last PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow. /Omitted/

## 8.2A Random access response - Type-2 random access procedure In response to a transmission of a PRACH and a PUSCH, or to a transmission of only a PRACH if the PRACH preamble is mapped to a valid PUSCH occasion, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding MsgB-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by msgB-ResponseWindow. /Omitted/

Actually, TTA+kmac is the UE-gNB RTT in MAC. Based on OA.5, there is no need to change the procedure for starting of RAR/MSGB window.

Likely, the procedure for starting contention resolution window which is started in the first symbol after the end of the Msg3 transmission plus the UE-gNB RTT can also remain unchanged.

Hence, there is no impact on Random access procedure for regenerative payload.

    - DRX

Based on current DRX procedure for NTN, the MAC entity will be considered Active Time after the SR transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT.

And timer HARQ-RTT-TimerDL-NTN/HARQ-RTT-TimerUL-NTN will set to drx-HARQ-RTT-TimerDL/ HARQ-RTT-TimerUL-NTN plus the latest available UE-gNB RTT value respectively, when HARQ feedback is enabled.

Based on OA.6, UE-gNB RTT can be reused for regenerative payload for regenerative payload, these procedures are not impacted either.

Other enhancements in NTN

    - TAR

Regarding TAR, it will be triggered when the following events occur:

- upon indication from upper layers to trigger a Timing Advance report; - upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell; - if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

These events are not related to the service link delay. Hence, there is no impact on TAR.

    - Koffset

Regarding Koffset, it is used to indicate the Scheduling offset used for the timing relationships that are modified for NTN which needs to be larger or equal to the sum of the service link RTT and the Common TA. Based on OA.1, the current value of Koffset is sufficient.

OA.7: The regenerative payload can be supported in MAC based on current specification for the case of gNB on board.