3GPP TSG-RAN WG2 #125-bis	                                  R2-2402386

# Changsha, China, April 15~19, April 2024

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1790 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous correction to TS 38.321 (rapporteur’s CR) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  |  | Date: |  | 2024-04 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Misc editorial changes are needed as clarified below: - In clause 5.4.9, it is not clear which type of SDU used in the DSR procedure refers to. It should be clarified as PDCP SDUs. - In clause 5.4,9, the singular form used in the first bullet level in the DSR triggering conditions should be changed to plural form. - In RAN2#125, RAN2 agreed that “It is up to UE implementation whether to cancel DSR even if the MAC PDU can accommodate all the delay-critical data but is not sufficient to include the DSR MAC CE and its subheader (i.e. similar to BSR).” The intention of this agreement is that all the pending DSRs shall still be canceled in the specified scenario but what the relaxation means is that it is up to UE implementation whether to include a DSR MAC CE in the MAC PDU. The current spec does not correctly capture this intention. Instead, it implies that UE may not cancel pending DSRs if a DSR MAC CE with delay status information of all SDUs associated with all the pending DSRs is included in a MAC PDU. - Need to clarify that UE only needs to check the availability conditions for a configured uplink grant only if it is configured with UTO-UCI reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.4.9, replace “SDU” in the DSR procedures by “PDCP SDU”. - In clause 5.4,9, the singular form used in the first bullet level in the DSR triggering conditions is changed to plural form. - In clause 5.4.9, change “can accommodate” to “includes” in the sentence “The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs”, because “accommodate” is more related to the size of a MAC PDU but “include” implies after PDU assembly. In addition, add a note which clarifies that “It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally include the DSR MAC CE and its subheader”. - In clause 5.8.2, add the clarification “configured with UTO-UCI reporting” to the top level in the availability conditions of a configured uplink grant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The texts that need change may cause confusion to users of the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9, 5.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- [Start of the first three changes] ------------------------------------------

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimer among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold: the threshold on remaining time for triggering a DSR for an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall:

1>	if the smallest remaining value of the running PDCP discardTimer among all the PDCP SDUs buffered for the LCG that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the LCG:

2>	trigger a DSR for the LCG.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE: 	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

---------------------------------- [End of the first three changes] -------------------------------------------

-------------------------------------- [Start of the 4th change] -----------------------------------------------

### 5.8.2	Uplink

(Text omitted)

For a configured uplink grant configured with UTO-UCI reporting (as specified in clause 9.3 in TS 38.213 [6]) , the MAC entity shall:

1>	if the configured uplink grant is associated with a multi-PUSCH configured grant:

2>	if the configured uplink grant has not been indicated to the lower layers as to be unused for PUSCH transmission; and

2>	if the configured uplink grant meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]:

3>	consider the configured uplink grant available for use;

1>	else if the configured uplink grant has not been indicated to lower layers as to be unused for PUSCH transmission:

2>	consider the configured uplink grant available for use.

The MAC entity determines if a configured uplink grant configured with UTO-UCI reporting is going to be used for PUSCH transmission or not by considering the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to the lower layers regarding this decision, for use in the procedure for reporting UTO-UCI .

(Text omitted)

---------------------------------------- [End of the 4th change] ---------------------------------------------