3GPP TSG-RAN WG2 Meeting #125bis	R2-2402471

# Changsha, P. R. China, 15th - 19th Apr., 2024

Title:		Remaining issues on MAC spec for R18 positioning

Source:	Huawei, HiSilicon

Agenda item:	7.2.6

Document for:	Discussion and Decision

- Background

This paper discusses the remaining RAN2 issues in the MAC spec for R18 positioning

2	Discussions

2.1	SP SRS activation/deactivation for CA positioning

The following has been agreed in RAN1 according to the LS R1-2401708

Hence, from the agreement above, it can be seen that RAN1 has reached the agreement that it is possible to configure positioning SRS on SCell not configured with DL, but containing only positioning SRS. The Positioning SRS configuration will be under the same framework for SCell configuration as now.

If the above agreements holds, for the activation/deactivation of SP Positioning SRS, it means that the SP SRS configuration always corresponds to an SCell configuration, hence has a SCell index. If this is the case, we think that previous SP SRS activation/deactivation MAC CE can still be reused for CA positioning. Although the use case for CA positioning is that the network needs to activate or deactivate multiple SP SRS at the same time, it would be sufficient if the network includes multiple SP SRS activation/deactivation MAC CE within a single DL MAC PDU. Then, the wanted functionality still can be achieved

Therefore, we propose the following:

Proposal1: Reuse the legacy SP SRS activation/deactivation MAC CE for CA positioning.

2.2	Maximum number of parallel SL-PRS to Multiple UEs

During the last RAN1 meeting, the following has been agreed and sent in the reply LS to RAN2 in R1-2401552

For the current spec, the following has been defined for the maximum number of parallel HARQ processes for shared/dedicated resource pool

From our thinking, similar with the other discussion on shared SL-PRS resource pool, the legacy procedures and conditions should be followed since the intention is to reused the legacy pool for PRS transmission.

Proposal2: The maximum number of transmitting sidelink processes for SL-PRS for each carrier is 16.

Proposal3: For SL-PRS transmission by sidelink resource allocation Scheme 2, RAN2 confirms on the following:

- For SL-PRS transmission on shared resource pool, the maximum number of transmitting sidelink processes associated with the sidelink HARQ entity is 4

- For SL-PRS transmission on dedicated resource pool, the maximum number of transmitting sidelink processes associated with the sidelink carrier is 4.

Adopt the following TP

##### 5.22.1.3.1 Sidelink HARQ Entity The MAC entity is configured by upper layers to transmit using pool(s) of resources on one or more carriers as indicated in clause 5.8.8 of TS 38.331 [5]. For each carrier, the MAC entity includes at most one Sidelink HARQ entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes. The maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 16. The maximum number of parallel SL-PRS transmissions associated with each sidelink carrier is 16. A sidelink process may be configured for transmissions of multiple MAC PDUs or multiple SL-PRSs. For transmissions of multiple MAC PDUs with Sidelink resource allocation mode 2 and with Sidelink resource allocation Scheme 2 on SL-PRS shared resource pool, the maximum number of transmitting Sidelink processes associated with the Sidelink HARQ Entity is 4. For transmission of multiple SL-PRSs with Sidelink resource allocation Scheme 2 on SL-PRS dedicated resource pool, the maximum number of transmitting Sidelink processes associated with the sidelink carrier is 4. A delivered sidelink grant and its associated Sidelink transmission information are associated with a Sidelink process. Each Sidelink process supports one TB.

3	Conclusions

Proposal1: Reuse the legacy SP SRS activation/deactivation MAC CE for CA positioning.

Proposal2: The maximum number of transmitting sidelink processes for SL-PRS for each carrier is 16.

Proposal3: For SL-PRS transmission by sidelink resource allocation Scheme 2, RAN2 confirms on the following:

- For SL-PRS transmission on shared SL-PRS resource pool, the maximum number of transmitting sidelink processes associated with the sidelink HARQ entity is 4

- For SL-PRS transmission on dedicated SL-PRS resource pool, the maximum number of transmitting sidelink processes associated with the sidelink carrier is 4.