3GPP TSG-RAN WG2 Meeting #125	bis                                                               R2-2402537

Changsha, China, April 15th – 19th, 2024

Source:	CATT

Title:	Discussion on CG-SDT Related TAT Handling with Two TAs

Agenda Item:	7.20.2

Document for:	Discussion and Decision

- 1. Introduction

In last meeting, how to handle the TAT when considering the CG-SDT co-existence with two TAs was discussed based on contribution [1], but without any consensus on this issue. In this paper, we further make discussion on this issue.

- 2. Discussion

Configuration of CG-SDT and two TAs upon RRC Release

The CG-SDT aims to use the CG resource configured in RRCRelease to perform data transmission in RRC_INACTIVE. Currently, upon receiving RRCRelease with suspend,

1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; 2> if the sdt-Config is configured: … 3> if sdt-MAC-PHY-CG-Config is configured: 4> configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

The above procedure means UE will continue the TA in RRC_CONNECTED and start the cg-SDT-TimeAlignmentTimer used for CG-SDT to monitor the validity of the TA. Then the issue is if UE is configured with two TAs in RRC_CONNECTED, which TA to continue using in RRC_INACTIVE for CG-SDT. Generally speaking, there two ways to solve this issue:

Option 1: CG-SDT and two-TA coexistence for PCell is not supported. If the NW wants to configure UE with CG-SDT in RRC_INACTIVE, it can just re-configure UE without two-TA for PCell.

Option 2: To restrict that the cg-SDT-TimeAlignmentTimer is always associated with the TA of PTAG ID=0.

Option 1 just leaves this issue to NW implementation, i.e., the NW can send the RRCReconfiguration to configure the PCell without 2TA configuration before releasing UE to RRC_INACTIVE, otherwise, the NW does not configure CG-SDT. If we go with Option 1, we only need to clarify the condition to configure CG-SDT by RRCRelease in the RRC spec. By this way, it can avoid other potential impact on legacy CG-SDT.

While if we go with Option 2, it requires the NW to only configure the CG resource associated with SSB(s) of PTAG ID=0. UE always considers the TA associated with PTAG with TAG ID 0 to be continued in RRC_INACTIVE if configured with CG-SDT. Thus, in addition to clarify the cg-SDT-timeAlignmentTimer only applies to the TA associated with TAG-ID (not TAG2-ID), it further needs to clarify in the field description of sdt-SSB-Subset that the SSB subset used to select CG resource can only associate with the TAG-ID but not TAG2-ID.

CG-SDT-Configuration-r17 ::= SEQUENCE {

cg-SDT-RetransmissionTimer   INTEGER (1..64)                                                 OPTIONAL,   -- Need R

sdt-SSB-Subset-r17       CHOICE {

shortBitmap-r17          BIT STRING (SIZE (4)),

mediumBitmap-r17         BIT STRING (SIZE (8)),

longBitmap-r17           BIT STRING (SIZE (64))

}                                                                                            OPTIONAL,   -- Need S

sdt-SSB-PerCG-PUSCH-r17   ENUMERATED {oneEighth, oneFourth, half, one, two, four, eight, sixteen}  OPTIONAL,   -- Need M

……

}

sdt-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not included in the SSB subset for SSB to CG PUSCH mapping while value 1 indicates that the corresponding SS/PBCH block is included in SSB subset for SSB to CG PUSCH mapping. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs. In case of an (e)RedCap-specific initial downlink BWP that is associated with NCD-SSB, the SSB is the NCD-SSB. Otherwise, the SSB is the CD-SSB.

Generally speaking, both ways are workable, but we think Option 1 has less impact on the spec and can avoid potential impact on legacy CG-SDT. Therefore, it is suggested that,

Proposal 1: CG-SDT should not be configured by RRCRelease if UE is configured with two TAs for PCell in RRC_CONNECTED. The TP in Annex 1 should be adopted.

Which TAT to start upon entering RRC_CONNECTED from CG-SDT procedure

If two TA is not configured before releasing UE to RRC_INACTIVE, as suggested by our P1, the NW can also configure UE with two TAs by RRCResume message. In this case, if UE is performing CG-SDT in RRC_INACTIVE, UE also needs to know which TAT (i.e., TAG) to start upon entering RRC_CONNECTED.

For the case of UE enters connected via RACH, the UE apply the TA value and starts the TAT based on the RACH procedure. Which TAG it should be associated is based on the handling of RAR/ successRAR/FallbackRAR. When UE starts the TAT, the UE hasn’t restored the UE context, hence two-TA is not applied for the UE. So for this case PTAG is clear, due to the fact that only one PTAG is applied.

Observation 1: For the case UE enters RRC Connected from inactive via RACH procedure, UE hasn’t restored the stored UE context when UE starts the TAT. Only one PTAG (TAG ID=0) is used for the UE. So no spec impact is introduced.

Different from UE entering RRC CONNECTED from inactive by RACH procedure, the TAT for the PTAG is started by instruction of RRC upon reception of RRCResume:

38.331: 1> if sdt-MAC-PHY-CG-Config is configured: 2> instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running; 2> instruct the MAC entity to start the timeAlignmentTimer associated with the PTAG, if it is not running; 38.321: 1> when instruction from the upper layer has been received for starting the TimeAlignmentTimer associated with PTAG: 2> start the TimeAlignmentTimer associated with PTAG.

That is if the two-TA is configured for PCell in RRCResume message, two PTAGs will be associated with the PCell, so it is not clear the TAT started is associated with which PTAG. So we think further clarification could be added in the 38.321, i.e. it is the PTAG with TAG ID 0 associated with the started TAT upon UE enters connected state from CG-SDT procedure.

Observation 2: For the case where UE enters RRC connected from on-going CG-SDT, it is upon the reception of RRCResume which could configure two-TA for the PCell. Since two PTAGs are associated with the PCell, it is not clear which PTAG is associated with the applied TA based on current 38.321.

In order to make it clear, we suggest to clarify in the MAC spec that when the UE enters RRC connected from on-going CG-SDT, the UE starts the TAT associated with the PTAG with TAG ID 0.

In summary, it is proposed that,

Proposal 2: When UE transits from ongoing CG-SDT in RRC_INACTIVE to RRC CONNECTED, UE starts TAT associated with TAG ID 0. Clarification on TS38.321 is needed. The TP in Annex 2 should be adopted.

- 3. Conclusion

In this paper, we discuss the remaining open issue with two TAs upon co-existence with CG-SDT. The observations and proposals are as follows,

Observation 1: For the case UE enters RRC Connected from inactive via RACH procedure, UE hasn’t restored the stored UE context when UE starts the TAT. Only one PTAG (TAG ID=0) is used for the UE. So no spec impact is introduced.

Observation 2: For the case where UE enters RRC connected from on-going CG-SDT, it is upon the reception of RRCResume which could configure two-TA for the PCell. Since two PTAGs are associated with the PCell, it is not clear which PTAG is associated with the applied TA based on current 38.321.

Proposal 1: CG-SDT should not be configured by RRCRelease if UE is configured with two TAs for PCell in RRC_CONNECTED. The TP in Annex 1 should be adopted.

Proposal 2: When UE transits from ongoing CG-SDT in RRC_INACTIVE to RRC CONNECTED, UE starts TAT associated with TAG ID 0. Clarification on TS38.321 is needed. The TP in Annex 2 should be adopted.

- 4. Reference

[1] R2-2401048	Considerations On Remaining Issues for 2TA	ZTE Corporation, Sanechips discussion

- Annex 1

–	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

omitted

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-DL-r17        BWP-DownlinkDedicatedSDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17         TimeAlignmentTimer                                            OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17          RSRP-Range                                                    OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17        SetupRelease { CG-SDT-TA-ValidationConfig-r17 }               OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                    RNTI-Value                                                    OPTIONAL,   -- Need M

...,

[[

cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-v1800

OPTIONAL,   -- Need N

cg-MT-SDT-MaxDurationToNext-CG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                 OPTIONAL    -- Need R

]]

}

CG-SDT-TA-ValidationConfig-r17 ::=  SEQUENCE {

cg-SDT-RSRP-ChangeThreshold-r17     ENUMERATED { dB2, dB4, dB6, dB8, dB10, dB14, dB18, dB22,

dB26, dB30, dB34, spare5, spare4, spare3, spare2, spare1}

}

BWP-DownlinkDedicatedSDT-r17 ::=    SEQUENCE {

pdcch-Config-r17                    SetupRelease { PDCCH-Config }                                       OPTIONAL,   -- Need M

pdsch-Config-r17                    SetupRelease { PDSCH-Config }                                       OPTIONAL,   -- Need M

...

}

BWP-UplinkDedicatedSDT-r17 ::=      SEQUENCE {

pusch-Config-r17                    SetupRelease { PUSCH-Config }                                       OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r17                 ConfiguredGrantConfigToAddModList-r16             OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r17                ConfiguredGrantConfigToReleaseList-r16            OPTIONAL,   -- Need N

...

}

CG-SDT-ConfigLCH-Restriction-r17 ::= SEQUENCE {

logicalChannelIdentity-r17          LogicalChannelIdentity,

configuredGrantType1Allowed-r17     ENUMERATED {true}                                                   OPTIONAL,   -- Need R

allowedCG-List-r17                  SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL    -- Need R

}

CG-SDT-ConfigLCH-Restriction-v1800 ::= SEQUENCE {

cg-SDT-MaxDurationToNext-CG-Occasion-r18 ENUMERATED {

ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600,

sec1200, sec1800, sec3600,

spare5, spare4, spare3, spare2, spare1}                    OPTIONAL    -- Need R

}

omitted

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

SDT-Config field descriptions

sdt-DRB-ContinueROHC Indicates whether the PDCP entity of the radio bearers configured for SDT continues or resets the ROHC header compression protocol during PDCP re-establishment during SDT procedure, as specified in TS 38.323 [5]. Value cell indicates that ROHC header compression continues when the UE resumes for SDT in the same cell as the PCell when the RRCRelease message was received. Value rna indicates that ROHC header compression continues when the UE resumes for SDT in a cell belonging to the same RNA as the PCell where the RRCRelease message was received. If the field is absent, the UE releases any stored value for this field and the PDCP entity of the radio bearers configured for SDT always resets the ROHC header compression protocol during PDCP re-establishment when SDT procedure is initiated, as specified in TS 38.323 [5].

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT.

sdt-MAC-PHY-CG-Config Indicates the CG-SDT specific configurations. This field is configured with setup only if the PCell is not configured with tag2.

sdt-SRB2-Indication Indiates whether SRB2 is configured for SDT or not.

- Annex 2

- 5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG;

-	inactivePosSRS-TimeAlignmentTimer which controls how long the MAC entity considers the Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned;

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned;

-	srs-ValidityAreaTimeAlignmentTimer which controls how long the MAC entity considers Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned when SRS positioning validity area is configured.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell configured with two TAGs or in a MSGB for an SpCell configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for the TAG indicated in the received Random Access Response message or MSGB;

3>	start or restart the timeAlignmentTimer associated with TAG indicated in the received Random Access Response message or MSGB.

2>	else if the timeAlignmentTimer associated with the TAG indicated in the received Random Access Response message or MSGB is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	stop the timeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell not configured with two TAGs or in a MSGB for an SpCell not configured with two TAGs:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

5>	set the NTA value to the value before applying the received Timing Advance Command as in TS 38.211 [8].

3>	when the Contention Resolution is considered successful for Random Access procedure while the CG-SDT procedure is ongoing:

4>	stop timeAlignmentTimer associated with this TAG;

4>	start or restart the cg-SDT-TimeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered successful for Random Access procedure while SRS transmission in RRC_INACTIVE is ongoing:

4>	if SRS positioning validity area is configured:

5>	start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG.

4>	else:

5>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell configured with two TAGs:

2>	apply the Timing Advance Command for the PTAG indicated in the Absolute Timing Advance Command MAC CE;

2>	start or restart the timeAlignmentTimer associated with this PTAG.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE, as specified in clause 5.1.4a, for an SpCell not configured with two TAGs:

2>	apply the Timing Advance Command for PTAG;

2>	if there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	if SRS positioning validity area is configured:

4>	start or restart the srs-ValidityAreaTimeAlignmentTimer associated with the indicated TAG.

3>	else:

4>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with PTAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with PTAG.

1>	when the MAC entity is configured with rach-LessHO:

2>	set the NTA value (as defined in TS 38.211 [8]) to the value indicated by targetNTA in rach-LessHO for PTAG;

2>	start the timeAlignmentTimer associated with PTAG.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer:

2>	stop the inactivePosSRS-TimeAlignmentTimer.

1>	when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer:

2>	start or restart the inactivePosSRS-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for starting the cg-SDT-TimeAlignmentTimer:

2>	start the cg-SDT-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for stopping the cg-SDT-TimeAlignmentTimer:

2>	consider the cg-SDT-TimeAlignmentTimer as expired.

1>	when the indication is received from upper layer for starting the srs-ValidityArea-TimerAlignmentTimer:

2>	start or restart the srs-ValidityArea-TimerAlignmentTimer.

1>	when the indication is received from upper layer for stopping the srs-ValidityArea-TimerAlignmentTimer:

2>	stop the srs-ValidityArea-TimerAlignmentTimer.

1>	when instruction from the upper layer has been received for starting the TimeAlignmentTimer associated with PTAG:

2>	start the TimeAlignmentTimer associated with PTAG with TAG ID 0.

1>	when an LTM Cell Switch Command MAC CE including a Timing Advance Command is received:

2>	apply the Timing Advance Command for the PTAG;

2>	start or restart the timeAlignmentTimer associated with the PTAG.

1>	when an LTM Cell Switch Command MAC CE is received and the UE has successfully measured the Timing Advance as in clause 5.18.35:

2>	apply the measured Timing Advance for the PTAG;

2>	start or restart the timeAlignmentTimer associated with the PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or

2>	if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else if the timeAlignmentTimer is associated with a TAG for an SCell, then for all SCells configured with only this TAG; or

2>	if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running, then for all such SCells:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

2>	else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, then for all such Serving Cells:

3>	clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer;

3>	clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer;

3>	clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

1>	when the inactivePosSRS-TimeAlignmentTimer expires:

2>	notify RRC to release Positioning SRS for RRC_INACTIVE configuration(s).

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell not configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

When the MAC entity stops uplink transmissions associated to a STAG for an SCell configured with two TAGs due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the STAG as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer(s) associated with all TAG(s) to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or srs-ValidityAreaTimeAlignmentTimer is not running. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.