3GPP TSG-RAN WG2 Meeting #125bis	R2-2402554

Changsha, China,  April 15th–19th, 2024

Agenda Item:	8.10.4

Source:	CMCC

Title:	Discussion on SON/MDT Enhancements for SDT and MHI

WID: 	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion

# 1 Introduction

In RAN#102 meeting, the new WI on Data collection for SON/MDT in NR standalone and MR-DC Phase 4 was approved [1] with the following objectives:

4.1 Objective of SI or Core part WI or Testing part WI The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

In this contribution, we would discuss on RACH optimization for SDT and MHI Enhancement for SCG Deactivation/ Activation in leftovers from Rel-18 and provide our proposals.

# 2 Discussion

2.1 RACH optimization for SDT

According to the discussion on RACH optimization in Rel-18 SON/MDT WI, the initial agreement “UE includes RA and SDT information in RA report when an SDT operation fails” has been achieved in RAN2 [2]. But due to time limit, the related work on RACH optimization for SDT was not completed in Rel-18 and delayed to Rel-19.

Agreements:

1	For RACH report for RACH partitioning, RAN2 to agree to include NSAG ID when the applicable feature is slicing.

2	 RACH report enhancements required for NE-DC are de-prioritized.

3	 For EN-DC and NG-EN-DC, the UE collects SN RA report container (for NR) and reports to the LTE MN. FFS on whether and which PSCell identity UE should report outside the RACH report.

4	UE includes RA and SDT information in RA report when an SDT operation fails.

Network configure a lot of conditions and parameters (e.g., RSRP Threshold, Data Volume Threshold, t319a, etc) to UE for SDT. SDT procedure can be initiated by UE only when these conditions are fulfilled and the valid SDT resource is available. But sometimes the configuration of the conditions and parameters of SDT are not improper, that may result in data transmission failure even if the conditions for SDT are fulfilled.

Observation 1: Sometimes the configuration of the conditions and parameters of SDT are not optimal, that may result in data transmission failure even if the conditions for SDT are fulfilled.

The aim of RACH optimization for SDT is to detect and optimize the non-optimal the configurations of the conditions and parameters of SDT based on the analysis of SDT failure information reported by UE. In particular, UE logs the SDT information in RRC_INACTIVE state in RA report when an SDT operation fails and reports it to network.

Proposal 1: For RACH optimization for SDT, UE logs the SDT information in RRC_INACTIVE state in RA report when an SDT operation fails and reports it to network.

From our point of view, the following information could be logged in the RA report when the SDT triggers an RA procedure, which helps network analysis the problems of SDT failure and optimize the related configuration to improve the performance of SDT.

- DL RSRP at the time of SDT initiation

- Data volume buffered at UE side upon SDT initiation

- Configured RSRP threshold and data volume threshold

- Configured value of T319a

Proposal 2: RAN2 discuss the SDT information logged in the RA report when the SDT triggers an RA procedure, e.g.,

- DL RSRP at the time of SDT initiation

- Data volume buffered at UE side upon SDT initiation

- Configured RSRP threshold and data volume threshold

- Configured value of T319a

2.2 MHI Enhancement for SCG Deactivation/Activation

To enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured, an activation/deactivation mechanism of SCG is supported in Rel-17. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

Upon SCG deactivation and while the SCG is deactivated, the network ensures that there is no uplink control PDU transmission to the deactivated SCG (e.g. the network releases statusReportRequired from PDCP entities of SCG bearers if configured, the network does not perform QoS flow remapping from a DRB associated to the deactivated SCG to another DRB).

While the SCG is deactivated, the UE will not transmit PUSCH, SRS and CSI report on SCG, and the UE is not required to monitor PDCCH or receive DL-SCH on SCG. If configured by the network, the UE performs radio link monitoring on the SCG and beam failure detection on the SCG while SCG is deactivated.

On the other hand, for MHI, according to the information that UE reported in MobilityHistoryReport, i.e., the visited cell id and time spent in the cell, the network could understand coverage of the wireless network and adjust the mobility related parameters, e.g., A3-offset, hysteresis, timeToTrigger. So that, the problem of ping pong handover or handover failure could be mitigated.

Similarity, for Dual Connectivity, the MHI of SCG is also supported in Rel-17, where the cell id of the PSCell and time spent in the current PSCell while being connected to the current PCell are included in the visitedPSCellInfoList of MHI. Based on the MHI of SCG, the MN/SN could understand the coverage of the SN, enable fast DC setup, and adjust SCG mobility parameters. So that the frequent ping pong PSCell change or PSCell change failure could be mitigated.

Therefore, it is observed that the current MHI of SCG has no information about SCG activation and deactivation, since only PSCell id and time spent in the PSCell are recorded by the UE.

Observation 2: No information about SCG activation or deactivation is recorded by the UE in current MHI.

In our understanding, including the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity. The detailed information could be the time of SCG activation, or percentage of time that SCG activation. For instance, if the time or percentage of time that SCG activation is quite low, it means that the Dual Connectivity is not so necessary for the UE, and the network could not configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. On the other hand, if the time or percentage of time that SCG activation is quite high, it means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficient.

Observation 3: Including the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Therefore, it is proposed that:

Proposal 3: Include in MHI the information of SCG activation/deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation.

# 3 Conclusion

In this paper, we discuss on RACH optimization for SDT and MHI Enhancement for SCG Deactivation/Activation in leftovers from Rel-18, and we have the following observations and proposals:

RACH optimization for SDT

Observation 1: Sometimes the configuration of the conditions and parameters of SDT are not optimal, that may result in data transmission failure even if the conditions for SDT are fulfilled.

Proposal 1: For RACH optimization for SDT, UE logs the SDT information in RRC_INACTIVE state in RA report when an SDT operation fails and reports it to network.

Proposal 2: RAN2 discuss the SDT information logged in the RA report when the SDT triggers an RA procedure, e.g.,

- DL RSRP at the time of SDT initiation

- Data volume buffered at UE side upon SDT initiation

- Configured RSRP threshold and data volume threshold

- Configured value of T319a

MHI Enhancement for SCG Deactivation/Activation

Observation 2: No information about SCG activation or deactivation is recorded by the UE in current MHI.

Observation 3: Including the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Proposal 3: Include in MHI the information of SCG activation/deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation.

# Reference

- RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, CMCC

- R2-2300002, Report of 3GPP TSG RAN WG2 meeting #120