3GPP TSG-RAN WG2 Meeting #125bis	R2-2402673

Changsha, China, April 15-19 2024

Agenda Item:	8.3.5

Source:	NEC

Title:	Simulation assumption and evaluation methodology

Document for:	Discussion, Decision

# Introduction

This paper intends to discuss the simulation assumptions and evaluation methodology for the study on AI/ML for mobility.

# Discussion

As described in the WID, simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839 and the detail discussion can leave to RAN2.

Following the guidance of the WID, basically, the start point of the simulation assumption and methodology for AI based mobility is already available, which is in TR36.839. It should be noted that TR36.839 was developed when the HetNet Mobility was studied in 3GPP for HetNet scenarios at Release-11.

## Basic Radio Configurations

In TR36.839, the basic radio configurations for the HetNet mobility simulation was introduced for LTE. In Rel-17, there was a study carried on by RAN1 on the study of XR (Extended Reality) evaluations for NR, with the result captured in TR38.838. We think the simulation assumption in TR38.838 can be a good reference for the basic radio configurations for AIML mobility evaluation.

The first thing that needs to clarify is the frequency for the simulation assumption and methodology. Should RAN2 focus on FR1 only or focus on both FR1 and FR2. We suggest to focus on FR1 for simplicity in the initial stage.

Proposal-1: RAN2 to focus on FR1 for the evaluation of AIML based mobility in the initial stage.

The second thing is which deployment scenario(s) RAN2 should focus on. In XR study, three deployment scenarios were studied: Dense Urban, Urban Macro and Indoor Hotspot. We think that Urban Macro is a typical scenario, for the purpose of AIML based mobility study, RAN2 should focus on Urban Macro scenario.

Proposal-2: RAN2 should focus on Urban Macro scenario for the evaluation of AIML based mobility.

The third one is which UE speed, RAN2 should assume for the study. As we can see, AIML mobility may benefit the scenario of high speed for UE move, we may need to cover both low speed and high speed scenarios.

Proposal-3: RAN2 to cover both low speed and high speed scenarios (e.g. 3km/hr, 30km/hr, 60km/hr and 120km/hr) for the evaluation of AIML based mobility.

An example of the simulation parameters can be listed with the table below (with reference to TR38.838):

| Parameter | Deployment scenarios |
| --- | --- |
|  | Dense Urban (38.913 w/ following parameters) |
| Layout | 21cells with wraparound ISD: 500m |
| Channel model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor Note: Other UE distribution can be evaluated optionally. |
| Carrier frequencyk | 4 GHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25m |
| UE height | 1.5m |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3km/hr, 30km/hr, 60km/hr and 120km/hr |
| MCS | Up to 256QAM |
| BS Antenna Pattern | 3-sector antenna radiation pattern, 8 dBi |
| BS Antenna Configuration | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) Note: Other BS antenna parameters can also be optionally evaluated. |
| UE Antenna Pattern | Omni-directional, 0 dBi |
| UE Antenna Configuration | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ Optional: 4T/4R, 1T/2R, 2T2R |
| Down Tilt | 12 degrees Note: Other downtilt values can also be optionally evaluated |
| BS Transmit Power | 49 dBm per 20 MHz |
| UE max tx power | 23dBm |
| System Bandwidth | Single Carrier (SC) evaluations only: - Baseline: 100 MHz - Optional: 20/40 MHz |

Proposal-4: RAN2 to use the simulation parameters in the table as a start point for the  evaluation of AIML based mobility.

## Handover Failure Model and Ping-pong Model

In TR36.839, the Handover Failure Modelling was introduced and the following definitions were captured:

- Definition of Handover states

- RLF modelling and definition of RLF states

- Handover/PDCCH failure modelling

The parameters for determine the RLFs and the PDCCH failures include:

- Qout

- Qin

- T310

- N310

- T311

- N311

We think that the same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

Proposal-5: The same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

In TR36.839, the Ping-pong Modelling was introduced. Within the TR, a ping-pong handover is defined if the UE’s time-of-stay connected in cell A is less than a pre-determined minimum-time-of-stay (MTS), when the handover was from cell B to cell A then the UE hands over back to cell B. Ping-pong rate is defined as (number of ping-pongs)/(total number of successful handovers excl. handover failures).

It is proposed that the same Ping-pong Modelling as described within TR36.839 can be reused for AIML based mobility.

Proposal-6: The same Ping-pong Modelling as described within TR36.839 can be reused for AIML based mobility.

Other than the Handover Failure Model and Ping-pong Model, more discussion may be needed to define the suitable model before simulation evaluation.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: RAN2 to focus on FR1 for the evaluation of AIML based mobility in the initial stage.

Proposal-2: RAN2 should focus on Urban Macro scenario for the evaluation of AIML based mobility.

Proposal-3: RAN2 to cover both low speed and high speed scenarios (e.g. 3km/hr, 30km/hr, 60km/hr and 120km/hr) for the evaluation of AIML based mobility.

Proposal-4: RAN2 to use the simulation parameters in the table (section 2.1) as a start point for the  evaluation of AIML based mobility.

Proposal-5: The same Handover Failure Modelling as described within TR36.839 can be reused for AIML based mobility.

Proposal-6: The same Ping-pong Modelling as described within TR36.839 can be reused for AIML based mobility.

# Reference

- RP-234055 (AI)Machine Learning (ML) for mobility

- TR36.839

- TR38.838