3GPP TSG-RAN WG2 Meeting #125bis	 R2-2402699

Changsha, China, April 15-19, 2023

Source:	KDDI Corporation

Title:	Considerations on RLC re-transmission related enhancements for XR

Tdoc Type:	Discussion

Agenda Item:	8.7.5 RLC enhancements

Document for: 	Discussion

Release:       	Rel-19

1	Introduction

In this contribution, we will share our views on the following objective, RLC re-transmission related enhancements [1].

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

2	Discussion

Avoid avid retransmitting in-flight old data

RLC-AM is useful to limit data loss, however RLC-AM feedback or retransmission triggering mechanisms are not well adapted for short packet delay budgets applicable to XR traffic. Also, for RLC AM, considerable amounts of data may be in-flight, i.e. in the window, and there is no current way to avoid retransmitting this data, even if the data is old.

The WID mentions in-flight old data, which does not need to be retransmitted in its justification part. In our understanding, this in-flight old data refers to the PDCP SDUs that are discarded when the PDCP discard timer expires. With short packet delay budgets, we believe that more discarded PDCP SDUs lead to more useless RLC retransmissions of RLC PDUs. Thus, we propose to discuss solutions to avoid this unnecessary retransmission.

Proposal 1: RAN2 agree to discuss mechanisms to avoid retransmitting RLC PDUs associated with the discarded PDCP SDUs.

Identification of discarded RLC SDUs

To avoid this unnecessary retransmission, first transmitting side of an AM RLC entity in UE needs to identify discarded RLC SDUs. Regarding this identification, we confirm that the current PDCP, RLC specifications specify that PDCP indicates the discard to RLC. Also, the current specification does not allow to introduce an RLC SN gap. However, if we consider discarding all in-flight old data, all discarded PDCP SDUs, it will result in RLC SN gap(s).

TS38.323

5.3 SDU discard When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

TS38.322

5.4 SDU discard procedures When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

How to introduce RLC SN gap(s)

To consider introducing RLC SN gaps, first we must discuss whether the transmitting side notifies the RLC sequence numbers of discarded RLC PDUs to the receiving side. In our view, this indication of RLC sequence numbers is mandatory for the case where transmitting side will not retransmit the discarded RLC PDUs. Without this indication, the receiving side will continue to request the retransmission of the discarded RLC PDUs forever, nevertheless the transmitting will not retransmit the discarded RLC PDUs.

Proposal 2: RAN2 agree to introduce the indication of the RLC sequence numbers of discarded RLC PDUs, the indication is from the transmitting side to the receiving side.

The next discussion will be the behaviour of the receiving side after the indication. In our view, it is better to have the receiving side refrain from requesting the retransmission of discarded RLC PDUs to the transmitting side and consider the discarded RLC PDUs as if they were received correctly. It means that the receiving side of an AM RLC entity will not include NACK_SN information of discarded RLC PDUs and include ACK_SN information of discarded RLC PDUs instead, in other word, this STATUS PDU will not request the transmission of discarded RLC PDUs.

Proposal 3: RAN2 agree that after the SN indication of discarded RLC PDUs, the reviving side AM entity will not include NACK_SN information of discarded RLC PDUs in STATUS PDU, will include ACK_SN information of discarded RLC PDUs in STATUS PDU instead.

Final discussion is the behaviour of the transmitting side after the SN indication. The transmitting side will receive the STATUS PDU including ACK_SN information of discarded RLC PDUs. At this point the transmitting side can treat discarded RLC PDUs as if those were received correctly at the receiving side. Furthermore, the transmitting side will not receive NACK_SN information of discarded RLC PDUs, thus no more retransmission for the discarded RLC PDUS is triggered.

Proposal 4: RAN2 agree that after the SN indication of discarded RLC PDUs, when the transmitting side AM entity receive the STASU PDU including ACK_SN information of discarded RLC PDUs from the receiving side, then the transmitting side treat the discarded RLC PDUs as if those were received correctly at the receiving side.

3	Conclusion

We make the following proposals:

Proposal 1: RAN2 agree to discuss mechanisms to avoid retransmitting RLC PDUs associated with the discarded PDCP SDUs.

Proposal 2: RAN2 agree to introduce the indication of the RLC sequence numbers of discarded RLC PDUs, the indication is from the transmitting side to the receiving side.

Proposal 3: RAN2 agree that after the SN indication of discarded RLC PDUs, the reviving side AM entity will not include NACK_SN information of discarded RLC PDUs in STATUS PDU, will include ACK_SN information of discarded RLC PDUs in STATUS PDU instead.

Proposal 4: RAN2 agree that after the SN indication of discarded RLC PDUs, when the transmitting side AM entity receive the STASU PDU including ACK_SN information of discarded RLC PDUs from the receiving side, then the transmitting side treat the discarded RLC PDUs as if those were received correctly at the receiving side.

4	　Reference

[1]　 RP-240791	Revised WID on XR (eXtended Reality) for NR Phase 3		Nokia (Rapporteur)