3GPP TSG-RAN WG2 Meeting #125bis	R2-2402711

# Changsha, China, 15th of April – 19th of April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1796 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating RO Mask index for multiple PRACH repetitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The work done in Coverage Enhancements for Multiple PRACH Transmissions includes the use of RO sets for the transmission of more than one Msg1 preamble. The existing parameter ra-ssb-OccasionMaskIndex points to a table which is not updated to mention these sets, it only mentions RO (RACH Occasions). Furthermore, RO sets can be more or fewer than ROs and thus the existing mapping suffers from legacy restrictions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new table for the purpose of RO sets which is only applicable to Msg1 reptitions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network can communicate a mask index that allows one RO and disallows another RO, but the UE might have an RO set that includes both these ROs. Hence, there will be ambiguities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 7.4	PRACH Mask Index values

Table 7.4-1: PRACH Mask Index values without Msg1 repetitions

| PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex | Allowed PRACH occasion(s) of SSB |
| --- | --- |
| 0 | All |
| 1 | PRACH occasion index 1 |
| 2 | PRACH occasion index 2 |
| 3 | PRACH occasion index 3 |
| 4 | PRACH occasion index 4 |
| 5 | PRACH occasion index 5 |
| 6 | PRACH occasion index 6 |
| 7 | PRACH occasion index 7 |
| 8 | PRACH occasion index 8 |
| 9 | Every even PRACH occasion |
| 10 | Every odd PRACH occasion |
| 11 | Reserved |
| 12 | Reserved |
| 13 | Reserved |
| 14 | Reserved |
| 15 | Reserved |

Table 7.4-X: PRACH Mask Index values for Msg1 repetitions

| PRACH Mask Index/ | Allowed set(s) of ROs of SSB |
| --- | --- |
| 0 (0000) | Reserved |
| 1 (0001) | Every 2nd RO set starting with even RO set |
| 2 (0010) | Every 3rd RO set starting with even RO set |
| 3 (0011) | Every 4th RO set starting with even RO set |
| 4 (0100) | Every 5th RO set starting with even RO set |
| 5 (0101) | Every 6th RO set starting with even RO set |
| 6 (0110) | Every 7th RO set starting with even RO set |
| 7 (0111) | Every 8th RO set starting with even RO set |
| 8 (1000) | Every 2nd RO set starting with odd RO set |
| 9 (1001) | Every 3rd RO set starting with odd RO set |
| 10 (1010) | Every 4th RO set starting with odd RO set |
| 11 (1011) | Every 5th RO set starting with odd RO set |
| 12 (1100) | Every 6th RO set starting with odd RO set |
| 13 (1101) | Every 7th RO set starting with odd RO set |
| 14 (1110) | Every 8th RO set starting with odd RO set |
| 15 (1111) | All |

END OF CHANGE