3GPP TSG-RAN WG2 Meeting #125bis	R2-2402779

Changsha, China, 15th April – 19th April, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on capturing RAN1 agreements regarding repetitions for cell DTX/DRX in the MAC specification

Agenda Item:	7.3.2

Document for:	Discussion and decision

# Introduction

In the RAN1#116 meeting, RAN1 has sent an LS to RAN2 and asked RAN2 to capture the agreements made by RAN1 related to repetitions during the cell DTX/DRX operation in RAN2 specifications [1]:

RAN1 would also like to ask RAN2 to consider capturing the following RAN1 agreements on handling of transmission and reception of channels during cell DTX/DRX operation in appropriate RAN2 specifications.

- From RAN1 #115

  - UE transmits a subset of the repetitions in a CG bundle that do not overlap with the cell DRX non-active period.

- From RAN1 #116

  - UE transmit a subset of the repetitions of a PUCCH with SR and/or P/SP-CSI that do not overlap with the cell DRX non-active period.

  - UE receives a subset of the repetitions of a SPS PDSCH that do not overlap with the cell DTX non-active period.

In this contribution, we will discuss how to capture the above agreements in RAN2 specifications and provide the corresponding TP.

# Discussion

The transmission and reception of signals/channels impacted by cell DTX/DRX operation have been discussed in previous meetings and specified in TS 38.321. The SPS/CG/SR transmission and/or P/SP-CSI report behaviours have been captured as follows [2]:

Cite from TS 38.321 For each Serving Cell configured with cell DTX, the MAC entity need not: 1> if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period: 2> monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause; 2> instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS; 2> indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity; 2> set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment; 2> consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment. ------Unrelated text omitted------------ 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> not instruct the physical layer to signal a SR on a PUCCH resource for SR; 2> not increment the SR_COUNTER for a SR; 2> not start the sr-ProhibitTimer for a SR; 2> not deliver any configured uplink grant and the associated HARQ information to the HARQ entity; 2> not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission; 2> not report CSI on PUCCH and semi-persistent CSI configured on PUSCH; 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1).

The subset of repetition means that the whole transmission/reception can be divided into different subsets. In some cases, the transmission/reception of signal/channel may not fully fall into the cell DTX/DRX active period, which means a subset of repetitions may fall into the cell DTX/DRX non-active period. From RAN1’s agreement, we can conclude that it is allowed to transmit a subset of CG, SR and/or P/SP-CSI during cell DRX active period and receive a subset of SPS PDSCH overlapping with the cell DTX active period. On the other hand, the subset which overlaps with the cell DTX/DRX non-active period should not be transmitted/received.

Since RAN2 has already captured SPS/CG/SR/P/SP-CSI reception and transmission behaviour during cell DTX/DRX operation, such subset of the above signals/channels has been implicitly included in the description. There is no need to add more procedural text related to a subset repetition of the signals/channels. To clarify the behaviour for repetitions a note can be added at the bottom of the corresponding procedure to cover such cases.

Therefore, we propose the add a note in the current spec to capture the RAN1 agreements separately for cell DTX and cell DRX operation.

- Add a note in MAC to capture RAN1 agreements from R1-2401810. Adopt the TP in the Annex.

# Conclusion

In this contribution, we discussed how to capture the subset of repetitions transmission/reception behaviour, and made the following proposals:

- Add a note in MAC to capture RAN1 agreements from R1-2401810. Adopt the TP in the Annex.

# References

- R1-2401810 LS on Network Energy Savings

- R2-2402025 Miscellaneous MAC corrections for network energy savings, InterDigital

# Annex: TP for 38.321

<<<<<<<<<<<<<<<<<<<<<<<<<<<<unchanged text omitted>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 5.34	Cell-Level Energy Saving

### 5.34.1	General

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation affects UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTXDRX-Config per Serving Cell:

-	cellDTXDRXconfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	celldtxdrx-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	celldtxdrx-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	celldtxdrx-SlotOffset: the delay before starting the celldtxdrx-onDurationTimer;

-	celldtxdrx-Cycle: the cell DTX/DRX cycle period.

-	cellDTXDRXactivationStatus: the initial activation status of cell DTX and cell DRX operation.

### 5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTXDRXconfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DTX is configured and cellDTXDRXactivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if CellDTXDRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

NOTE:	UE does not receive the subset of the repetitions in a configured downlink assignment for SPS during cell DTX non-active period.

### 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE 1:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

NOTE 2:	UE does not transmit the subset of the repetitions in a configured uplink grant bundle, SR and/or CSI/semi-persistent CSI report on a PUCCH resource during cell DRX non-active period.