3GPP TSG-RAN WG2 Meeting #125bis	R2-2402800

Changsha, China, April 15th – 19th, 2024

Agenda item:	7.7.3

Source:	Samsung

Title:	RIL S486, V500, V501, H063

WID/SID:	NR_NTN_enh -Core

Document for:	Discussion and Decision

# Introduction

In this contribution, we share our views on some open RILs in ASN.1 review.

# Discussion

## S486

The RIL was raised as the current TA-Report field description states when the field “is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync”. However, it was agreed in RAN2#123 that the TAR MAC CE may be sent during RACH-less:

- ta-Report can be included in ServingCellConfigCommon in the RACH-less HO command

Thus, there needs to be clarification in the field description that ta-Report is also valid for RACH-less handovers.

Proposal 1: Accept S486 – clarify in ta-Report field description that it also applies to RACH-less handovers.

## V500, V501

We find it is already clear UE uses the target satellite epoch time and validity when performing the satellite switch procedure, as how it is specified below in clause 5.7.19. In our view, no other clarification is needed.

##### 5.2.2.4.21 Actions upon reception of SIB19 Upon receiving SIB19 in an NTN cell, the UE in RRC_CONNECTED shall: 1> start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell; 1> if SatSwitchWithReSync and t-Service are included, and the UE supports hard satellite switch with resynchronization; 2> if t-ServiceStart is included and the UE supports soft satellite switch with resynchronization: 3> start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell. 2> perform the satellite switch with resynchronization as specified in 5.7.19 at the time indicated by t-Service for the serving cell. NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation. ### 5.7.19 Satellite switch with resynchronization in RRC_CONNECTED The UE shall: 1> stop timer T430 if running; 1> inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization; 1> start synchronising to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if the UE has not acquired DL synchronization; 1> start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync; 1> inform lower layers when UL synchronisation is obtained.

Observation 1: It is clear UE uses the target satellite epoch time and validity when performing the satellite switch procedure as specified in clause 5.7.19.

Proposal 2: V500 and V501 are not needed.

## H063

For satellite switch with resync, NW-enabled UE-initiated RACH was proposed and discussed in the last meeting, as below.

R2-2400195 [H063] RACH-based satellite switching with re-sync Huawei, HiSilicon, CATT, CMCC discussion Rel-18 NR_NTN_enh-Core Proposal 1: The NW can inform UE whether it can initiate RACH autonomously during satellite switch with re-sync. - LG thinks that allowing RACH would cause RACH congestion at t-service - CATT supports this and thinks we cannot always rely on PDCCH order - Google supports this - Nokia also supports - Samsung does not support this and thinks we already decided not to have it - Ericsson thinks that RACH-less switch in satellite switch with resync is an integral part of the feature - Inmarsat and SES think this is not needed - CB next meeting to check whether to have a NW indication that UEs in RRC Connected mode need to perform RACH during satellite switch with re-sync

In our view, UE-initiated RACH for satellite switch resync is not necessary, as what we have already discussed and agreed in RAN2#124.

R2-2313785 Summary of [AT124][303][NR-NTN Enh] Unchanged PCI Apple discussion Proposal 2: Discuss whether to support RACH-based procedure. - IDC thinks would not work - Apple thinks there is no need to support this. - Google thinks we cannot exclude the possibility to trigger PDCCH order We don’t introduce specific changes (e.g. no new indication in SIB19) to a support RACH-based procedure but this does not exclude the possibility for the NW to trigger PDCCH order

There are some concerns about the current RACH-less satellite switch procedure. One concern is about the UE capability that RACH-less HO is an optional UE capability so UE may not be capable of satellite switch with RACH-less procedure. However, in our view RACH-less HO and satellite switch are two different capabilities. This is addressed by the following agreement in the last meeting.

Agreement RAN2#125:

- All UEs supporting satellite switch with resync shall be able to perform satellite switch with re-sync without RACH (this does not mean that a UE supporting satellite switch with resync needs to support RACH-less HO)

Observation 2: UE have separate capabilities to support RACH-less HO and satellite switch with resync without RACH. Supporting one does not mean the other one is always supported.

The second concern is that PDCCH order RACH may not be sufficient and NW should enable UE to perform UE initiated RACH for satellite switch with resync. In our view, UE initiated RACH by NW enabling cannot work well. Since the configuration of satellite switch with resync is provided in SIB19 and AS security activation is not needed (discussed in last meeting), NW cannot enable UE initiated RACH dedicatedly. If such enabling configuration is given in common signalling, all UEs in the cell would initiated RACH at the same time, causing RACH congestion.

Observation 3: For satellite switch with resync, UE initiated RACH by NW enabling cannot work well since NW cannot enable UE-initiated RACH dedicatedly if AS security is not activated and such enabling configuration in common signalling causes RACH congestion.

Furthermore, we think PDCCH order RACH for satellite switch with resync should be sufficient for satellite switch with resync. For different UEs,

- if there is DL/UL data for UE, NW can send PDCCH order CBRA/CFRA when TAT is expired to recover UL synchronization (same as legacy PDCCH order RACH), or

- if immediate resynchronization is needed, NW can send PDCCH order CFRA for the purpose of resynchronization in satellite switch even there is no DL/UL data or TAT is still running (i.e., satellite switch with resync can be a new trigger for PDCCH order CFRA).

In this way, RACH is under control of NW request, no RACH congestion is expected.

Also, there should be no issue for UE to monitor PDCCH from the target satellite. As UE applies PDCCH configuration which contains searching space, coreset, QCL-info, etc. for the same serving cell and NW should provide workable configuration for the serving cell anyway.

Proposal 3: RAN2 confirms for satellite switch with resync NW can send PDCCH order CFAR/CBRA as legacy trigger (i.e., DL/UL date arrives but TAT is expired).

Proposal 4: RAN2 discuss if NW can send PDCCH order CFRA for the purpose of resynchronization in satellite switch even there is no DL/UL data or TAT is still running (i.e., satellite switch with resync become a new trigger for PDCCH order CFRA).

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: Accept S486 – clarify in ta-Report field description that it also applies to RACH-less handovers.

Observation 1: It is clear UE uses the target satellite epoch time and validity when performing the satellite switch procedure as specified in clause 5.7.19.

Proposal 2: V500 and V501 are not needed.

Observation 2: UE have separate capabilities to support RACH-less HO and satellite switch with resync without RACH. Supporting one does not mean the other one is always supported.

Observation 3: For satellite switch with resync, UE initiated RACH by NW enabling cannot work well since NW cannot enable UE-initiated RACH dedicatedly if AS security is not activated and such enabling configuration in common signalling causes RACH congestion.

Proposal 3: RAN2 confirms for satellite switch with resync NW can send PDCCH order CFAR/CBRA as legacy trigger (i.e., DL/UL date arrives but TAT is expired).

Proposal 4: RAN2 discuss if NW can send PDCCH order CFRA for the purpose of resynchronization in satellite switch even there is no DL/UL data or TAT is still running (i.e., satellite switch with resync becomes a new trigger for PDCCH order CFRA).

# References

[1] R2-2313563 Report from Break-out session on NR-NTN and IoT-NTN, 3GPP TSG-RAN WG2 Meeting #124

[2] R2-2401543 Report from Break-out session on NR-NTN and IoT-NTN, 3GPP TSG-RAN WG2 Meeting #125