3GPP TSG-RAN WG2 Meeting #125bis	R2-2402804

Changsha, China, April 15th – 19th, 2024

Agenda item:	7.20.3

Source:	Samsung, Xiaomi, Ericsson

Title:	RIL S958, S959 on codebook CBSR

WID/SID:	NR_MIMO_evo_DL_UL-Core

Document for:	Discussion and Decision

# Introduction

In the paper, we discuss RIL S958, S959 regarding the parameter n1-n2-codeBookSubsetRestrictionList for Rel-18 codebook-Config.

# Discussion

For Rel-18 codebook, n1-n2-codebookSubsetRestrictionList is used to indicate two parameters, n1-n2 and codebookSubsetRestriction (CBSR), for NTRP CSI-RS resources, where NTRP can be up to 4. As RAN1 indicated below, at least one element must be configured to indicate n1-n2 since it is a mandatory parameter applied for all CSI-RS resources, while CBSR is optional.

n1-n2-codebookSubsetRestrictionList-r18   SEQUENCE (SIZE (1..4)) OF CBSR-r18,

CBSR-r18 ::=    CHOICE {

two-one                                BIT STRING (SIZE (8)),

two-two                                BIT STRING (SIZE (27)),

four-one                               BIT STRING (SIZE (16)),

three-two                              BIT STRING (SIZE (35)),

six-one                                BIT STRING (SIZE (24)),

four-two                               BIT STRING (SIZE (43)),

eight-one                              BIT STRING (SIZE (32)),

four-three                             BIT STRING (SIZE (59)),

six-two                                BIT STRING (SIZE (59)),

twelve-one                             BIT STRING (SIZE (48)),

four-four                              BIT STRING (SIZE (75)),

eight-two                              BIT STRING (SIZE (75)),

sixteen-one                            BIT STRING (SIZE (64))

}

In the current ASN.1, n1-n2 and CBSR are signaled together as above, where n1-n2 is indicated as a choice of different sizes of CBSR bitmaps. Even if no CBSR restriction is needed for any CSI-SR resource, NW anyway has to signal one bitmap with all bits setting to 1 to indicate n1-n2. The bitmap length can be up to 75 bits, which causes unnecessary overhead.

In Rel-18, RAN1 has indicated the intention of separating n1-n2 from CBSR. For Rel-19, the same parameter is needed for codebook enhancement, and RAN1 has discussed the same intention in the last meeting. See RAN1#116 Chair notes (FFS: Whether to refine CBSR design to reduce RRC overhead).

To reduce signaling redundancy, it would be better to revise the ASN.1 structure sooner than later by RAN2. Here we discuss feasible ways to separately signal the two parameters.

As the CBSR bitmap length is a function of n1-n2, we can either specify in the field description the mapping between n1-n2 and the bitmap length or signal a parameter for the bitmap length.

- Option 1:

n1-n2-r18		ENUMERATED { two-one, two-two, four-one, three-two, six-one, four-two, eight-one, four-three, six-two, twelve-one, four-four, eight-two, sixteen-one },

codebookSubsetRestrictionList-r18		SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (INTEGER (8,16,24,27,32,35,43,48,59,64,75))) OPTIONAL    -- Need R

codebookSubsetRestrictionList List of bitmaps for codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.8). The number of elements is up to the number of elements of nzp-CSI-RS-Resources in NZP-CSI-RS-ResourceSet(S) indicated by nzp-CSI-RS-ResourceSetList in the CSI-ResourceConfig indicated by resourcesForChannelMeasurement in the CSI-ReportConfig in which the CodebookConfig is included. An element in the list corresponds to the element at the same position in nzp-CSI-RS-Resources. The bitmap length is same for all elements and calculated by CEIL(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!). For a given n1-n2-r18, a corresponding bitmap can be configured with the length as follows: two-one: 8; two-two: 27; four-one: 16; three-two: 35; six-one: 24; four-two: 43; eight-one: 32; four-three: 59: six-two: 59; twelve-one: 48; four-four: 75; eight-two: 75; sixteen-one: 64.

n1-n2-codebookSubsetRestriction Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.3). Number of bits for codebook subset restriction is CEIL(log2(nchoosek(O1*O2,4)))+8*n1*n2 where nchoosek(a,b) = a!/(b!(a-b)!).

n1-n2 Number of antenna ports in first (n1) and second (n2) dimension and codebook subset restriction (see TS 38.214 [19] clause 5.2.2.2.1, 5.2.2.2.8).

- Option 2:

n1-n2-r18		ENUMERATED { two-one, two-two, four-one, three-two, six-one, four-two, eight-one, four-three, six-two, twelve-one, four-four, eight-two, sixteen-one },

codebookSubsetRestrictionList-r18		SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (CBSR-BitStringLength)) OPTIONAL    -- Need R

CBSR-BitStringLength ::=  INTEGER (8,16,24,27,32,35,43,48,59,64,75)

Option 2 requires an additional parameter to indicate the length of each CBSR bitmap. Option 1 specifies the bitmap length explicitly in field description and does not require additional parameter, thus, it is preferred.

In the last meeting RIL H070 was discussed regarding how to indicate the mapping between a CBSR bitmap in the CBSR list and a CSI-RS resource in the CSI-RS resource list. The conclusion is to use the position correspondence between the elements of two lists, as below.

However, this agreement was not captured in the final CR R2-2402024. To capture this, we include a sentence describing the position correspondence between the elements in CBSR list and CSI-RS resource list, as in Option 1.

Proposal 1: Separate n1-n2 and CBSR for Rel-18 codebook-Config, and adopt option 1 (specify CBSR bitmap length in field description) as baseline.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: Separate n1-n2 and CBSR for Rel-18 codebook-Config, and adopt option 1 (specify CBSR bitmap length in field description) as baseline.

# References

[1] 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification (Release 18), version 18.1.0.