3GPP TSG-RAN WG2 Meeting #125bis                                    	R2-2402811

Changsha, China, April 15th – 19th, 2024

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	Discussion on EDT enhancements

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

- Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

It is to note that the scope of this objective is to have none to minimal core network impact with full eNB on board. In this document, we provide some challenges related to the solutions, which need to be discussed.

- Discussion

First objective

The first objective of the EDT enhancement is mainly enhancement of the random-access procedure as existing preamble transmission and RAR reception is not efficient, i.e., collision is detected by UE only after reception of Msg4 and restarting whole RACH procedure is too costly in NTN due to very long RTT. The example of enhancement includes Msg3 transmission without msg1/ Random Access Response (RAR). This enhancement is even more relevant in NTN, since the UEs are GNSS capable (i.e., they know their own location before accessing the NTN cell), and they know the satellite’s location (from the ephemeris information broadcasted in SIB). As a result, these NTN UEs have a valid timing advance from these two quantities (UE’s location and satellite’s location), and a PRACH preamble transmission is not strictly required.

Currently EDT is a four-step random access procedure, i.e., EDT specific Msg1, RAR, RRCEarlyDataRequest message (Msg3) and RRCEarlyDataComplete message (Msg4). In our understanding the objective is to reduce the EDT procedure from 4 steps to 2 steps, i.e., UE directly transmitting RRCEarlyDataRequest message (Msg3) and receiving RRCEarlyDataComplete message (Msg4). And this is without Msg1. i.e., preamble, therefore, it is not 2 step RACH procedure.

Figure 1 An example of EDT without Msg1 and RAR

As shown in figure 1, this can be interpreted as combination of EDT and PUR. However, PUR configuration is UE specific and will not work once a satellite moves out and a new satellite comes in the service. Therefore, it could be considered as RACH-less EDT and a common PUSCH resource can be provided via SIB. The collision can still be handled in the same existing way, i.e., by the use of contention resolution timer and contention resolution MAC CE in Msg4.

For the first objective, the enhancement of Msg3 (RRC connection request/UL data) transmission over contention-based common PUSCH resource, i.e., RACH-less EDT is considered.

For the first objective, confirm that, for Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Contention Resolution Diversity Slotted Aloha (CRDSA)

Current random-access procedures in 3GPP can be seen as a form of slotted Aloha: if two users choose the same preamble (or NPUSCH resource, for the objective under discussion), at most one of the UE’s Msg3 (PUSCH) will be successful and, in most cases, both UE’s Msg3 will fail the random-access attempt due to a collision. If we want to design a system targeting a small probability of collision, the random-access resources need to be dimensioned with a very large margin - for example, for a probability of collision of 1%, the random-access resources need to be approximately 100x larger than the expected arrival rate. Equivalently, for a probability of collision of 1%, 99% of the resources will remain unused.

Modern satellite communication systems incorporate improved random-access techniques that allow for a more efficient utilization of the random-access resources while keeping a low probability of collisions/errors. For instance, DVB-RCS2 incorporates Contention Resolution Diversity Slotted Aloha (CRDSA) [2, Section 7.2.5.2.2], and S-band Mobile Interactive Multimedia (S-MIM) incorporates Spread Spectrum Aloha (SSA) [3]. These techniques outperform basic slotted Aloha, e.g., in terms of supporting a larger traffic arrival rate at a given collision probability, as shown in [4-6]. This directly translates to improved system capacity (in terms of the number of UEs that can simultaneously be supported in the uplink over a given set of resources). In NTN—especially NB-IoT over NTN, where spectrum (frequency resources) is scarce and expensive, such techniques are pivotal for an efficient design (and specification) of “RACH-less EDT” (Msg3 transmission without msg1).

Observation 1: Enhanced random-access techniques such as CRDSA would better utilize available uplink resources and improve uplink capacity, vis-à-vis basic slotted ALOHA-based access techniques.

These enhanced random-access techniques should be considered an integral component of the design and specification for “Msg3 transmission without msg1”. Therefore, RAN2 should consider introducing some enhancements to the random-access procedure such that 3GPP technology remains competitive with state-of-the-art satellite standards.

For the first objective, RAN2 to study enhanced random access techniques, such as Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of msg3 transmission without NPRACH.

Second objective

The second objective of the EDT enhancement is to reduce the congestion/overhead for transmission of Msg4 to several UEs. For example, may UEs may initiate EDT and network may exhaust with PDCCH or PDSCH resources scheduling Msg4 to each individual UE. There may be cases where network may not have any downlink response data and network would need to schedule Msg4 just including RRCEarlyDataComplete message (without DL NAS PDU) and contention resolution MAC CE of multiple UEs. Such procedure already exists in case of Random-Access response. As shown in figure 2, network can schedule multiple RARs in a single MAC PDU.

Figure 2 TS 36.321 Figure 6.1.5-4: Example of MAC PDU consisting of a MAC header and MAC RARs

In this case there could be several possible options for the enhancements, some examples being:

- Reusing a PUR-like L1 ACK procedure

  - The L1 ACK may also be used to resolve contention between UEs

  - The L1 ACK can move UE back to RRC_IDLE

- Reusing concept of RAR multiplexing multiple users RAPIDs

  - A single PDU can multiplex contention resolution MAC CEs (or Msg4) of more than one UE.

- Others such as existing Multi-TB scheduling for Msg4 could also be studied for multi-user scenario.

The second objective should be primarily on the top of existing 4 steps EDT procedure as shown in figure 3, however, it could be applicable for the case of RACH-less EDT as described for the first objective.

For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

For the second objective, multicast Msg4 (multi-user Msg4 multiplexing) or multiple Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Figure 3 Msg4 enhancement for EDT

- Conclusion

Following proposals are made:

Proposal 1	For the first objective, the enhancement of Msg3 (RRC connection request/UL data) transmission over contention-based common PUSCH resource, i.e., RACH-less EDT is considered.

Proposal 2	For the first objective, confirm that, for Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 3	For the first objective, RAN2 to study enhanced random access techniques, such as Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of msg3 transmission without NPRACH.

Proposal 4	For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 5	For the second objective, multicast Msg4 (multi-user Msg4 multiplexing) or multiple Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] ETSI EN 301 545-2, “Digital Video Broadcasting (DVB); Second Generation DVB Interactive Satellite System (DVB-RCS2); Part 2: Lower Layers for Satellite standard”

[3] ETSI TS 102 721-3, “Satellite Earth Stations and Systems (SES); Air Interface for S-band Mobile Interactive Multimedia (S-MIM); Part 3: Physical Layer Specification, Return Link Asynchronous Access”

[4] S. Scalise, C. P. Niebla, R. De Gaudenzi, O. Del Rio Herrero, D. Finocchiaro and A. Arcidiacono, "S-MIM: a novel radio interface for efficient messaging services over satellite," in IEEE Communications Magazine, vol. 51, no. 3, pp. 119-125, March 2013

[5] E. Casini, R. De Gaudenzi and O. Del Rio Herrero, "Contention Resolution Diversity Slotted ALOHA (CRDSA): An Enhanced Random Access Schemefor Satellite Access Packet Networks," in IEEE Transactions on Wireless Communications, vol. 6, no. 4, pp. 1408-1419, April 2007

[6] De Gaudenzi, R., Del Rio Herrero, O., Gallinaro, G., Cioni, S., and Arapoglou, P.-D. (2018) Random access schemes for satellite networks, from VSAT to M2M: a survey. Int. J. Satell. Commun. Network., 36: 66–107.