3GPP TSG-RAN WG2 Meeting #125bis	R2-2402823

Changsha, China, 15 – 19 April, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

It has been agreed to specify on-demand SSB for intra/inter-band CA for R19 NES. The corresponding objective is given as follows [1]:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we provide our considerations on on-demand SSB SCell operation.

Discussion

2.1 Scenarios

RAN1 has achieved some preliminary agreements on the scenarios for on-demand SSB:

Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

- Before SCell activation: The motivation for on-demand SSB is clear, if no always-on SSB is transmitted before triggering on-demand SSB transmission, the on-demand SSB transmission can be used for L3 measurement reporting, based on the report the network can decide whether to activate the SCell or not to avoid blind activation. If SSB with long periodicity is transmitted before triggering on-demand SSB transmission with a short periodicity, the on-demand SSB transmission with a short periodicity can be used for fast L1/L3 measurements, AGC setting, DL synchronisation before SCell activation. The use case for triggering on-demand SSB could be the PCell cannot satisfy the traffic demand of the UE and an SCell on which no always-on SSB is transmitted needs to be activated, and it is up to network implementation to trigger this procedure based on the buffer status.

- During SCell activation: The motivation for on-demand SSB is also clear, if no always-on SSB is transmitted before triggering on-demand SSB transmission, the on-demand SSB transmission can be used for cell search, L1/L3 measurements, AGC setting, DL synchronisation. If SSB with long periodicity is transmitted before triggering on-demand SSB transmission with a short periodicity, the on-demand SSB transmission with a short periodicity can be used for fast L1/L3 measurements, AGC setting, DL synchronisation during SCell activation. The on-demand SSB can be triggered when SCell activation MAC CE is sent.

- After SCell activation: The motivation for on-demand SSB after SCell activation needs to be further investigated. In legacy mechanism, after SCell activation, SSB is transmitted with a proper periodicity for DL synchronization tracking and L1/L3 measurements. If on-demand SSB is applied to this phase, it needs to be figured out under which cases the SSB can be muted and under which condition the on-demand SSB should be triggered. Since RAN1 is having parallel discussion on this, we can wait for further RAN1 progress on this scenario.

Proposal 1: RAN2 starts discussion from Scenario #2 (before SCell activation when SCell is configured) and Scenario #3 (during SCell activation). The FFS part of Scenario #3 (after SCell activation) waits for further RAN1 progress.

There are two related issues on scenario clarification:

- Issue 1: whether on-demand SSB operation applies to CD-SSB or NCD-SSB

- Issue 2: whether to consider combining the R15/R18 SSB-less and on-demand SSB

On Issue 1, we think there is no need to restrict the SSB type. There could be concerns on impacting legacy UEs, for instance, if an NCD-SSB serves as SCell for a R19 NES UE while serving as PCell for a legacy UE, the on-demand operation may affect the legacy UEs as well. However, there are some implementation methods to avoid such case, for instance, handover can be used to offload RRC_CONNECTED UEs to other cells and cell bar can be used to offload RRC_IDLE/INACTVE UEs to other cells, or TAC in SIB1 can be absent so that this cell cannot serve as PCell (it is captured in the field description of trackingAreaCode that “The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN) or is an NTN cell.”).

On Issue 2, it has been agreed in RAN1 #116 that “RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.” The CA scenarios include both the intra/inter-band CA satisfying SSB-less operation requirements and the CA scenarios not satisfying the SSB-less operation requirements. Actually the CA scenarios not satisfying the SSB-less operation requirements would achieve more benefits from the on-demand SSB operation, while the CA scenarios satisfying the SSB-less operation requirements can simply omit the SSB transmission on the corresponding SCell and only benefit from on-demand SSB in corner cases like when the reference cell is deactivated (it has been agreed by RAN4 that the reference cell should be activated [2]).

Proposal 2: It is up to network implementation to apply on-demand SSB operation to CD-SSB or NCD-SSB.

2.2 Configuration and signaling

In the WID, three possible triggering method are listed for consideration, UE UL WUS, cell on/off indication via backhaul, and SCell activation/deactivation signalling. They can be categorized into UE-triggered and NW-triggered.

From our perspective, the cell on/off indication via backhaul is more of RAN3 expertise. In R18, RAN3 already introduced inter-node beam activation over F1 and Xn interface, and gNB-CU can request gNB-DU to activate certain SSB beams by indicating the SSB index, and it could be evaluated whether this can be used for on-demand SSB triggering over the F1 interface.

--cite from TS 38.423 v18.0.0 If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

Proposal 3: It is up to RAN3 to decide on the cell on/off indication via backhual.

In RAN1#116 meeting, RAN1 agreed to support NW-triggered on-demand SSB:

Agreement Support on-demand SSB SCell operation triggered by gNB. - FFS Details of associated signaling/indication/configuration provided to UE

The remaining issue is whether UE-triggered on-demand SSB needs to be supported in addition. In our understanding, the NW-triggered method is enough, considering the gNB can already obtain the BSR reporting from the UE to determine the UL buffer status, and the gNB is always in control of the DL data scheduling, it is unclear in which use case the triggering cannot be performed by the NW and can only be performed by the UE.

Proposal 4: UE-triggered on-demand SSB is not supported, unless certain use cases are identified where the network is unable to make the triggering decision.

With the introduction of on-demand SSB, the SSB transmission status should be known to the UE. It could be achieved by UE blind detection or explicit indication from the network. From UE power saving perspective, it is preferred that the network explicitly indicates whether SSB is transmitted to the UE, and some SSB-related UE behaviours can be adjusted (e.g. SSB-based RRM measurements, SSB-based BFD).

Proposal 5: The SSB transmission state is indicated to the UE explicitly.

Regarding the triggering signalling design, we think MAC CE is an appropriate option. In current spec, the SCell activation/deactivation signaling is based on MAC CE, and this MAC CE can be reused if on-demand SSB is only supported during the SCell activation (i.e. Scenario #3). If Scenario #2 is also supported (i.e. on-demand SSB before SCell activation), a new MAC CE is needed, and in this case, it would be better to have a unified solution for Scenario #2 and Scenario #3. The RRC signalling is not preferred since it is less flexible compared with MAC CE and introduces larger delay. As for the DCI, it is up to RAN1’s discussion and the issue on reliability (e.g. confirmation of the L1 signaling) should be considered.

Proposal 6: MAC CE can be considered to trigger SSB transmission. FFS if legacy or new MAC CE is used.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: RAN2 starts discussion from Scenario #2 (before SCell activation when SCell is configured) and Scenario #3 (during SCell activation). The FFS part of Scenario #3 (after SCell activation) waits for further RAN1 progress.

Proposal 2: It is up to network implementation to apply on-demand SSB operation to CD-SSB or NCD-SSB.

Proposal 3: It is up to RAN3 to decide on the cell on/off indication via backhual.

Proposal 4: UE-triggered on-demand SSB is not supported, unless certain use cases are identified where the network is unable to make the triggering decision.

Proposal 5: The SSB transmission state is indicated to the UE explicitly.

Proposal 6: MAC CE can be considered to trigger SSB transmission. FFS if legacy or new MAC CE is used.

References

- RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson

- R4-2317306, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon

- TS 38.473, v18.0.0