3GPP TSG-RAN WG2 Meeting #125bis	R2-2402912
Changsha, China, April 15th – 19th, 2024

Agenda Item:	8.5.4

Source:	InterDigital

Title:	Time domain adaptation of common signalling and channels

Document for:	Discussion, Decision

# Introduction

The Rel-19 New WID on Network Energy Savings for NR has the following objective on on-demand SSB SCell [1]:

The WID includes the following objective accordingly:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The agreements made in RAN1#116 meeting are provided in the appendix. In this contribution, we discuss the enhancements for enabling on-demand SSB SCell.

# Discussion

For CA deployments, SCells with SSB-less Scell has been supported since Rel-15 for both FR1 and FR2. In Rel-18 the support for SSB-less Scell is extended for collocated cells in FR1 that are in inter-band CA (e.g. within max receive time, frequency and power difference). In the inter-base CA case, the NW signals to the UE which cell is the reference cell (e.g. a Pcell or another Scell), where the reference cell acts as the timing reference and AGC source of the SSB-less Scell.

Supporting inter-band CA for FR2 comes with challenges, e.g. achieving synchronization, L1/L3 measurements and beam management, especially when the cells are non-collocated and only the Scells operate in FR2 while other cells in FR1. For energy saving purposes in CA, it is important for the Scells to operate in a NES mode as long as possible without or with limited SSB transmission.

## Scenarios where OD-SSB is applicable

RAN1 has already identified some scenarios where OD-SSB can be applicable was discussed in the context of Scell activation procedure, including the following scenarios:

    - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command.

    - Scenario #3: After UE receives SCell activation command and the SCell is not yet activated.

OD-SSB transmission in these scenarios can be beneficial to expedite the Scell activation procedure. For example, in Scenario #2 delay due to Scell activation can be reduced by initiating L3 measurements at UE earlier on the OD-SSBs transmitted after the Scell is configured and before Scell activation. In Scenario #3 the measurements/reporting by UE on the OD-SSBs, which are triggered immediately after the Scell activation command, can be useful to speed up completion of Scell activation.

After Scell activation procedure is complete, it is possible that the Scell can be transitioned to a dormant mode where the SSBs may be transmitted sparsely or with longer periodicity or completely SSB-less for realizing further energy savings. In this case, triggering OD-SSB transmission after Scell activation is completed can be beneficial to improve synchronization, timing reference and AGC at the UE, especially when the last transmission of SSBs may be outdated.

Proposal 1:	On-demand SSB is transmitted in the following scenarios:

  - SCell is configured to a UE but before the UE receives SCell activation command.

  - After UE receives SCell activation command and the SCell is not yet activated.

  - After Scell activation is completed

## Assumption on OD-SSB cells and transmission

RAN1 already identified some assumptions on the SCell for which OD-SSB transmission is possible, including:

    - No always-on SSB on the cell

    - Always-on SSB is periodically transmitted on the cell

transmitting always-on SSBs results in lower NES gains even when using longer SSB periodicity (e.g. 160ms) compared to considering the cell as SSB-less. With a prolonged SSB periodicity on the SCell, the impacts to legacy UEs can be higher when the SSBs are not transmitted per legacy periodicity.

Proposal 2:	As baseline, UE assumes the SCell is SSB-less prior to receiving OD-SSB.

With regards to the timing of the OD-SSB, the following options above can be broadly categorized as follows:

-	Category 1: Periodic OD-SSB transmission with single periodicity

-	Category 2: Periodic OD-SSB transmission with periodicity switching

-	Category 3: OD-SSB is transmitted once or a-periodically

For Category 1, the Scell is assumed to transmit OD-SSBs regularly with certain periodicity. This option makes sense, as once the SCell is activated, there will be a need for recurring SSBs, as the UE is in connected mode and will periodically report channel measurements. The applicability of this category is thus suitable for scenario where the UE has received an SCell activation command. Category 2 requires predefining instances in time at which SSB transmission assumptions will change. However, predefining such instances should be specific for different scenarios and UEs, and thus will be tough to configure or specify. Category 3 can be suitable when the SSB periodicity is already too long, and an on demand SSB occasion is needed. In the case of Category 3, the network has more flexibility transmit OD-SSB in a limited time period only when there is a need (e.g. for measurements at UE) and hence, enables higher NES gains. The aperiodic/semi-persistent OD-SSB transmission can be useful for all assumed deployment scenarios, including the case where the Scell is already activated and is applying NES.

Proposal 3:	Consider the following assumption/options for OD-SSB transmission timing:

  - UE expects that on-demand SSB is periodically transmitted, with single periodicity

  - UE expects that on-demand SSB is transmitted once or a-periodically

## Triggers for OD-SSB reception

During Rel-18 SI, It was discussed that the Scells which are used as capacity cells and operating in NES mode are not expected to transmit SSBs or only transmit sparse SSBs. gNB initiated triggering of OD-SSB in the context of Scell activation procedure has been agreed by R1. In addition to gNB initiated triggering, triggering OD-SSB transmission from the Scells only when there is a need from the UE (e.g. based on channel conditions when the UE moves into coverage, specifically when cells are not geolocated) is also practical.

The request for on-demand SSB can thus be conveyed by the UE, as likely the conditions for requesting such demand is known at the UE. The UE can therefore send the UL signal to gNB, corresponding to UL wake-up signal (WUS), to request for OD-SSB tranmisison. Using UL WUS as a triggering method for OD-SSB transmission also results in considerable NES gains. The Rel-18 TR [2] mentions that with sparse SSB of 160ms periodicity transmitted before gNB reception of WUS, 5 sources show technique A-3-1 of UE WUS triggering gNB for SSB/SIB/RACH, with the assumption of ideal detection of UL WUS, could achieve BS energy savings by 6.2% – 80.7% in range with UPT loss by 0%-24.2%, where the gain increases as a function of the WUS period. Given the benefits in terms of low delay, energy savings and low UPT losses, UL WUS can be supported as a triggering method for OD-SSB at SCell.

Proposal 4:	Support on-demand SSB SCell operation triggered by:

  - gNB command (e.g. L1/L2 SCell activation signalling)

  - UE request by transmission of UL WUS. FFS: Conditions for triggering UL WUS

Options considered in the R18 SI for UL WUS include using PRACH resource and PUCCH resource (e.g. SR). Using a PUCCH resource for triggering on demand SSBs can be useful for UEs in connected mode, but this may require introducing a new procedure for triggering the transmission and ensuring suffient reliability/coverage when receiving at Scell and hence, less preferable for UL WUS. Using PRACH is thus simpler, since the gNB monitors PRACH on the PCell regardless of NES states.

contention-based PRACH resources for UL WUS can be configured to consist of a set of PRACH preambles and RACH occasions. Given there is no CBRA on the Scell, the UL WUS would have to be transmitted by the UE on the PCell.

The resources for UL WUS can be provided by the PCell to the NES-capable UEs as a separate configuration or a sub-configuration of the RACH config.

Proposal 5:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell.

# Conclusion

The following proposals are provided for on demand SSB reception on SCells:

Proposal 1:	On-demand SSB is transmitted in the following scenarios:

  - SCell is configured to a UE but before the UE receives SCell activation command.

  - After UE receives SCell activation command and the SCell is not yet activated.

  - After Scell activation is completed

Proposal 2:	As baseline, UE assumes the SCell is SSB-less prior to receiving OD-SSB.

Proposal 3:	Consider the following assumption/options for OD-SSB transmission timing:

  - UE expects that on-demand SSB is periodically transmitted, with single periodicity

  - UE expects that on-demand SSB is transmitted once or a-periodically

Proposal 4:	Support on-demand SSB SCell operation triggered by:

  - gNB command (e.g. L1/L2 SCell activation signalling)

  - UE request by transmission of UL WUS. FFS: Conditions for triggering UL WUS

Proposal 5:	Support using PRACH preamble on the PCell as UL WUS for triggering OD-SSB on the SCell.

# References

RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023

# Appendix: Agreements on OD-SSB SCell operation

RAN1#116 Agreements

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)