3GPP TSG-RAN WG2 Meeting #125bis                                                                                 R2-2402980

Changsha, China, April 15th – 19th, 2024

Agenda Item:	8.7.4

Source: 	Meta

Title:  	Discussion on Scheduling Enhancement for XR

Document for:	Discussion and Decision

# Introduction

In RAN #102 plenary meeting, it was agreed that one of the WI objectives in the Phase 3 XR WID is [1]:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

In this paper, we discuss scheduling enhancements for XR.

# Discussion

In Release 18, the Delay Status Report (DSR) was introduced to provide feedback on the remaining time of buffered data when a UE requests uplink resources. This enables the gNB to perform delay-aware uplink scheduling. One of the limitations of delay status reporting is that when multiple PDU sets in a LCG have different remaining times, only the smallest remaining time below the threshold is reported.

Proposal 1: Enhance DSR to report remaining time with a finer granularity.

The legacy Logical Channel Prioritization at the MAC may not prioritize DSR-reported delay-critical data since LCP does not take delay into account when they are in the low-priority LCH. Enhancing the LCP mechanism could be beneficial but must be evaluated carefully based on the trade-off between the potential performance improvement and the resulting increase in UE complexity.

Proposal 2: Study the performance improvement of LCP enhancement that considers delay information.

# Conclusion

In this paper, we discuss scheduling enhancement for XR.

Proposal 1: Enhance DSR to report remaining time with a finer granularity.

Proposal 2: Study the performance improvement of LCP enhancement that considers delay information.

# References

[1] RP-240791, “Revised WID on XR (eXtended Reality) for NR Phase 3”