3GPP TSG-RAN WG2 Meeting #125bis	R2-2403139

# Changsha, China, April 15th – 19th, 2024

Source:                Qualcomm Incorporated

Title:	UE capabilities on MP relay

Agenda Item:	7.9.3

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses UE capabilities on MP relay.

# Discussion

1) IMS voice over split bearer

In existing specification, IMS voice over split bearer is not supported for NR-DC and NE-DC.

NOTE:	In this release of specification, IMS voice over split bearer is not supported for NR-DC and NE-DC.

MP relay reuse DC architecture, then it should be clarified that IMS voice over split bearer is not supported for MP operation.

Proposal 1: Clarify IMS voice over split bearer is not supported for MP operation.

2) UL transmission path

Today, there are two UE capabilities to indicate UL transmission path when split bearers are configured as follows.

| splitSRB-WithOneUL-Path Indicates whether the UE supports UL transmission via MCG path and DL reception via either MCG path or SCG path, as specified for the split SRB in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | No | No | No |
| --- | --- | --- | --- | --- |
| splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | Yes | No | No |

Similarly, in MP operation, UE should also indicate whether to support transmission via direct path and direct path for UL and DL. Since the existing capabilities are defined for MCG and SCG, then new capabilities needs to be introduced.

Proposal 2: Introduce new UE capability to indicate whether UE supports UL transmission via direct path and DL reception via either direct path or indirect path for split SRB.

Proposal 3: Introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

# Conclusion

Proposal 1: Clarify IMS voice over split bearer is not supported for MP operation.

Proposal 2: Introduce new UE capability to indicate whether UE supports UL transmission via direct path and DL reception via either direct path or indirect path for split SRB.

Proposal 3: Introduce new UE capability to indicate whether UE supports UL transmission via both direct path and indirect path for the split DRB.

# Reference