3GPP TSG-RAN WG2 Meeting #125bis								R2-2403154

Changsha, China, April 15 - 19, 2024

Agenda Item:	4.1.1

Source:	Huawei, HiSilicon

Title:	Discussion on LTE QoE configuration handling during inter-RAT mobility

Document for:	Discussion and decision

Introduction

At RAN2#125 meeting, QoE configuration handling during inter-RAT mobility was discussed [1], and the minutes is shown as below:

E099 (IRAT HO) R2-2401133 On FFS for LTE QoE configurations release for inter-RAT HO from LTE to NR [E099] Nokia, Nokia Shanghai Bell discussion Rel-18 NR_QoE_enh-Core Proposal 1: For inter-RAT Handover from LTE to NR, LTE AS layer (with RRC procedure defined in LTE spec) informs APP layer to release the “old” QoE configuration configured by source (LTE) node. R2-2401160 QoE configuration handling during inter-RAT mobility Qualcomm Incorporated discussion NR_QoE_enh-Core Proposal: Current specification already captures that all LTE QoE configurations are released at handover to NR, then specification is not needed. DISCUSSION: - Huawei agrees with QCM’s proposal. This is legacy issue. If we fix this, then it should be from Rel-17. - Ericsson agrees with Nokia. General statements refer to AS configuration, but this is about app layer configuration. Nokia agrees. - Ericsson agrees this issue exists also in previous releases. We do not fix this as part of R18 WI on QoE Proponents can bring a CR for legacy specifications to fix this

The above discussion addressed the following issue in RAN2:

For HO from LTE/5GC to NR, UE should release all LTE QoE configurations and apply NR QoE configuration if received. How UE releases old QoE configurations can be handled in CR phase.

This issue is now a legacy issue, and we suggest to discuss it for LTE QoE feature.

Discussion

In 36.331 [2], it states that the UE should inform upper layers to clear QoE related configuration and discard QoE reports (if any), when the UE is leaving RRC_Connected or RRC_Inactive. Details can be found below (i.e. highlighted parts).

For HO from LTE/5GC to NR, the UE will anyway leave LTE RRC_Connected state and go into NR RRC_Connected state. In this case, the UE behaviors should be clear according to the current 36.331.

- 5.3.12 UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall: - reset MAC; <Partially omitted> 1> else: 2> upon leaving RRC_INACTIVE: 3> discard the UE Inactive AS context; 3> discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key; 2> release rrc-InactiveConfig, if configured; 2> remove all entries within VarConditionalReconfiguration, if any; 2> for each measId, that is part of the current UE configuration in VarMeasConfig, if the associated reportConfig has condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR configured: 3> remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig; 3> if the associated measObjectId is only associated with condReconfigurationTriggerEUTRA/condReconfigurationTriggerNR: 4> remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig; 3> remove the entry with the matching measId from the measIdList within the VarMeasConfig; 2> release all radio resources, including release of the MAC configuration, the RLC entity and the associated PDCP entity and SDAP (if any) for all established RBs, except for the following: - pur-Config, if stored; 2> indicate the release of the RRC connection to upper layers together with the release cause; 1> release the stored serviceType, if any; 1> inform upper layers to clear the stored application layer measurement configuration; 1> discard received application layer measurement report information from upper layers, if any; 1> consider itself not to be configured to send application layer measurement report; 1> if leaving RRC_CONNECTED was triggered neither by reception of the MobilityFromEUTRACommand message nor by selecting an inter-RAT cell while T311 was running; or <Partially omitted>

Proposal: In current TS 36.331, all LTE QoE configurations are released upon releaving LTE RRC_Connected or RRC_Inactive, which also includes the case of HO from LTE/5GC to NR. No specification changes are needed.

Conclusions

In this contribution, we share our views on LTE QoE configuration handling during inter-RAT mobility, and we propose:

Proposal: In current TS 36.331, all LTE QoE configurations are released upon releaving LTE RRC_Connected or RRC_Inactive, which also includes the case of HO from LTE/5GC to NR. No specification changes are needed.

- References

- [1]	R2-2401546	Report from session on MBS and QoE	Session chair (Huawei)

- [2]	TS 36.331 v17.7.0