3GPP TSG-RAN WG2 Meeting #125bis	R2-2403196

Changsha, China, 15 – 19 April 2024

Agenda item:	7.13.3

Source:	Nokia

Title:	Considerations on including TAC in RA reports

WID/SID:	TEI-18, NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

At RAN2#125 the decision on adding TAC in RA reports remained open (R2-2401549):

R2-2401732	Correction on the reporting of TAC in Random access report	Fujitsu

Postponed, companies can check further with their RAN3 details. If there is no RAN3 impact, RAN2 sees no roadblocks for agreeing this in the next meeting.

This paper summarizes the corresponding RAN3 status.

# 2	Discussion

During the Rel-18 SON/MDT work the following agreement was made at RAN3#120:

Not consider the NG and S1 forwarding of RA report.

Observation 1: RAN3 explicitly decided not to support RA report forwarding over NG and S1 interfaces.

The current RAN3 specification does not support this functionality and additional RAN3 specification would be needed to support it. For example, NG specification would require some extension to enable it. SON-related reports are sent over NG from the NG-RAN node to the AMF in the UPLINK RAN CONFIGURATION TRANSFER message, and from the AMF to the NG-RAN node in the DOWNLINK RAN CONFIGURATION TRANSFER message. These messages convey, among other information, the SON Information Report IE defined as follows [TS 38.413]:

#### 9.3.3.35	SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  | - |  |
| >Failure Indication Information |  |  |  |  |  |  |
| >>Failure Indication | M |  | 9.3.3.37 |  | - |  |
| >HO Report Information |  |  |  |  |  |  |
| >>HO Report | M |  | 9.3.3.39 |  | - |  |
| >Successful HO Report Information |  |  |  |  | YES | ignore |
| >>Successful HO Report List |  | 1 |  |  | - |  |
| >>>Successful HO Report Item |  | 1..<maxnoofSuccessfulHOReports> |  |  | - |  |
| >>>>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in TS 38.331 [18]. | - |  |
| >Successful PSCell Change Report Information |  |  |  |  | YES | ignore |
| >>Successful PSCell Change Report List |  | 1 |  |  | - |  |
| >>>Successful PSCell Change Report Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>>>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [18]. | - |  |

The SON Information Report IE includes the different types of SON reports that can be sent over NG, such as the Failure Indication IE, which is used to transfer the RLF Report, the Successful HO Report Information UE, which is used to transfer SHR. In the current specification there is no IE for sending the RA report. The situation is similar in S1AP [TS 36.413].

Observation 2: Supporting RA report forwarding over NG and S1 interfaces requires RAN3 specification work.

As the decision on introducing RA report forwarding via the core network is in the scope of RAN3, we think that adding TAC to RA reports in RAN2 specification should depend on RAN3 decision.

Proposal: The addition of TAC into RA reports should be specified in RAN2 after a RAN3 agreement that RA report forwarding via the core network is supported.

# 3	Conclusion

This document has made the following observations and proposal:

Observation 1: RAN3 explicitly decided not to support RA report forwarding over NG and S1 interfaces.

Observation 2: Supporting RA report forwarding over NG and S1 interfaces requires RAN3 specification work.

Proposal: The addition of TAC into RA reports should be specified in RAN2 after a RAN3 agreement that RA report forwarding via the core network is supported.