3GPP TSG-RAN WG2 Meeting #125bis	R2-2403203

Changsha, China, April 15th – 19th, 2024

Agenda item:	7.11.3

Source: 	Langbo

Title: 	Error data handling for MBS

Document Type:	Discussion and Decision

1       Introduction

In this contribution, we discuss the error data handling for MBS.

2       Discussion

According to the current TS 38.331[1], the multicast MRB(s) associated with multicast session(s) not configured to receive in RRC_INACTIVE will be suspended when UE enter into RRC_INACTIVE.

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: … 2> suspend all SRB(s) and DRB(s), except SRB0 and broadcast MRBs; 2> suspend all multicast MRB(s) associated with multicast session(s) not configured to receive in RRC_INACTIVE; …

In the current TS 38.321, once the MAC entity receives a multicast data in RRC_INACTIVE, the multicast data will always be acceptted by the MAC entity.

However, if a subPDU contains an LCID or eLCID value which is associated with a suspended multicast MRB in RRC_INACTIVE, this subPDU will result in error. Therefore, multicast data associated with a suspended multicast MRB should be accounted as unknown, unforeseen and erroneous protocol data and be discarded.

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

In the current spec, MAC PDU received for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured is handled in clause 5.3.3 [2]. Since unknown, unforeseen and erroneous protocol data is usually handled in clause 5.13 rather than clause 5.3.3, to facilitate the reading, it’s better to move this case from clause 5.3.3 to clause 5.13.

5.3.3 Disassembly and demultiplexing The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a. When a MAC entity receives a MAC PDU for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, the MAC entity shall at least: 1> discard the received subPDU. 5.13 Handling of unknown, unforeseen and erroneous protocol data When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least: 1> discard the received subPDU. When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall: 1> discard the received subPDU. When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU.

Proposal 2: Move the handling of MAC PDU received for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured from clause 5.3.3 to clause 5.13.

Proposal 3: Adopt the TPs in the Annex.

3       Conclusion

In this contribution, we discuss error data handling for MBS, and propose as follows:

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Proposal 2: Move the handling of MAC PDU received for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured from clause 5.3.3 to clause 5.13.

Proposal 3: Adopt the TP in the Annex.

4       Reference

[1] TS 38.331, Radio Resource Control (RRC) protocol specification, v18.1.0, April, 2024.

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.1.0, January, 2024.

5       Text proposal

Text proposal is based on 38.321 v18.1.0.

START OF CHANGE

5.3.3	Disassembly and demultiplexing

The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clauses 6.1.2 and 6.1.5a.

When a MAC entity receives a MAC PDU for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, the MAC entity shall at least:

1>	discard the received subPDU.

...

5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU for MAC entity's G-RNTI or G-CS-RNTI, or by the configured downlink assignment for MBS multicast containing an LCID or eLCID which is not configured, or an LCID or eLCID value associated with a suspended multicast MRB as specified in TS 38.331 [5], the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

END OF CHANGE