# 3GPP TSG RAN WG2 Meeting #125 bis                          	   R2-2403204

# Changsha, China, April 15-19, 2024

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on Adaptation of Common Signal/Channel Transmissions

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The WID for “Enhancements of network energy savings for NR” was approved at RAN #102, and further updated at RAN # 103 with the follows [1].

The objectives of the work item are the following: - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we discuss scenarios or use cases for adaptation of common signal and channel transmissions and potential impact to specification work.

# Discussion

## Adaptation of SSB in time domain

For adaptation of SSB in time domain, the following agreements were made at RAN1 #116 for further study [2].

Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study. - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration

NR supports cell-defining SSB (CD-SSB) and non-cell-defining SSB (NCD-SSB). The CD-SSB has associated SIB1 and transmitted in a cell supporting initial access/idle mode operation for UEs at Idle or Inactive state. The NCD-SSB does not have associated SIB1 and can be transmitted in a cell that may not support initial access/idle mode operation, e.g., the secondary cell (Scell) in CA deployments. Furthermore, the NCD-SSB is only needed for UE connected mode operation, e.g., beam management, mobility management, power control etc. or for a BWP without CD-SSB.

Two main questions were brought up during the RAN1#116 meeting discussion. First, whether SSB transmission adaptation is applicable to UEs at both Idle/Inactive state and Connected state or to UEs at Connected state only. Second, for UEs at Connected state, whether SSB transmission adaptation should be applied to both the Pcell and Scell or to only one of them.

SSB design is mainly under RAN1’s scope, RAN2 can wait for RAN1’s decision and not discuss SSB adaptation at the first meeting of Rel-19 NES.

Proposal 1: For SSB adaptation, RAN2 can wait for RAN1's conclusion on the applicable scenarios.

## Adaptation of PRACH in time domain

For adaptation of PRACH in time domain, the following agreement wwas made at RAN1 #116 for further study [2].

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study. - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH. - Concentrating ROs in time domain - Other options are not precluded

Based on RAN1’s agreement, PRACH adaptation in time may be based on the configuration of additional or different PRACH resources for NES-capable UEs only.

Furthermore, the additional PRACH resources may be based on the adaptation of PRACH resource period or occasions, adaptation of association pattern with SSB, adaptation on cell DRX, etc.

Proposal 2. On RACH occasion (resource) adaptation, RAN2 can evaluate possible RRC and MAC spec impacts and leave the study conclusion to RAN1.

## Adaptation of PRACH in spatial domain

Adaptation of PRACH in spatial domain may achieve network energy saving by optimizing the number of ROs for each direction based on the UE distribution in the coverage area, e.g., the number of UEs at a spatial direction.

Adaptation of PRACH in spatial domain is mostly under RAN1’s scope, RAN2 should not spend time on it for now.

Proposal 3. RAN2 wait for RAN1’s progress on Adaptation of PRACH in spatial domain.

## Adaptation of paging occasions

For adaptation of paging occasions, the following agreement was made at RAN1 #116 for further study [2].

Agreement For adaptation of paging, - Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings. - Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

For saving NES Cell power consumption, paging frame (PF) may be adapted within a paging cycle for Rel-19 NES capable UEs, e.g., one PF within a paging cycle. Additionally, paging occasion (PO) within a PF may also be adapted for Rel-19 NES capable UEs, e.g., allocating new POs within a PF only for NES capable UEs where the new POs may be evenly distributed or grouped together. With new PO/PF, should NES capable UEs still be allowed to monitor the legacy POs?

Proposal 4. For paging adaptation, RAN2 discusses how to adapt PO/PF with new PO/PF for R19 NES-capable UEs.

Proposal 5. For paging adaptation, RAN2 discusses whether R19 NES-capable UEs are restricted with new PO/PF or not.

For new POs, R19 NES-capable UEs needs to know how to determine the new PO and PF.

Proposal 6. RAN2 discusses how to determine new PO/PF for R19 NES-capable UEs.

Additionally, the new PO/PF may be adapted based on traffic and UE distribution and thus the network needs to indicate to the new PO/PF Rel-19 NES capable UEs, e.g., broadcasted in SIB1 or configured via RRC.

Proposal 7. RAN2 discusses how to indicate new PO/PF to R19 NES-capable UEs.

# Conclusion

In this contribution, we discussed scenarios or options for on-demand SIB1 operation and concluded with the following observations and proposals.

Adaptation of SSB in time domain

Proposal 1: For SSB adaptation, RAN2 can wait for RAN1's conclusion on the applicable scenarios.

Adaptation of PRACH in time domain

Proposal 2. On RACH occasion (resource) adaptation, RAN2 can evaluate possible RRC and MAC spec impacts and leave the study conclusion to RAN1.

Adaptation of PRACH in spatial domain

Proposal 3. RAN2 wait for RAN1’s progress on Adaptation of PRACH in spatial domain.

Adaptation of paging occasions

Proposal 4. For paging adaptation, RAN2 discusses how to adapt PO/PF with new PO/PF for R19 NES-capable UEs.

Proposal 5. For paging adaptation, RAN2 discusses whether R19 NES-capable UEs are restricted with new PO/PF or not.

Proposal 6. RAN2 discusses how to determine new PO/PF for R19 NES-capable UEs.

Proposal 7. RAN2 discusses how to indicate new PO/PF to R19 NES-capable UEs.

# References

[1] RP-240170 “Revised WID for Enhancements of network energy savings for NR”, RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[2] “Chair notes RAN1#116 eom0” RAN1 #116	 Athens, Greece, February 26th – March 1st, 2024.