3GPP TSG-RAN WG2 Meeting #125bis	R2-2403312

Changsha, China, Apr 15th –19th, 2024

Agenda item:	7.9.7

Source: 	Huawei, HiSilicon

Title: 	UE capability corrections for multi-path operation

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss if we need to intoduce the UE capability for supporting UL transmission of the split DRB via both MCG (i.e. direct path) and indirect path that is there for the DC scenario.

# Discussion

In legacy DC scenario, there is a parameter to indicate the UE capability of UL transmission via both MCG path and SCG path with  the split DRB, as defined in TS 38.306:

Excerpt from TS 38.306 ### 4.2.2 General parameters Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. <…unrelated content skipped…>. splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode).; UE; Yes; No; No.

Observation 1: There is a parameter splitDRB-withUL-Both-MCG-SCG to indicate whether the UE supports UL transmission via both MCG path and SCG path for the split DRB in DC scenario.

In R18 L2 multi-path relay operation, there may be a case where the UE only supports DL reception of the split DRB from MCG (i.e. direct path) and indirect path, but does not support UL transmission of the split DRB on both MCG and indirect path. Therefore, we feel  RAN2 should also  discuss whether we need an additional UE capability to indicate that the UE supports UL transmission via both MCG and indirect path.

Proposal 1: RAN2 to discuss whether a UE capability is needed to indicate that the UE supports UL transmission via both MCG (i.e. direct path) and indirect path.

# Conclusion

Observation 1: There is a parameter splitDRB-withUL-Both-MCG-SCG to indicate whether the UE supports UL transmission via both MCG path and SCG path for the split DRB in DC scenario.

Proposal 1: RAN2 to discuss whether a UE capability  is needed to indicate that the UE supports UL transmission via both MCG (i.e. direct path) and indirect path.