3GPP TSG-RAN WG2 Meeting #125bis	R2-2403336

Changsha, Athens, 14th April – 19th April, 2024

Agenda item:	7.6.5

Source:	Samsung

Title:	On procedures and capabilities for GNSS position fix during C-DRX

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the WID for Rel-18 IoT NTN was agreed [1] and in RAN#98-e it was updated [2]. The updated objectives of the WID is the following:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements. 4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4]. 4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

In this contribution we address some issues on GNSS position fix during C-DRX.

# Discussion

- In RAN2#123 the following was agreed regarding GNSS position fix during inactive periods of C-DRX:

- UE can autonomously start GNSS measurement during the inactive state of C-DRX.

- The exact time of starting GNSS measurement during the inactive state of C-DRX can be left for UE implementation.

- If UE failed to autonomously re-acquire the GNSS position fix and the GNSS position is still valid during the inactive state of C-DRX, UE does not move to RRC_IDLE. There is no specification impact. FFS if we still allow the UE not to move to Idle in case GNSS position is outdated

- For GNSS measurements during C-DRX there are still several open issues that have yet to be discussed, which we discuss in this contribution.

## Procedures applicable to GNSS position fix during C-DRX

When a UE finishes the GNSS position fix during C-DRX, it has not been clarified whether the UE shall trigger the GNSS Validity Duration Reporting MAC CE. Since the UE signals a GNSS validity duration in a number of –Complete message, it is important for the network to receive information about any update of the GNSS validity duration as result of GNSS position fixes. Otherwise, the network may have to assume that the UE will go to RRC idle according to the GNSS validity duration signalled in any of the –Complete messages. Therefore, we think that it would be crucial for the UE to trigger GNSS Validity Duration Report MAC CE after completing GNSS position fix during C-DRX.

Proposal 1: Clarify that UE reports GNSS Validity Duration Report MAC CE after successful GNSS position fix during C-DRX.

We believe that no procedural change is needed to reflect this and that a clarification in 36.306 is sufficient. To this end, consider the text proposal in Appendix A.

## GNSS position fix during C-DRX capabilities

In 36.306, the GNSS position fix during C-DRX is a capability without capability signaling [3]:

--------------- 36.306 V18.1.0 ---------------

6.19.8	GNSS measurements during inactive time

It is optional for UE in RRC_CONNECTED in an NTN cell to perform GNSS measurements during inactive time of a C-DRX cycle. This feature is only applicable if the UE supports ntn-Connectivity-EPC-r17.

--------------- 36.306 V18.1.0 ---------------

With the current standardized signalling, the network will not be aware of whether a UE is capable of performing GNSS position fix during C-DRX. This raises the question whether it could be beneficial that a network is made aware of whether the UE has this capability or not.

For instance, one scenario may be that the UE supports both GNSS measurement during inactive time and triggered GNSS fix. In this case, it would be useful for the network be made aware, so that the network does not have to trigger the GNSS position fix, and instead rely on GNSS measurement during inactive time.

- Observation 1: A UE can support both “GNSS measurement during inactive time” and “ntn-Triggered-GNSS-Fix-r18”.

If the UE can support a combination of these features, we think it would be very useful for the network to know whether a UE can support “GNSS measurements during inactive time”. This can for instance decide whether it is needed for a UE to trigger a GNSS position fix and in configuring C-DRX to the UE. This is also useful for managing a UEs GNSS validity duration. This is because support of GNSS measurement during inactive does not mean that GNSS validity duration is infinity.

- Observation 2: Signalling support for “GNSS measurement during inactive time” is useful for the network to configure the UE and to manage the UEs GNSS validity duration.

Proposal 2: “GNSS measurement during inactive” time is changed to a signalled capability.

The CR in Appendix A shows how this can be captured.

Proposal 3: Agree text proposal to 36.306 in Appendix A.

# Conclusion

In this contribution we have the following observations and proposals:

- Observation 1: A UE can support both “GNSS measurement during inactive time” and “ntn-Triggered-GNSS-Fix-r18”.

- Observation 2: Signalling support for “GNSS measurement during inactive time” is useful for the network to configure the UE and to manage the UEs GNSS validity duration.

Proposal 1: Clarify that UE reports GNSS Validity Duration Report MAC CE after successful GNSS position fix during C-DRX.

Proposal 2: “GNSS measurement during inactive” time is changed to a signalled capability.

Proposal 3: Agree text proposal to 36.306 in Appendix A.

# Reference

RP-213596, New WID on IoT NTN enhancements, Mediatek Inc., RAN#94-e, December 2021

RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022

3GPP TS 36.306, Evolved Universal Terrestrial Radio Access; User Equipment (UE) radio access capabilities, Release 18, V18.1.0, March 2024.

# Appendix

## Appendix A – 36.306 TP

--------------- Text proposal based on 36.306 V18.0.0 ---------------

4.3.38		IoT NTN parameters

. . . OMITTED . . .

4.3.38.XY	ntn-C-DRX-GNSS-Fix-r18

This field indicates whether the UE supports performing GNSS position fix in RRC_CONNECTED during C-DRX as specified in TS 36.331 [5]. This field is only applicable if the UE supports ce-ModeA-r13 or any ue-Category-NB. A UE supporting this feature shall also indicate the support of ntn-Connectivity-EPC-r17. If the UE indicates this capability, the UE shall support the following enhancements:

-	UE reports the remaining GNSS validity duration with MAC CE in RRC_CONNECTED.

6.19	IoT NTN Features

. . . OMITTED . . .

6.19.8		Void

--------------- Text proposal based on 36.306 V18.0.0 ---------------